| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0222-v0 |
| --- | --- |

| Title: | EE2-4.4b and 4.4c: Combined Tests for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ikram Jumakulyyev, Nan Hu, Zhi Zhang, Vadim Seregin, Marta Karczewicz, Han Huang 5775 Morehouse Drive, San Diego, CA 92121, USA Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | ijumakul@qti.qualcomm.com nanh@qti.qualcomm.com yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Incorporated Bytedance Inc. |  |  |

_____________________________

- Abstract

There are 3 ALF-related EE tests during AC meeting cycle. In test 4.1, the additional fixed-filter for ALF is tested. In test 4.2, the residual based classifier for ALF is tested. In test 4.3, the ALF filter shape with more taps applied to fixed-filter result is tested. In this contribution, the results of ALF-related combined tests 4.4b and 4.4c are reported.

On top of ECM-8.0, simulation results of EE2-4.4a/4.4b/4.4c are reported as below:

4.4b:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

4.4c:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Introduction

In ALF of ECM-8.0, online-trained filters consist of 4 kinds of filter taps: spatial taps, reconstruction-before-DBF based taps, residual based taps and fixed-filter-output based taps, as shown in Figure 1. The spatial taps (i.e., tap #0 ~ #19) located in a cross shape are followed by 3 reconstruction-before-DBF based taps (i.e., tap #26, #27, #30), 2 residual based taps (i.e., tap #31 ~ #32), and the offline-filtered taps (i.e., tap #20 ~ #25, #28, #29), resulting in 33 taps in total.

There are 2 classifiers for the online-trained filters, which are texture based and band based classification methods.

Figure 1 Filter shape of an online derived ALF in ECM-8.0.

Test 4.1

Test 4.1 introduces an additional fixed-filter for ALF and 7 extended taps are introduced into luma online-trained filters. The reconstruction before DBF is fed into the additional fixed-filter to produce the filter outputs, then these filter outputs are used as input for the newly extended taps.

Only one additional fixed filter with a shape of diamond 7x7 is introduced, the filter parameters are stored at both encoder and decoder.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without any filter shape switching.

Test 4.2

Test 4.2 introduces a third classifier based on luma residual sample values. For each 2x2 luma block, the sum of absolute values of residual samples in a neighbouring 8x8 window is calculated, and the class index is derived as:

classIdx = sum >> (sample bit depth – 4).

The value of classIdx is in the range of 0 to 24, same as in ECM-8.0. The classifier usage is signalled for each luma filter set in APS.

Test 4.3

Test 4.3 increases the number of taps that are applied to the first fixed filter results. The overall number of taps for an online-trained luma filter is increased by 2 more taps. More precisely, the filter shape of an online derived ALF is modified as shown in Figure 2. The number of taps (i.e., taps #0-#9) that applied to neighboring reconstructed samples is reduced and the number of taps (i.e., taps #10-#29) that applied to offline-filtered results is increased.

Figure 2. Proposed filter shape of an online derived ALF in test 4.3.

Test 4.4

Three combined tests are performed in EE2-4.4 as follows:

| # | Test |
| --- | --- |
| 4.4b | Combination of 4.1, 4,2 and 4.3 (Additional fixed-filter, residual based classifier and more fixed-filter-output based taps) |
| 4.4c | Combination of 4.1 and 4.2 (Additional fixed-filter and residual based classifier) |

Simulation Results

The combined tests were implemented on top of the ECM-8.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. Table 1 shows the results of test 4.4b and Table 2 shows the results of test 4.4c.

TABLE 1 Performance of EE2-4.4b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.09% | 0.02% | 0.02% |  |  | -0.35% | -0.10% | -0.03% |  |  |  |  |  |  |  |
| Class E | -0.05% | 0.00% | 0.00% |  |  |  |  |  |  |  | -0.12% | 0.30% | -0.32% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.03% | 0.06% | 0.06% |  |  | -0.48% | 0.37% | 0.04% |  |  | -0.44% | -0.45% | 0.89% |  |  |
| Class F |  |  |  |  |  | -0.16% | -0.10% | -0.05% |  |  |  |  |  |  |  |

TABLE 2 Performance of EE2-4.4c

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.06% | 0.02% | 0.02% |  |  | -0.21% | -0.01% | -0.06% |  |  |  |  |  |  |  |
| Class E | -0.05% | 0.00% | 0.00% |  |  |  |  |  |  |  | -0.17% | 0.98% | 0.33% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.04% | 0.04% |  |  | -0.15% | 0.12% | 0.06% |  |  | -0.16% | -0.36% | -0.30% |  |  |
| Class F |  |  |  |  |  | -0.14% | -0.01% | 0.03% |  |  |  |  |  |  |  |

Conclusions

In this contribution, the combined tests for ALF are reported. It is recommended to adopt one of the combined tests into the next version of ECM.

Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Qualcomm incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).