| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0223-v1 |
| --- | --- |

| Title: | Non-EE2: Filtering for IBC predicted block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Muhammed Coban, Vadim Seregin, Marta Karczewicz (Qualcomm) | Email: | bray@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

# Abstract

In ECM-8.0, IBC is enabled for screen content and in EE2-test 1.8 IBC performance in camera captured content is being studied. This contribution proposes to apply filtering on top of IBC prediction as an additional mode. The 6-tap linear filter consists of 5 spatial luma samples in the reference block and a bias term. Filter coefficients are derived for each block using the regression based the minimized MSE on samples between the reference template and current template.

The test results on top of EE2-test1.8.a are as follows:

AI(Y/U/V): -0.14%/-0.18%/-0.16%, EncT 101%, DecT 100%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT 1xx%, DecT 1xx%

# Introduction

In ECM-8.0, IBC is enabled for screen content and in EE2-test 1.8 IBC performance is being evaluated for camera captured content. Current IBC prediction involves copying pixels from the reference block in the current picture as indicated by the block vector. This contribution proposes to build a filter model between the reference template and current template and apply the filter model to reference block. Then the filtered reference block is used as the prediction block.

# Proposed method:

The proposed 6-tap filter consist of a 5-tap plus sign shape spatial component and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) sample in the reference block which is at corresponding locations with the sample in the current block to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the filter

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5B

The filter coefficients ci are calculated by minimising the MSE between the reference template and current template, very similar to the CCCM process.

This filtered mode is used as an additional mode for non-merge IBC blocks. For non-merge blocks, this mode can not be applied together with IBC-LIC, IBC-CIIP or RR-IBC. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed, so there is no extra signaling.

## Performance on top of EE2-test 1.8a:

In this test, the anchor is EE2-test 1.8a and the proposed method is tested on top of this anchor. The results are as follows. Class SCC results are the same as that for ECM8+IBC (which is reported in Table 1.2), as EE2-test1.8.a does not impact SCC coding.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Test-1.8a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | -0.11% | 101% | 100% |
| Class A2 | -0.20% | -0.23% | -0.14% | 101% | 100% |
| Class B | -0.20% | -0.28% | -0.22% | 100% | 100% |
| Class C | -0.05% | -0.01% | -0.07% | 101% | 100% |
| Class E | -0.24% | -0.27% | -0.28% | 101% | 100% |
| Overall | -0.14% | -0.18% | -0.16% | 101% | 100% |
| Class D | -0.03% | -0.10% | 0.00% | 100% | 99% |

## Performance on top of ECM8+IBC

In this test, the anchor is ECM8 with IBC enabled and the proposed method is tested on top of this anchor. The results are as follows.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8+IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | -0.03% | 103% | 100% |
| Class A2 | -0.17% | -0.12% | -0.15% | 102% | 100% |
| Class B | -0.16% | -0.16% | -0.18% | 102% | 100% |
| Class C | -0.05% | -0.02% | -0.04% | 102% | 100% |
| Class E | -0.19% | -0.14% | -0.36% | 102% | 99% |
| Overall | -0.12% | -0.08% | -0.15% | 102% | 100% |
| Class D | -0.01% | -0.15% | -0.10% | 102% | 101% |
| Class F | -0.98% | -1.10% | -1.12% | 102% | 100% |
| Class TGM | -0.31% | -0.31% | -0.26% | 103% | 101% |

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- B.Ray, et al., “Non-EE2: IBC adaptation for coding of natural content”, JVET-AC0161, Jan 2023.

- W. Chen et al. “Non-EE2: IBC with fractional block vectors”, JVET-AC0172, Jan 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).