| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0226-v2 |
| --- | --- |

| Title: | EE1-1.5: Ablation Study and Minor Improvements on RTNN |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Yang Liu Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

In JVET-AC0156 [1], an NN-based in-loop filter, named RTNN, was presented to enhance the quality of reconstructed frame. The RTNN explores the combination of Resblock [6] and Transformer block (TB) [2] to build a new backbone network with efficiency. Moreover, a new attention module is proposed to better refine features by introducing auxiliary information. In this contribution, some experiments are designed to verify the performance of RTNN. The main contents of EE1-1.5 test are as follows:

EE1-1.5.1: Valid the gains of Transformer block and attention block.

EE1-1.5.2: Make minor optimization of the network to further improve performance.

EE1-1.5.3: Valid the performance of JVET-AC0156 filter.

And the test results are shown below:

EE1-1.5.1:

RTNN-No Transformer: {-7.32%, -13.27%, -15.09%} in AI configuration, {-8.05%, -14.95%, -15.11%} in RA configuration.

Transformer BD-rate gain: {1.29%, 13.39%, 12.64%} in AI configuration and {1.56%, 10.53%, 9.51%} in RA configuration.

RTNN-No Attention: {-8.74%, -21.70 %, -22.56%} in AI configuration, {-9.55%, -21.54%, -21.25%} in RA configuration.

Attention BD-rate gain: {-0.30%, 1.63%, 2.07%} in AI configuration and {-0.11%, 1.25%, 1.09%} in RA configuration.

RTNN-All have: {-8.46%, -22.86%, -24.01%} in AI configuration, {-9.45%, -22.43%, -21.97%} in RA configuration.

EE1-1.5.2: {-8.85%, -22.99%, -23.99%} in AI configuration.

EE1-1.5.3: {-8.49%, -22.98%, -24.07%} in AI configuration, {-9.23%, -22.47%, -22.31%} in RA configuration.

Introduction

In RTNN, there are two key components: Transformer block and attention block. The transformer block is used to capture non-local correlation between features and extract deep features. The attention block is used to emphasize important intermediate features. However, how much benefit due to transformer block and attention block? To address this issue, we first conduct ablation study on these two components. Then, we make minor optimization for RTNN to further improve its performance. Finally, we verify the test results of JVET-AC0156 to confirm its correctness.

Proposed method

Network architecture

In JVET-AC0156, an NN-based in-loop filter RTNN was proposed for reconstructed frame enhancement. In the feature extraction part and reconstruction part, some changes have been made to reduce the complexity while ensuring performance. The backbone and sub-models of RTNN are shown in Fig. 1. For the luma model, the backbone network includes 3 RABs and 6 TBs, while for the chroma model, the backbone includes 1 RAB and 2 TBs.

Fig. 1 Network architectures of the proposed network backbone. (a) Residual block. (b) Residual attention block (RAB). (c) Backbone. TB: Transformer block.

Based on RTNN, we do minor modification to make it work better. Firstly, in EE1-1.5.1, we have found that the attention block reduces performance of luma component. However, for chroma component, it improves performance. Thus, we remove the attention block for luma model. At the same time, we have found that a residual path for communicating the feature of ResBlock and transformer is beneficial for luma component. Because shallow features contain more detailed information, they are important for the recovery of the Y component. Based on the above observations, we have modified the luma model, as shown in Fig. 2.

Fig. 2 Network architectures of the modified RTNN. (a) Residual block. (b) Modified residual attention block (RAB). (c) Modified backbone. TB: Transformer block. Modifications are displayed by red.

Training

Table 1. Network information of the proposed CNN filter in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | AI (Luma/Chroma): 200 RA(Luma/Chroma): 260 |
|  | Batch size: | 32 |
|  | Training time: | ~150h/luma model, ~70h/chroma model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {12,17,22, 27, 32, 37, 42} |
|  | Loss function: | L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

Table 2: Network information for the NN-based video coding tool in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | NoTrans:1.07M (luma), 0.36M (chroma) NoAtt: 1.27M (luma), 0.43M (chroma) All have: 1.4M (luma), 0.47M (chroma) Modified_RTNN: 1.27M (luma), 0.47M (chroma) |
|  | Total Number of Parameters (All Models) | NoTrans: ~2.90M NoAtt: ~3.40M All have: ~3.80M Modified_RTNN: ~3.48M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | NoTrans: ~11.06M NoAtt: ~12.97M All have: ~14.50M Modified_RTNN: ~13.28M |
|  | Multiply Accumulate (MAC)/pixel | NoTrans:270K (luma,144), 358K (chroma,72) NoAtt: 374K (luma,144), 448K (chroma,72) All have: 393K (luma,144), 482K (chroma,72) Modified_RTNN: 374K (luma,144), 482K (chroma,72) |
| Optional |  |  |
|  | Total Conv. Layers | 54 Depthwise convolution layers 174 Convolution layers (4 models total) |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 (luma) 72x72 (chroma) |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

All the model is evaluated on the top of VTM-11.0_NNVC-3.0 [4] according to the common test conditions (CTCs) defined in [5]. The evaluation is performed in the AI and RA configurations.

Results of EE1-1.5.1

Table 3 Performance of the RTNN-No_Transformer in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -6.52% | -12.72% | -14.54% | -6.30% | -14.72% | -16.52% |
| Class A2 | -6.89% | -14.00% | -11.69% | -7.29% | -14.47% | -10.61% |
| Class B | -6.70% | -10.90% | -14.78% | -6.72% | -14.05% | -17.26% |
| Class C | -7.11% | -13.73% | -16.26% | -7.01% | -16.80% | -18.23% |
| Class E | -9.89% | -16.42% | -18.03% | -9.90% | -16.57% | -19.82% |
| Overall | -7.32% | -13.27% | -15.09% | -7.34% | -15.26% | -16.67% |
| Class D | -7.14% | -13.30% | -16.43% | -6.46% | -16.60% | -18.58% |
| Class F | -4.72% | -11.02% | -10.64% | -4.90% | -12.96% | -12.95% |

Table 4 Performance of the RTNN-No_Transformer in RA configuration.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -7.96% | -10.64% | -13.19% | -7.87% | -13.15% | -14.77% |
| Class A2 | -8.70% | -15.03% | -11.35% | -7.90% | -14.96% | -10.11% |
| Class B | -7.67% | -15.83% | -16.50% | -7.18% | -15.85% | -17.00% |
| Class C | -8.10% | -17.04% | -17.61% | -7.77% | -15.16% | -15.54% |
| Class E |  |  |  |  |  |  |
| Overall | -8.05% | -14.95% | -15.11% | -7.62% | -14.95% | -14.79% |
| Class D | -9.41% | -17.49% | -18.41% | -6.68% | -16.42% | -15.47% |
| Class F | -3.68% | -10.04% | -10.20% | -3.89% | -10.75% | -11.58% |

Table 5 Performance of the RTNN-No_Attention in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -6.64% | -19.63% | -22.57% | -6.71% | -22.63% | -24.48% |
| Class A2 | -8.45% | -23.25% | -25.71% | -8.83% | -25.26% | -25.07% |
| Class B | -8.22% | -22.88% | -20.91% | -8.11% | -24.75% | -23.49% |
| Class C | -8.84% | -18.59% | -21.34% | -8.57% | -21.68% | -23.41% |
| Class E | -11.86% | -24.38% | -23.80% | -11.67% | -25.74% | -26.64% |
| Overall | -8.74% | -21.70% | -22.56% | -8.69% | -23.97% | -24.42% |
| Class D | -8.25% | -19.59% | -21.50% | -7.36% | -22.97% | -23.89% |
| Class F | -5.82% | -15.29% | -14.92% | -6.06% | -17.42% | -17.33% |

Table 6 Performance of the RTNN-No_Attention in RA configuration.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -8.92% | -14.99% | -16.76% | -8.67% | -19.50% | -19.72% |
| Class A2 | -10.35% | -22.14% | -24.11% | -9.10% | -22.81% | -23.75% |
| Class B | -9.41% | -26.48% | -22.57% | -8.19% | -25.83% | -23.52% |
| Class C | -9.61% | -19.84% | -20.82% | -8.61% | -18.40% | -19.02% |
| Class E |  |  |  |  |  |  |
| Overall | -9.55% | -21.54% | -21.25% | -8.58% | -21.98% | -21.61% |
| Class D | -10.65% | -22.21% | -22.31% | -6.96% | -21.28% | -19.39% |
| Class F | -4.54% | -11.89% | -12.04% | -4.67% | -13.38% | -13.50% |

Table 7 Performance of the RTNN-With_Attention_and_Transformer in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -5.65% | -20.81% | -22.90% | -5.45% | -23.92% | -24.69% |
| Class A2 | -7.55% | -25.16% | -27.86% | -8.31% | -27.14% | -27.21% |
| Class B | -8.29% | -24.53% | -22.50% | -8.18% | -26.63% | -24.97% |
| Class C | -8.86% | -19.30% | -22.82% | -8.62% | -22.23% | -24.99% |
| Class E | -11.90% | -24.60% | -25.38% | -11.64% | -26.30% | -28.60% |
| Overall | -8.46% | -22.86% | -24.01% | -8.42% | -25.23% | -25.91% |
| Class D | -8.26% | -20.17% | -22.30% | -7.36% | -23.41% | -24.78% |
| Class F | -5.82% | -16.06% | -15.57% | -5.97% | -18.27% | -18.95% |

Table 8 Performance of the RTNN-With_Attention_and_Transformer in RA configuration.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -8.55% | -15.71% | -16.33% | -8.14% | -20.58% | -18.67% |
| Class A2 | -10.15% | -22.94% | -25.44% | -9.06% | -23.43% | -25.12% |
| Class B | -9.42% | -27.65% | -23.69% | -8.24% | -27.23% | -24.13% |
| Class C | -9.64% | -20.55% | -21.47% | -8.64% | -19.13% | -20.11% |
| Class E |  |  |  |  |  |  |
| Overall | -9.45% | -22.43% | -21.97% | -8.49% | -22.98% | -22.16% |
| Class D | -10.82% | -22.99% | -23.24% | -7.29% | -21.95% | -19.65% |
| Class F | -4.50% | -12.64% | -12.36% | -4.62% | -14.40% | -14.75% |

Experimental results show that the transformer block achieves {1.29%, 13.39%, 12.64%} and {1.56%, 10.53%, 9.51%} BD-rate gain on average for {Y, Cb, Cr} channel in AI and RA configurations, respectively. Meantime, Attention block achieves {-0.30%, 1.63%, 2.07%} and {-0.11%, 1.25%, 1.09%} BD-rate gain on average for {Y, Cb, Cr} channel in AI and RA configurations, respectively. It can be seen that the transformer has a significant overall gain in both luma and chroma. However, the attention module is generally not beneficial to the luma component but is beneficial to the chroma component. At the same time, for low-resolution video sequences (such as Class B, C, D, E, F), the attention module is generally beneficial to the luma component. One possible reason for this is that the content of 4K video frames (Class A1 and A2) is relatively uniform within a coding tree unit (CTU), which can cause the attention mechanism to fail.

Results of EE1-1.5.2

Table 8 Performance of the Modified_RTNN in AI configuration.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -7.16% | -20.21% | -23.01% | -7.05% | -23.52% | -24.69% |
| Class A2 | -8.38% | -25.13% | -28.11% | -8.91% | -27.15% | -27.42% |
| Class B | -8.36% | -24.64% | -22.38% | -8.25% | -26.82% | -24.86% |
| Class C | -8.92% | -19.62% | -22.67% | -8.68% | -22.73% | -24.86% |
| Class E | -11.75% | -25.40% | -25.29% | -11.63% | -26.82% | -28.46% |
| Overall | -8.85% | -22.99% | -23.99% | -8.82% | -25.42% | -25.86% |
| Class D | -8.30% | -20.67% | -23.03% | -7.45% | -23.65% | -25.44% |
| Class F | -5.75% | -15.89% | -14.82% | -6.07% | -18.16% | -18.26% |

In EE1-1.5.2, we modified the architecture of RTNN for luma component. Specifically, we removed the attention module and added a skip connection. That means the modified RTNN is obtained by adding a residual path on No_attention. Experimental results show that the modified RTNN achieves 0.42% BD-rate gain on average in Y-PSNR in AI configuration. Among them, the residual path achieves 0.12% BD-rate gain on average for Y channel. This indicates that introducing shallow features in the reconstruction part is beneficial for luma components.

Conclusions

This contribution firstly performs some ablation studies on RTNN to explore the effects of transformer block and attention block. Then, this contribution conducts some minor optimization to further improve RTNN’s performance. Experimental results show that the transformer has a significant overall gain in both luma and chroma, while the attention module is generally not beneficial to the luma component but is beneficial to the chroma component. At the same time, we have also confirmed the beneficial improvement of RTNN in AI configuration.

Reference

[1]	H. Zhang, C. Jung, Y. Liu and M. Li, “[EE1-related] RTNN: An In-loop Filter Based on Resblock and Transformer,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-AC0156, January 2023.

[2] S. W. Zamir, A. Arora, S. Khan et al., “Restormer: Efficient transformer for high-resolution image restoration,” Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, 2022: 5728-5739.

[3]	D. Ma, F. Zhang, and D. Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

[4]	https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

[5]	S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

[6]	Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, and M. Karczewicz, “EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3,” JVET-Z0113, by teleconference, April 2022.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).