| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0228-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0221 (EE2-4.1: Additional Fixed Filter for ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AD0221 (EE2-4.1: Additional Fixed Filter for ALF). The simulations were carried out based on the common test condition. The results are matched.

# Introduction

The ALF in ECM-8.0 contain 4 kinds of taps: spatial taps, taps based on reconstructed samples before deblocking filter, taps based on fixed filter results and taps based on residual data. In JVET-AD0221, an additional 7x7 diamond shape fixed filter is introduced. Additional taps (blue taps in Figure 1) based on this fixed filter results are introduced to on-line trained ALF to improve the coding efficiency.

Figure 1 Modified filter shape for ALF in JVET-AD0221.

# Simulation Results

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | 0.01% | 102% | 104% |
| Class A2 | -0.07% | 0.03% | 0.03% | 100% | 104% |
| Class B | -0.06% | 0.02% | 0.02% | 101% | 102% |
| Class C | -0.08% | 0.02% | 0.02% | 103% | 103% |
| Class E | -0.07% | -0.01% | -0.01% | 100% | 103% |
| Overall | -0.07% | 0.01% | 0.01% | 101% | 103% |
| Class D | -0.03% | 0.04% | 0.05% | 104% | 102% |
| Class F | -0.10% | 0.03% | 0.03% | 101% | 102% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.01% | -0.04% | 100% | 100% |
| Class A2 | -0.12% | 0.01% | 0.05% | 100% | 100% |
| Class B | -0.03% | 0.02% | 0.01% | 101% | 100% |
| Class C | -0.08% | 0.08% | 0.05% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.03% | 0.02% | 101% | 100% |
| Class D | -0.10% | 0.32% | 0.06% | 108% | 101% |
| Class F | -0.17% | -0.11% | 0.06% | 105% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.35% | 0.02% | 102% | 101% |
| Class C | -0.12% | 0.09% | -0.02% | 103% | 99% |
| Class E | -0.18% | 0.61% | 0.03% | 106% | 100% |
| Overall | -0.11% | 0.04% | 0.01% | 103% | 100% |
| Class D | -0.16% | -0.29% | -0.21% | 108% | 99% |
| Class F | 0.00% | -0.15% | 1.14% | 104% | 100% |