| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0231 |
| --- | --- |

| Title: | Non-EE2: Non-adjacent spatial candidates for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents to use non-adjacent spatial candidates for IBC merge candidate list construction. On top of ECM-8.0, simulation results of the proposed method are reported as below:

AI: Class F {-0.27%, -0.27%, -0.31%; 100%, 101%}, Class TGM {-0.41%, -0.36%, -0.34%; 100%, 103%};

RA: Class F{-0.24%, -0.68%, -0.32%, 100%, 100%}, Class TGM {-0.26%, -0.29%, -0.26%; 100%, 99%}.

Introduction

In ECM, non-adjacent spatial candidates have been used in inter prediction (e.g., affine mode and non-affine mode). It is straightforward to extend the concept of non-adjacent spatial candidates to IBC.

Proposed method

It is proposed to use non-adjacent spatial candidates for IBC merge candidates list construction. As shown in Figure 1, the same pattern of non-spatial merge candidates as regular merge mode is used. Additionally, the restriction that spatial candidates are not used to construct IBC merge candidate list for a 44 CU is removed.

Figure 1. Spatial neighbouring blocks used to derive the non-adjacent spatial candidates for IBC.

Simulation results

The proposed method is implemented on top of ECM-8.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results compared to ECM-8.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | -0.27% | -0.31% | 100% | 101% |
| Class TGM | -0.41% | -0.36% | -0.34% | 100% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.24% | -0.68% | -0.32% | 100% | 100% |
| Class TGM | -0.26% | -0.29% | -0.26% | 100% | 99% |

Conclusions

In this contribution, it is proposed to use non-adjacent spatial candidates for IBC. It is recommended to study the proposed method in EE2.

Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).