| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0233-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous NNPF topics (set 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu1, Ye-Kui Wang1, Chaoyi Lin2, Yue Li1, Junru Li3, Li Zhang1, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | xujizheng@bytedance.com yekui.wang@bytedance.com linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following items:

- Add a constraint on bit depth between the input and output to avoid some getting greater and some getter smaller.

- Skip signalling of chroma related syntax elements when the input is of 4:0:0.

- Add a constrain to require the presence of chroma components in the NNPF output for colourization.

- Skip the signalling of component related syntax elements when the corresponding components are not present.

- Fix an asserted bug on value assignment of StrengthControlVal, by setting StrengthControlVal equal to ( SliceQpY + 48 ) ÷ 111, instead of SliceQpY ÷ 63, where SliceQpY is that of the first slice of currCodedPic.

- Signal VUI parameter changes due to NNPF. Specifically, when the purpose indicates colourization and the output is of the 4:2:0 chroma format, the location of chroma samples is signalled, and when colour description is changed, whether the output is of full range is signalled.

# On bit depth constraint between NNPF input and output

## Problem statement

In the current design, the bit depth of NNPF input (luma and chroma) and output (luma and chroma) can be specified in the NNPFC SEI message. Some weird setting may happen, e.g. for one component, the input has a greater bit depth than the output, while for the other component, the input has a smaller bit depth than the output. Therefore a constraint is proposed to avoid such weird settings.

## Proposed change

We propose to add a bit depth constraint between the input and output.

In the following, added or modified parts are highlighted in turquoise and deleted parts are highlighted in yellow strikethrough in red fonts. Unless indicated otherwise, the text changes are based on the document JVET-AC2032-v2.docx. It is also applied to other sessions of this document.

It is required that, when the bit depth of one colour component of the network output is greater than the bit depth of the corresponding colour component of the network input, the bit depth of each of the other colour components of the network output shall be greater than or equal to the bit depth of the corresponding colour component of the network input.

# On signalling of chroma related syntax elements when the input is of 4:0:0

## Problem statement

Some chroma related syntax elements are still signalled even when the input is of 4:0:0 chroma format, which may cause confusion.

## Proposed change

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( ChromaFormatIdc ! = 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| ... |  |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| if( ChromaFormatIdc ! = 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| ... |  |

# On output constraint for colourization

## Problem statement

In the current design, when the purpose includes colourization, it is possible to output only luma components, which does not make sense for the colourization purpose.

## Proposed change

It is proposed to disallow only outputting luma components when the purpose includes colourization.

8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 3.

When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0.

...

# On signalling of component related syntax elements

## Problem statement

In the current design, when nnpfc_inp_format_idc and nnpfc_out_format_idc indicate whether there are some colour components, signalling of component related syntax elements even for non-existing components still happens, which does not make sense and may cause confusion.

## Proposed change

It is proposed to skip the signalling of component related syntax elements when the corresponding components are not present.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc ! = 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc ! = 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc ! = 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc ! = 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc ! = 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc ! = 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| ... |  |

# On StrengthControlVal value assignment

## Problem statement

In the current design, StrengthControlVal is an input parameter of NNPFC SEI message. It is set as SliceQpY ÷ 63 of the first slice of currCodedPic. However, SliceQpY can be a negative value when the bit depth is greater than 8, while a negative value is out of the specified range for StrengthControlVal.

## Proposed change

It is proposed to change the value assignment of StrengthControlVal to guarantee that it is always within the valid range. The smallest value of SliceQpY is −48 and the largest value is 63 in the current design.

The following change is based on JVET-AC2005.

D.12.11 Use of the neural network post-filter characteristics SEI message

...

–	StrengthControlVal is set equal to the value of SliceQpY ÷ 63 ( SliceQpY + 48 ) ÷ 111 of the first slice of currCodedPic.

...

# On VUI parameters change due to NNPF

## Problem statement

In the current design, some VUI parameters may be changed by a NNPF, but the changes are not indicated, which may cause confusion.

## Proposed change

It is proposed to signal some VUI parameters when they are changed. Specifically, when the purpose indicates colourization and the output is of the 4:2:0 chroma format, the location of chroma samples is signalled, and when colour description is changed, whether the output is of full range is signalled.

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) { |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( nnpfc_out_colour_format_ idc = = 1 ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| } |  |
| ... |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| ... |  |

8.28.2 Neural-network post-filter characteristics SEI message semantics

...

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_chroma_sample_loc_type_frame, when not equal to 6 and nnpfc_out_colour_format_idc is equal to 1 (4:2:0 colour format), specifies the location of chroma samples of the output pictures, as shown in Figure 1. nnpfc_chroma_sample_loc_type_frame equal to 6 and nnpfc_out_colour_format_idc is equal to 1 (4:2:0 colour format) indicates that the location of the chroma samples is unknown or unspecified or specified by other means not specified in this Specification. The value of nnpfc_chroma_sample_loc_type_frame shall be in the range of 0 to 6, inclusive.

...

nnpfc_separate_colour_description_present_flag equal to 1 indicates that a distinct combination of colour primaries, transfer characteristics, and matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is specified in the SEI message syntax structure. nnpfc_separate_colour_description_present_flag equal to 0 indicates that the combination of colour primaries, transfer characteristics, and matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is the same as indicated in VUI parameters for the CLVS.

nnpfc_colour_primaries has the same semantics as specified in subclause 7.3 for the vui_colour_primaries syntax element, except as follows:

–	nnpfc_colour_primaries specifies the colour primaries of the picture resulting from applying the NNPF specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When nnpfc_colour_primaries is not present in the NNPFC SEI message, the value of nnpfc_colour_primaries is inferred to be equal to vui_colour_primaries.

nnpfc_transfer_characteristics has the same semantics as specified in subclause 7.3 for the vui_transfer_characteristics syntax element, except as follows:

–	nnpfc_transfer_characteristics specifies the transfer characteristics of the picture resulting from applying the NNPF specified in the SEI message, rather than the transfer characteristics used for the CLVS.

–	When nnpfc_transfer_characteristics is not present in the NNPFC SEI message, the value of nnpfc_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

nnpfc_matrix_coeffs has the same semantics as specified in subclause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	nnpfc_matrix_coeffs specifies the matrix coefficients of the picture resulting from applying the NNPF specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for nnpfc_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

–	When nnpfc_matrix_coeffs is equal to 0, nnpfc_out_order_idc shall not be equal to 1 or 3.

nnpfc_full_range_flag has the same semantics as specified in subclause 7.3 for the vui_full_range_flag syntax element, except as follows:

–	nnpfc_full_range_flag specifies the scaling and offset values applied in association with the matrix coefficients of the picture resulting from applying the NNPF specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When nnpfc_colour_primaries is not present in the NNPFC SEI message, the value of nnpfc_full_range_flag is inferred to be equal to vui_full_range_flag.

...

# Patent rights declaration

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).