| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 30th Meeting: Antalya, TR, 21–28 April 2023 | Document: JVET-AD0235-v2 |
| --- | --- |

| Title: | Non-EE2: Enhanced subblock-based motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to enhance subblock-based motion compensation in three aspects. Firstly, an interweaved prediction method is presented for affine motion compensation (AMC). Secondly, Regression-     based Motion Vector Field (RMVF) candidate derivation is enhanced by allowing the MVF from multiple      CUs as input. Thirdly, it is proposed to determine whether to skip the OBMC operation for each boundary subblock independently. It is reported that on top of ECM-8.0, simulation results of the proposed method are as below:

RA: -0.12 % (Y), -0.07 % (U), -0.12 % (V), 106% (EncT), 98% (DecT)

LB: -0.07 % (Y), -0.03 % (U), 0.03 % (V), 113% (EncT), 98% (DecT)

- Introduction

Affine motion compensation (AMC) is an important inter tool which can provide subblock-level motion information. It faces a dilemma on the size of subblocks. With smaller subblocks, AMC can achieve a better coding performance but suffers a higher complexity burden. In ECM-8.0, when OBMC is applied, the minimum affine subblock size is set to be 4×4 for luma component. Otherwise, it is set to be 1×1.

In ECM, Regression-based Motion Vector Field (RMVF) affine merge candidates are derived and added to the affine merge list, where MVF from one previously coded affine CU and adjacent subblocks of a current CU are used as the input to the regression process.

When OBMC is applied, top and left boundaries of a CU can be refined with a weighted prediction using neighboring motion information, which is operated upon each 4×4 subblock. As shown in Fig. 1, suppose B(0, 0) is the subblock to be processed, if upper neighboring subblock B(-1, 0) and N successive subblock {B(-1, 1),…, B(-1, N)} have identical motion (termed as Mnei) which is different from B(0, 0)’s motion M(0, 0), then {B(0,  0),…, B(0, N)} will be simultaneously processed with Mnei. Otherwise, if  Mnei is identical to M(0, 0), {B(0,  0),…, B(0, N)} will skip OBMC operation. It is asserted such a strategy may lead to inappropriate skips for subblock based coding modes.

Fig. 1. Illustration of OBMC operation on B(0,  0)

- Proposed methods

In this proposal, three aspects are presented to enhance subblock motion compensation:

Aspect #1: Interweaved affine prediction.

Aspect #2: RMVF candidate derivation with multiple CUs as input.

Aspect #3: Determining of the usage of OBMC for each boundary subblock independently.

  - Interweaved affine prediction

Interweaved affine prediction was proposed in JVET-M0268 [1] as shown in Fig. 2. With the interweaved prediction, a coding block is divided into subblocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted sum of the two auxiliary predictions.

First, a coding block is divided into subblocks with two different dividing patterns. The first dividing pattern is the same as that in ECM as pattern 0 shown in Fig. 2, while the second dividing pattern also divides the coding block into 4×4 subblocks but with a 2×2 offset as pattern 1 shown in Fig. 2.

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each subblock in a dividing pattern is derived from CPMVs by the affine model.

The final prediction is calculated as a weighted sum of the two auxiliary predictions, formulated as:

| . | (1) |
| --- | --- |

As shown in Fig. 3, an auxiliary prediction sample located at the center of a subblock is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a subblock is associated with a weighting value 1.

The subblocks associated with two dividing patterns are respectively refined by PROF unless the width or height is smaller than 4. Besides, subblock boundary deblocking/OBMC for affine mode is disabled.

Fig. 2. Interweaved prediction procedure.

Fig. 3. Weighting values in a subblock.

  - RMVF candidate derivation with multiple CUs

It is proposed to derive additional RMVF affine candidates by feeding the MVF of multiple CUs as regression input. In particular, MVF of M (M=2 in the test) previously affine-coded CUs are simultaneously used to generate a RMVF candidate. The additional RMVF candidates are reordered together with other RMVF candidates through ARMC.

  - OBMC modification

For subblock-based coding, it is proposed to determine whether OBMC is skipped or not depends only on the adjacent left or upper subblock for each boundary subblock independently. For a top (or left) boundary subblock, if the motion information of the current and adjacent upper (or left) subblock is identical, then OBMC is skipped. This method can be applied to subblock-based coding mode such as affine, SbTMVP, and multi-pass DMVR.

- Experimental results

The proposed method has been implemented on top of ECM-8.0 [2], the joint solution of the proposed method and EE2-2.4 is also tested. Results under RA/LDB configurations are presented in Table 1 and Table 2 respectively. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.11% | -0.12% | 105% | 95% |  |  |  |  |  |
| Class A2 | -0.26% | -0.14% | -0.11% | 106% | 95% |  |  |  |  |  |
| Class B | -0.08% | -0.11% | -0.18% | 107% | 100% | -0.07% | -0.56% | -0.16% | 111% | 96% |
| Class C | -0.07% | -0.10% | -0.06% | 106% | 101% | -0.07% | -0.16% | -0.07% | 111% | 100% |
| Class E |  |  |  |  |  | -0.05% | 1.02% | 0.48% | 118% | 98% |
| Overall | -0.12% | -0.07% | -0.12% | 106% | 98% | -0.07% | -0.03% | 0.03% | 113% | 98% |
| Class D | -0.05% | 0.04% | 0.07% | 106% | 102% | -0.14% | -1.25% | -0.61% | 112% | 99% |
| Class F | -0.41% | -0.48% | -0.47% | 105% | 100% | -0.17% | 0.23% | 1.13% | 110% | 97% |

Table 2: Coding performance of the joint solution with EE2-2.4.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.11% | -0.10% | -0.12% | 82% | 116% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.05% | -0.02% | -0.18% | 80% | 107% | -0.19% | -0.92% | -0.48% | 86% | 121% |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Conclusions

Enhanced subblock motion compensation is presented in this contribution. Simulation results indicate 0.12%/0.07% coding gain in RA/LDB configurations with 106%/98% and 113%/98% complexity increase. It is recommended to include the proposed methods in the next round of EE2.

- References

[1] K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “Non-CE2: Interweaved Prediction for Affine Motion Compensation,” JVET-M0268, Jan. 2019.

[2] ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).