| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0236-v0 |
| --- | --- |

| Title: | Non-EE2: On Classification of In-Loop Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In this contribution, three additional rules for classification of in-loop filters are proposed, including deblocking filter-boundary strength (DBF-BS) based classification for ALF, DBF-BS based classification for SAO and variance based classification for biliteral filter (BF).

On top of ECM-8.0, simulation results of the proposed methods are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

- Introduction

In ALF of ECM-8.0, there are two classifiers which are band-based and texture-based, respectively. The band-based classification divides the sample value range into 25 groups. The texture-based classification considers the gradient information to classify each unit into 5 texture directions and 5 texture strengths, resulting in 25 classes in total too.

In SAO of ECM-8.0, there are also two classifiers which are band-based and edge-based. The band-based classifier divides the sample value range into 32 groups and selects the best 4 bands for signaling the offsets. The edge-based classifier uses neighbor samples to derive 4 edge patterns and signals the corresponding offsets for a selected pattern.

In bilateral filter (BF) of ECM-8.0, there is no TU level classification. The filter strength is only related to prediction mode, size and QP of current TU. The texture strength inside a TU is not considered.

- Proposed Method

In this contribution, three additional rules for classification of in-loop filters are proposed.

- For ALF, the deblocking filter-boundary strength (DBF-BS) is introduced into both band-based and texture-based classifiers. The DBF-BS in considered into 2 levels (boundary and non-boundary) for each classification unit. Besides the existing classification methods, the DBF-BS is also considered when generating the final classification results for each classification unit.

- For SAO, the DBF-BS is also involved in both band-based and edge-based classifiers. There are 2 levels of DBF-BS for each sample position. In the training data collection and filtering process, the DBF-BS is taken into consideration. For each band or edge pattern, the number of signaled offsets is increased from 4 to 8, each 4 of 8 corresponds to a different DBF-BS level.

- For BF, the variance is utilized to identify the texture/content strength for each TU. More filter strengths are introduced into BF. Different texture strengths lead to different filtering strengths.

- Simulation Results

The proposed method was implemented on top of the ECM-8.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-8.0 are shown in TABLE 1.

TABLE 1 Performance of the Proposed Method

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  | -0.03% | -0.03% | -0.04% |  |  |  |  |  |  |  |
| Class C | -0.03% | -0.11% | -0.01% |  |  | -0.24% | 0.01% | 0.14% |  |  | -0.25% | 0.05% | 0.44% |  |  |
| Class E | -0.04% | -0.03% | -0.04% |  |  |  |  |  |  |  | -0.27% | 1.97% | -0.57% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.03% | -0.11% | -0.03% |  |  | -0.08% | 0.11% | -0.19% |  |  | -0.07% | -0.52% | 0.74% |  |  |
| Class F |  |  |  |  |  | -0.16% | -0.44% | -0.17% |  |  |  |  |  |  |  |

- Conclusions

In this contribution, three classification improvements for different loop filters are proposed. The proposed methods can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to include the proposed method in the next round of EE2.

- Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

- Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).