| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 |  |  | Document: JVET-AD0241-v1 |  |
| --- | --- | --- | --- | --- |
| Title: | AHG13: On Film Grain Subjective Metrics |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Informative Report |  |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Wenhao Zhang, Scott Labrozzi | Email: |  | xuewei.meng@disneystreaming.com wenhao.zhang@disneystreaming.com Scott.Labrozzi@disneystreaming.com |
| Source: | Disney Streaming |  |  |  |

_____________________________

# Abstract

The subjective evaluation of film grain synthesis is recommended in the last JVET meeting. However, there are no widely used metrics for the evaluation of film grain synthesis results, which highly limits the development of film grain related technologies. This document provides the evaluation of metrics, i.e., NSS (Natural Scene Statistics), for the film grain synthesis. The main observations and conclusions are as follows,

- For images with relatively simple content, the NSS is highly correlated with the subjective perception of the human eye.

- For images with complex content and film grain characteristics, the NSS has a poor relationship with subject results.

- For videos, the NSS shows even worse results.

- NSS is not suitable to be the film grain synthesis objective metric of videos.  An improved NSS or other methods are recommended to be studied for the film grain evaluation.

# 1. 	Introduction

In the entertainment industry, film grain is widely present in TV and motion pictures. Although digital cameras do not produce film grain, film grain is often added to the captured videos for creative intent. When encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Fig. 1.

Figure 1. A typical framework of Film Grain Synthesis.

Although film grain synthesis has been regarded as one of the most important features in the state-of-the-art video coding standards, such as H.266/VVC, and AV1. There is no widely used subjective metrics to evaluate the quality of synthesized grainy video, which highly limits the development of film grain related technologies.

In this proposal, we evaluate the reliability of metrics, i.e., NSS (Natural Scene Statistics), for the film grain synthesis.

# 2. 	Test Conditions [1]

## 2.1 Test Platform

Anchor: VTM-19.0

Film grain synthesis methods:

- Frequency-domain film grain synthesis method in VTM-19.0

- Auto-regressive model

## 2.2 Test Sequences and Configuration

Test configuration: Random Access (RA) with GOP32

Test QP: 22, 27, 32, 37

Table 1. Test sequences.

|  | Sequences | Resolution | Bitdepth | Frame rate | Chroma Format | Frame Number |
| --- | --- | --- | --- | --- | --- | --- |
| JVET | oldTownCross | 1920x1080 | 8 | 50 | YUV420 | 500 |
|  | DinnerScene | 4096x2160 | 10 | 60 | YUV420 | 600 |
| Disney | PrinceOfPersia | 1920x1080 | 10 | 24 | YUV420 | 240 |
|  | StarwarESB | 3840x2160 | 10 | 24 | YUV420 | 240 |
|  | TheMandalorian | 3840x2160 | 10 | 24 | YUV420 | 240 |
|  | TheHandmaid | 3840x2160 | 10 | 24 | YUV420 | 240 |

The examples of sequence “oldTownCross” and “DinnerScene” are shown in Fig. 2 and Fig. 3. The video content of sequences from Disney will be available when Disney Legal and Copyright approves.

Figure 2. A frame of test sequence “oldTownCross”.

Figure 3. A frame of test sequence “DinnerScene”.

## 2.3 Frequency-domain film grain synthesis configuration

In this proposal, we evaluate the frequency-domain film grain synthesis implemented in the current VTM-19.0. And the parameters are set as shown in Table 2.

Table 2. Frequency-domain film grain synthesis configuration.

| Parameters | Settings |
| --- | --- |
| SEIFGCEnabled | 1 |
| SEIFGCAnalysisEnabled | 1 |
| SEIFGCCancelFlag | 0 |
| SEIFGCPersistenceFlag | 1 |
| SEIFGCPerPictureSEI | 0 |
| SEIFGCModelID | 0 |
| SEIFGCSepColourDescPresentFlag | 0 |
| SEIFGCBlendingModeID | 0 |
| SEIFGCCompModelPresentComp0 | 1 |
| SEIFGCCompModelPresentComp1 | 0 |
| SEIFGCCompModelPresentComp2 | 0 |

## 2.4 Auto-regressive film grain synthesis configuration

The auto-regressive film grain synthesis is also evaluated in the VTM-19.0. And the parameters are set as shown in Table 3.

Table 3. Auto-regressive film grain synthesis configuration.

| Parameters | Settings |
| --- | --- |
| Block_size in flat block detection | 32x32 |
| Pattern size | 64x64 |

# 3.      Review of NSS [1]

Spatial domain NSS features are extracted from local image patches that effectively capture the essential low-order statistics of natural images. The conventional spatial NSS model that we use begins by pre-processing the image by local mean removal and divisive normalization,

INSSi,j=Ii,j-μ(i,j)σi,j+1,

where i∈1,2,…,M, j∈{1,2,…,N} are indices in the spatial domain. M and N represent the width and height of image, respectively. Ii,j is the pixel value of sample with location i,j. μ(i,j) which represents the local mean is calculated by,

μi,j=k=-KKl=-LLwk,lIi+k,j+l,

where w={wk,l|k=-K,…,K;l=-L,…,L} is a 2D circularly-symmetric Gaussian weighting function sampled out to 3 standard deviations (K=L=3) and rescaled to unit volume. σi,j which represents local contrast is derived by,

σi,j=k=-KKl=-LLwk,l[Ii+k,j+l- μi,j]2.

The product of diagonally adjacent coefficients captures local spatial correlations,

IPMSCNi,j= INSSi,jINSSi+1,j+1.

The goal is to compare the statistics of source video and film grain synthesized video. To do this, we use the Jensen Shannon divergence  computed on the empirical distributions of IPMSCNi,j, which denoted by JSDNSS. A smaller JSDNSSvalue indicates that two distributions are more similar.

# 4. 	Evaluation Results

oldTownCross

For the whole video frame, the NSS histograms of different film grain parameters look quite similar to each other. And the best JSDNSS score is not the best result with the highest subjective quality.

For the cropped video frame with relatively simple content, the JSDNSS score is highly correlated with the subjective perception of the human eye.

Figure 4. The first frame of test sequence “oldTownCross”.

Figure 5. The NSS histogram of the first frame of “oldTownCross”.

Table 4. The JSDNSS score of the first frame of “oldTownCross”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.0 | 0.0215 | 0.0192 | 0.0174 | 0.0165 | 0.0162 | 0.0166 | 0.018 | 0.0197 | 0.0219 |
| S0 | 0.0215 | 0.0 | 0.0096 | 0.0114 | 0.0147 | 0.0176 | 0.0208 | 0.0242 | 0.0276 | 0.0308 |
| S1 | 0.0192 | 0.0096 | 0.0 | 0.009 | 0.0116 | 0.0144 | 0.0174 | 0.0206 | 0.0239 | 0.027 |
| S2 | 0.0174 | 0.0114 | 0.009 | 0.0 | 0.0092 | 0.0117 | 0.0141 | 0.0172 | 0.0202 | 0.0234 |
| S3 | 0.0165 | 0.0147 | 0.0116 | 0.0092 | 0.0 | 0.0094 | 0.0115 | 0.0142 | 0.0168 | 0.02 |
| S4 | 0.0162 | 0.0176 | 0.0144 | 0.0117 | 0.0094 | 0.0 | 0.009 | 0.0111 | 0.0138 | 0.0167 |
| S5 | 0.0166 | 0.0208 | 0.0174 | 0.0141 | 0.0115 | 0.009 | 0.0 | 0.0093 | 0.0112 | 0.0135 |
| S6 | 0.018 | 0.0242 | 0.0206 | 0.0172 | 0.0142 | 0.0111 | 0.0093 | 0.0 | 0.0092 | 0.0113 |
| S7 | 0.0197 | 0.0276 | 0.0239 | 0.0202 | 0.0168 | 0.0138 | 0.0112 | 0.0092 | 0.0 | 0.009 |
| S8 | 0.0219 | 0.0308 | 0.027 | 0.0234 | 0.02 | 0.0167 | 0.0135 | 0.0113 | 0.009 | 0.0 |

Figure 6. The NSS histogram of the 11th  frame of “oldTownCross”.

Table 5. The JSDNSS score of the 11th  frame of “oldTownCross”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.0 | 0.0683 | 0.0572 | 0.0487 | 0.0417 | 0.036 | 0.0316 | 0.0294 | 0.0272 | 0.0268 |
| S0 | 0.0683 | 0.0 | 0.0156 | 0.0253 | 0.0338 | 0.0415 | 0.0482 | 0.0545 | 0.06 | 0.0651 |
| S1 | 0.0572 | 0.0156 | 0.0 | 0.0139 | 0.022 | 0.0297 | 0.036 | 0.0425 | 0.0479 | 0.0528 |
| S2 | 0.0487 | 0.0253 | 0.0139 | 0.0 | 0.013 | 0.0203 | 0.0265 | 0.0328 | 0.0383 | 0.0431 |
| S3 | 0.0417 | 0.0338 | 0.022 | 0.013 | 0.0 | 0.0133 | 0.0186 | 0.0249 | 0.0301 | 0.0349 |
| S4 | 0.036 | 0.0415 | 0.0297 | 0.0203 | 0.0133 | 0.0 | 0.0116 | 0.0171 | 0.0224 | 0.0272 |
| S5 | 0.0316 | 0.0482 | 0.036 | 0.0265 | 0.0186 | 0.0116 | 0.0 | 0.0122 | 0.0164 | 0.0208 |
| S6 | 0.0294 | 0.0545 | 0.0425 | 0.0328 | 0.0249 | 0.0171 | 0.0122 | 0.0 | 0.0117 | 0.0156 |
| S7 | 0.0272 | 0.06 | 0.0479 | 0.0383 | 0.0301 | 0.0224 | 0.0164 | 0.0117 | 0.0 | 0.0108 |
| S8 | 0.0268 | 0.0651 | 0.0528 | 0.0431 | 0.0349 | 0.0272 | 0.0208 | 0.0156 | 0.0108 | 0.0 |

Figure 7. The cropped first frame of test sequence “oldTownCross”.

Figure 8. The NSS histogram of the cropped first frame of “oldTownCross”.

Table 6. The JSDNSS score of the cropped first frame of “oldTownCross”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.0 | 0.0291 | 0.0238 | 0.0201 | 0.0176 | 0.019 | 0.0212 | 0.0241 | 0.0284 | 0.0318 |
| S0 | 0.0291 | 0.0 | 0.0152 | 0.0195 | 0.0253 | 0.031 | 0.0368 | 0.0423 | 0.0473 | 0.0519 |
| S1 | 0.0238 | 0.0152 | 0.0 | 0.0147 | 0.0193 | 0.0247 | 0.03 | 0.0353 | 0.0403 | 0.0448 |
| S2 | 0.0201 | 0.0195 | 0.0147 | 0.0 | 0.0146 | 0.0189 | 0.0239 | 0.0284 | 0.0337 | 0.0381 |
| S3 | 0.0176 | 0.0253 | 0.0193 | 0.0146 | 0.0 | 0.0147 | 0.0185 | 0.0227 | 0.0274 | 0.0318 |
| S4 | 0.019 | 0.031 | 0.0247 | 0.0189 | 0.0147 | 0.0 | 0.0155 | 0.0184 | 0.0227 | 0.0264 |
| S5 | 0.0212 | 0.0368 | 0.03 | 0.0239 | 0.0185 | 0.0155 | 0.0 | 0.0148 | 0.0181 | 0.0214 |
| S6 | 0.0241 | 0.0423 | 0.0353 | 0.0284 | 0.0227 | 0.0184 | 0.0148 | 0.0 | 0.0144 | 0.0173 |
| S7 | 0.0284 | 0.0473 | 0.0403 | 0.0337 | 0.0274 | 0.0227 | 0.0181 | 0.0144 | 0.0 | 0.0144 |
| S8 | 0.0318 | 0.0519 | 0.0448 | 0.0381 | 0.0318 | 0.0264 | 0.0214 | 0.0173 | 0.0144 | 0.0 |

Figure 9. The cropped 11th frame of test sequence “oldTownCross”.

Figure 10. The NSS histogram of the cropped 11th frame of “oldTownCross”.

Table 7. The JSDNSS score of the cropped 11th frame of “oldTownCross”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.0 | 0.1279 | 0.1004 | 0.0792 | 0.0626 | 0.05 | 0.0405 | 0.035 | 0.032 | 0.0327 |
| S0 | 0.1279 | 0.0 | 0.0353 | 0.0593 | 0.079 | 0.0949 | 0.1078 | 0.1192 | 0.1287 | 0.1368 |
| S1 | 0.1004 | 0.0353 | 0.0 | 0.0291 | 0.0484 | 0.0647 | 0.0777 | 0.0892 | 0.0991 | 0.1074 |
| S2 | 0.0792 | 0.0593 | 0.0291 | 0.0 | 0.0247 | 0.0414 | 0.0548 | 0.0663 | 0.076 | 0.0846 |
| S3 | 0.0626 | 0.079 | 0.0484 | 0.0247 | 0.0 | 0.0237 | 0.0362 | 0.0475 | 0.0573 | 0.0658 |
| S4 | 0.05 | 0.0949 | 0.0647 | 0.0414 | 0.0237 | 0.0 | 0.021 | 0.0321 | 0.042 | 0.0509 |
| S5 | 0.0405 | 0.1078 | 0.0777 | 0.0548 | 0.0362 | 0.021 | 0.0 | 0.0191 | 0.0288 | 0.0376 |
| S6 | 0.035 | 0.1192 | 0.0892 | 0.0663 | 0.0475 | 0.0321 | 0.0191 | 0.0 | 0.0185 | 0.0262 |
| S7 | 0.032 | 0.1287 | 0.0991 | 0.076 | 0.0573 | 0.042 | 0.0288 | 0.0185 | 0.0 | 0.0181 |
| S8 | 0.0327 | 0.1368 | 0.1074 | 0.0846 | 0.0658 | 0.0509 | 0.0376 | 0.0262 | 0.0181 | 0.0 |

Figure 11. The cropped 11th frame of “oldTownCross”. (version 2)

Figure 12. The NSS histogram of the cropped 11th frame of “oldTownCross”.

Table 8. The JSDNSS score of the cropped 11th frame of “oldTownCross”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.0 | 0.3664 | 0.2832 | 0.1829 | 0.1039 | 0.0463 | 0.0163 | 0.0373 | 0.0612 | 0.0812 |
| S0 | 0.3664 | 0.0 | 0.1331 | 0.2349 | 0.3012 | 0.3443 | 0.3736 | 0.3952 | 0.411 | 0.4237 |
| S1 | 0.2832 | 0.1331 | 0.0 | 0.1224 | 0.2027 | 0.2548 | 0.2899 | 0.3154 | 0.3343 | 0.3492 |
| S2 | 0.1829 | 0.2349 | 0.1224 | 0.0 | 0.0879 | 0.147 | 0.1874 | 0.2167 | 0.2386 | 0.2559 |
| S3 | 0.1039 | 0.3012 | 0.2027 | 0.0879 | 0.0 | 0.0624 | 0.1057 | 0.1376 | 0.1616 | 0.1806 |
| S4 | 0.0463 | 0.3443 | 0.2548 | 0.147 | 0.0624 | 0.0 | 0.0451 | 0.0783 | 0.1034 | 0.1235 |
| S5 | 0.0163 | 0.3736 | 0.2899 | 0.1874 | 0.1057 | 0.0451 | 0.0 | 0.0344 | 0.0601 | 0.0808 |
| S6 | 0.0373 | 0.3952 | 0.3154 | 0.2167 | 0.1376 | 0.0783 | 0.0344 | 0.0 | 0.027 | 0.0478 |
| S7 | 0.0612 | 0.411 | 0.3343 | 0.2386 | 0.1616 | 0.1034 | 0.0601 | 0.027 | 0.0 | 0.0222 |
| S8 | 0.0812 | 0.4237 | 0.3492 | 0.2559 | 0.1806 | 0.1235 | 0.0808 | 0.0478 | 0.0222 | 0.0 |

PrinceOfPersia (Picture and thumbnails will be available when Disney Legal and Copyright approves.)

The NSS histogram and JSDNSS can well distinguish different parameter settings, including film grain strength and cut-off frequency. And the parameter corresponding to the smallest JSDNSS  has the highest subjective quality.

Figure 13. The NSS histogram of the cropped 16th frame of “PrinceOfPersia”.

Table 9. The JSDNSS score of the cropped 16th frame of “PrinceOfPersia”.

|  | Source | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 0.000 | 0.366 | 0.283 | 0.183 | 0.104 | 0.046 | 0.016 | 0.037 | 0.061 | 0.000 |
| S0 | 0.366 | 0.000 | 0.133 | 0.235 | 0.301 | 0.344 | 0.374 | 0.395 | 0.411 | 0.366 |
| S1 | 0.283 | 0.133 | 0.000 | 0.122 | 0.203 | 0.255 | 0.290 | 0.315 | 0.334 | 0.283 |
| S2 | 0.183 | 0.235 | 0.122 | 0.000 | 0.088 | 0.147 | 0.187 | 0.217 | 0.239 | 0.183 |
| S3 | 0.104 | 0.301 | 0.203 | 0.088 | 0.000 | 0.062 | 0.106 | 0.138 | 0.162 | 0.104 |
| S4 | 0.046 | 0.344 | 0.255 | 0.147 | 0.062 | 0.000 | 0.045 | 0.078 | 0.103 | 0.046 |
| S5 | 0.016 | 0.374 | 0.290 | 0.187 | 0.106 | 0.045 | 0.000 | 0.034 | 0.060 | 0.016 |
| S6 | 0.037 | 0.395 | 0.315 | 0.217 | 0.138 | 0.078 | 0.034 | 0.000 | 0.027 | 0.037 |
| S7 | 0.061 | 0.411 | 0.334 | 0.239 | 0.162 | 0.103 | 0.060 | 0.027 | 0.000 | 0.061 |
| S8 | 0.000 | 0.366 | 0.283 | 0.183 | 0.104 | 0.046 | 0.016 | 0.037 | 0.061 | 0.000 |

Figure 14. The NSS histogram of the cropped 16th frame of “PrinceOfPersia”.

Table 10. The JSDNSS score of the cropped 16th frame of “PrinceOfPersia”.

|  | Source | S0 | S1 | S2 | S3 |
| --- | --- | --- | --- | --- | --- |
| Source | 0.000 | 0.242 | 0.123 | 0.017 | 0.099 |
| S0 | 0.242 | 0.000 | 0.139 | 0.245 | 0.324 |
| S1 | 0.123 | 0.139 | 0.000 | 0.121 | 0.215 |
| S2 | 0.017 | 0.245 | 0.121 | 0.000 | 0.104 |
| S3 | 0.099 | 0.324 | 0.215 | 0.104 | 0.000 |

# 5. 	Observations and Conclusions

The main observations and conclusions are as follows,

- For images with relatively simple content, the NSS is highly correlated with the subjective perception of the human eye.

- For images with complex content and film grain characteristics, the NSS has a poor relationship with subject results.

- For videos, the NSS shows even worse results.

# 6．  References

[1] X. Meng, W. Zhang, S. Labrozzi. AHG13: On Film Grain Synthesis Subjective Evaluation. JVET-AC0199, 29th JVET meeting, teleconference, 11-20 January 2023.

[2] Mittal, A., Soundararajan, R., & Bovik, A. C. (2012). Making a “completely blind” image quality analyzer. IEEE Signal processing letters, 20(3), 209-212.

# 7. 	Patent rights declaration(s)

Disney Streaming, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.