| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0244-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0199 (EE2-3.2: IBC MBVD list derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Ruiz Coll | Tel: Email: | duizcoll@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AD0199 (EE2-3.2: IBC MBVD list derivation) contribution, which proposes to derive the IBC MBVD list from the sample positions along the MBVD directions. The MBVD candidates search starts with checking template SAD costs of offsets added to BVP along each direction with the interval of M-pel. The search process repeats around the selected candidates having the lowest TM cost with the half interval. The MBVD list is derived when the interval reaches 1-pel, the candidates with the lowest TM cost are included into the final list. The BD-rate results available in this crosscheck report match those provided by the proponent, and the source code implementation is conforming the technical description provided in JVET-AD0199.

# Introduction

JVET-AD0199 proposes to allow adaptive BVD offsets along MBVD directions. A MBVD list of K candidates with lowest template SAD costs are derived as following steps:

- Denote the largest offset as N-pel, e.g. N=256, the number of directions D, e.g. D=4 (left, right, top, and bottom), the starting search interval M-pel, e.g. M=8, the number of candidates in MBVD list is K, e.g. K=8.

- Along each direction, check the TM SAD cost for an offset of every M-th position not exceeding N. The K lowest TM SAD cost candidates are kept in the list.

- For each candidate in the list, check the TM SAD cost of the two candidates with the offset equal to +-M/2 along the direction. The K lowest TM SAD cost candidates are kept in the list.

- Repeat step 3, while reducing the interval M by half until it reaches 1-pel.

When a MBVD candidate has a left or above template outside of the BV search area, the MBVD candidate is set to invalid candidate. When two BVP candidates have same BV component in horizontal or vertical direction, the BV difference in other direction has to be larger than a threshold T, e.g. T = 8-pel.

A Crosscheck of the source code and simulation results has been conducted. Inspected source code follows the description in the proposal. The crosscheck has been performed on top of ECM-8.0 in AI and RA configurations. Two tests have been performed according with the parameters proposed by the proponent:

|  | N (largest offset) | M (starting search interval) | T (threshold) |
| --- | --- | --- | --- |
| Test 3.2a | 128-pel | 4-pel | 6-pel |
| Test 3.2b | 256-pel | 4-pel | 32-pel |

The rest of the parameters are kept constant: D=4 and K=8.

The crosscheck results are provided in the tables below and match the test results provided by the proponent. Note that coding time may not be accurate. Below are the results for EE2-3.2a and EE2-3.2b tests:

Test 3.2a (N = 128-pel, M = 4-pel, T = 6-pel)

Table I. Simulations results of EE2-3.2a (N = 128-pel, M = 4-pel, T = 6-pel) on top of ECM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | -0.39% | -0.37% | 100% | 101% |
| Class TGM | -0.54% | -0.51% | -0.48% | 100% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.42% | -0.12% | 100% | 100% |
| Class TGM | -0.44% | -0.32% | -0.32% | 100% | 102% |

Test-3.2b (N = 256-pel, M = 4-pel, T = 32-pel)

Table II. Simulations results of EE2-3.2a (N = 256-pel, M = 4-pel, T = 32-pel) on top of ECM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.40% | -0.48% | -0.54% | 102% | 105% |
| Class TGM | -0.71% | -0.67% | -0.68% | 102% | 107% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.57% | -0.39% | 100% | 101% |
| Class TGM | -0.58% | -0.45% | -0.48% | 101% | 101% |

# Conclusion

Results reported of EE2-Test 3.2a and EE2-Test 3.b in JVET-AD0199 (EE2-3.2: IBC MBVD list derivation) match with the simulations results obtained in this crosschecking. It is recommended that one of the tests EE2-3.2a or EE2-3.2b will be adopted in the ECM-9.0.

# Reference

[1]	Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz, “EE2-3.2: IBC MBVD list derivation,” JVET-AD0199, April 2023.

[2]	ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[3]	V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023.

[4]	M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration

Ofinno LLC does not have any current or pending patent rights relating to the technology described in this contribution.