| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0247-v1 |
| --- | --- |

| Title: | EE2-1.20n: Combination Test of Test 1.20f + Test 1.14c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Sean McCarthy (Dolby) Jeeva Raj Arumugam, Ashwin Natesan, Vaibhav Valvaiker, Abhijeet Sarate, Jay N. Shingala (Ittiam) Xinwei Li, Ruling Liao , Jie Chen (Alibaba) Fan Wang, Lai Zhang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Yanzhuo Ma, Junyan Huo, Hongqing Du, Hongli Zhang, Zhenyao Zhang, Wenhan Qiao, Fuzheng Yang (Xidian) | Tel: Email: | {jeeva.raj, jay.shingala }@ittiam.com {fpu, tlu, pyin}@dolby.com {sid.lxw, ruling.lrl, jiechen.cj}@alibaba-inc.com wangfan6@oppo.com yzma@mail.xidian.edu.cn |
| Source: | Dolby Laboratories, Inc., Ittiam System, Alibaba group, OPPO, Xidian University |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 IntraTMP combination Test 1.20n. This is the combination test of Test 1.20f (combination of Test 1.12 which tests intra TMP with sub-pel precision and Test 1.13b which tests adaptive intra TMP fusion with up to two predicted blocks using MTMP search) and Test 1.14c (combination of Test 1.14a which extend the adjacent search area of Intra TMP and Test 1.14b which enlarge the search range for small blocks of Intra TMP). The impact on coding efficiency and runtimes over ECM-8.0 is reportedly {for Y, U, V, EncT an DecT} as follows:

EE2-1.20n: Test 1.20f + Test 1.14c

AI { -x.xx% Y, -x.xx% U, -x.xx% V, xxx% EncT, xxx% DecT }

RA { -x.xx% Y, -x.xx% U, -x.xx% V, xxx% EncT, xxx% DecT }

# Proposed method

This contribution reports the simulation results of EE2 Test 1.20n which studies the joint combination of Test 1.20f [2] and Test 1.14c [3]. Test 1.20f is combination test of Test 1.12 which tests intra TMP with sub-pel precision and Test 1.13b which tests adaptive intra TMP fusion with up to two predicted blocks using MTMP search. Test 1.14c is combination test of Test 1.14a which extend the adjacent search area of Intra TMP and Test 1.14b which enlarge the search range for small blocks of Intra TMP. The combination method is same as EE2 Test 1.20f with extended search range from Test 1.14c.

# Simulation results

The proposed method is implemented on top of ECM8.0 and evaluated using the common test conditions.

Table 2 Test 1.20f coding performance compared with ECM8.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.74% | -0.77% | -0.73% |  |  |
| Class C | -0.35% | -0.37% | -0.34% |  |  |
| Class E | -1.76% | -1.81% | -1.68% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.62% | -0.77% | -0.74% |  |  |
| Class F | -0.22% | -0.45% | -0.28% |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution reports the test results of EE2 IntraTMP combination: test 1.20n.

# References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, April 2023.

- F. Pu, T. Lu, P. Yin, S. McCarthy, J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala, X. Li, R.-L. Liao, J. Chen, “EE2-1.20f: Combination Test of Test 1.12 + Test 1.13b”, JVET-AD0192, April 2023.

- Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang, F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.14c combination test of EE2-1.14a and EE2-1.14b”, JVET-AD0075, Apr. 2023.

# Acknowledgement

We would like to thank Nokia for helping the crosschecks.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).