| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0251-v1 |
| --- | --- |

| Title: | AHG12 Shifting Quantizer Center |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Muhammet Balcilar Karam Naser Franck Galpin Fabrice Le Leannec | Email: | muhammet.balcilar@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a straightforward implementation of recently proposed Latent-shift algorithm on the end-to-end compression [1] to the conventional compression. This technique proposes to add some offset value which is driven by gradient of the entropy on the reconstructed transform coefficient.

The following results are obtained:

- AI: {%, %, %; %, %}.

- RA: {%, %, %; %, %}.

# Introduction

Recently, it was shown that end-to-end compression models are an unconstrained multi objective optimization problem where the solution should meet Karush–Kuhn–Tucker (KKT) conditions [1]. According to KKT conditions, the sum of gradient with respect to each objective should be zero. Since there are two objectives (rate and distortion), the gradient w.r.t distortion and the gradient w.r.t rate should cancel themselves out (they show opposite directions). Thus, one can be used instead of the other by simply changing direction. It was shown that shifting the latents (the transform coefficient) by the gradient w.r.t rate (available on decoder side) as it is negative of gradient w.r.t distortion (unavailable in decoder side) can allow 1-2% bd-rate gain on end-to-end compression models [1]. In this proposal, we shift the transform coefficient by using the gradient of very simple rate prediction in order to decrease the quantization error.

# Theoretical Justification

We assume that the RDO based quantization such as Dependent Quantization (DQ) with Trellis Coded Quantization procedure (TCQ) is an unconstrained multi objective optimization problem that can be generalized by Eq.(1).

y* = argminy[ Ry+λDx,Q-1y]                                          (1)

In this equation, x ∈ Rn is the n length real numbered coefficient to be quantized, y ∈ Zn is the quantization indices defined on discrete set of reconstruction points. Using any quantizer Q. and dequantizer function Q-1., the indices and reconstruction can be obtained by  y=Qx and  x=Q-1y respectively. Here, function R. is the rate function of the indices and function  D.,. is a distortion metric such as Mean Square Error (MSE).

Using KKT conditions on Eq.(1), it can be shown that if the solution found by RDO based quantization is optimal, the following condition should be met.

∇yRy=-λ∇yDx,Q-1y                                             (2)

Of the first order gradient-based optimization methods, if the current solution takes a step to the negative direction of its gradient w.r.t some objective function, this objective function gets smaller. For RDO based quantization case, if the quantization indices y are shifted by gradient w.r.t distortion such as  y←y-ρ∇yDx,Q-1y, where ρ∈R+ is step size, the distortion Dx,Q-1y gets smaller. However, it is not possible to know distortion without knowing original input data explicitly. But using Eq.(2), we use its negative gradient w.r.t rate as it is its gradient w.r.t distortion. Thus, our proposal is to shift the quantization indices in the loop of encoder’s dequantization and decoder’s dequantization by Eq.3 as follows.

y←y+ρ∇yRy                                                               (3)

Eq.(3) tells us to add some offset value to the quantization indices (y) (or dequantized coefficient Q-1y) which increases the rate. But since this added offset is during the dequantized stage, it has no effect on the rate but increases reconstruction quality in theory.

# Proposal

In this proposal, we assume very simple proxy of rate prediction where each quantization indices are independent, and rate increases by absolute value of the coefficient as in Eq.(4)

Ryi≅ayi+b                                                               (4)

Since gradient of rate in Eq.(4) is 0 where yi=0, a where yi>0 and -a where yi<0, we applied our offset as in Eq.(5).

yi←yi+ρ*           yi>0yi←yi-ρ*           yi<0yi←yi                    yi=0,     for i=1..n                                         (5)

Our proposal in Eq.(5) can be seen as it applies some offset to the quantization index that makes them far away from zero point. The amount of the offset ρ* ∈R+ can be finetuned over a validation set and used as a universal value for all videos.

In practice, since quantization indices should be integer, we first calculate the original reconstructed coefficient by Q-1yi) and reconstructed value when the quantization indices is shifted 1 quantization index to the opposite direction to the zero center as  Q-1y'i) where y'i=yi+(yi>0?1:-1). Then we take the weighted sum of   Q-1yi) and Q-1y'i) as the reconstructed coefficient in Eq.(6).

x=982*Q-1yi+42*Q-1y'i≫10                                    (6)

Note that this shifting on reconstruction coefficient is done only if the quantization index is not zero.

# Simulation Results

The proposed test is implemented on top of ECM-8.0. Simulations are performed following the common test conditions of EE2. The simulation results are shown in Table 1.

Table 1: Proposed method on top of ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | xxx | xxx | xxx |  |  |
| Class C | -0.07% | -0.16% | -0.29% | 99% | 100% |
| Class E | -0.09% | -0.14% | -0.08% | 99% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.17% | -0.16% | 99% | 99% |

# References

[1] Balcilar, M., Damodaran, B. B., Naser, K., Galpin, F., & Hellier, P. Vector Quantization and Shifting: Exploiting Latent Properties to Optimize Neural Codecs. https://openreview.net/forum?id=QVSoh6VM4nG

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).