| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0277-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AD0152 “EE2-2.2: AMVP with SbTMVP mode” |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy, Alexey Filippov | Tel: Email: | vrufitskiy@ofinno.com afilippov@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

This contribution is the cross-check report of contribution JVET-AD0152 “EE2-2.2: AMVP with SbTMVP mode”.

It is confirmed that the provided source code conforms to the technical description of JVET-AD0152 and the results reported in JVET-AD0152 match the results reproduced by the crosscheckers.

# Introduction

Contribution JVET-AD0152 [1] presents the simulation results of applying SbTMVP to AMVP mode, which is studied in EE2 Test 2.2. Given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from left neighboring block. The motion shift comprises a derived MVP and a signaled MVD. The number of MVD is set to 16 if a picture whose POC distance between current picture and collocated picture is larger than 2. Otherwise, if the picture is a low delay picture, the number of MVD is set to 32. Otherwise, the number of MVD is set to 64.

# Simulation results

The implementation of the method described in contribution JVET-AD0152 was tested under CTC for enhanced compression tool testing [2] for Random-Access and Low-Delay B configurations. 2 implementations were tested:

- with no fast encoding algorithm (Table 1)

- with fast encoding algorithm (Table 2).

Below, the partial simulation results obtained by the crosscheckers for both implementations method are shown in Table 1 and Table 2.

Table 1. Simulation results of Test EE2-2.2 (with no fast encoding algorithm)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.02% | -0.03% | 101% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.03% | 0.07% | -0.12% | 102% | 99% |
| Class C | -0.14% | -0.10% | -0.06% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.10% | 0.25% | 0.02% | 101% | 98% |
| Class F | -0.08% | 0.02% | 0.03% | 101% | 100% |
| Class TGM | 0.05% | 0.05% | 0.02% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.21% | 0.38% | -0.04% | 101% | 99% |
| Class E | -0.47% | 0.45% | -0.20% | 102% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.24% | -0.86% | 0.26% | 102% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Simulation results of Test EE2-2.2 (with fast encoding algorithm)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.01% | -0.11% | 101% | 99% |
| Class C | -0.13% | -0.09% | -0.04% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | 0.21% | 0.04% | 99% | 98% |
| Class F | -0.07% | -0.06% | 0.02% | 100% | 100% |
| Class TGM | 0.02% | 0.03% | 0.06% | 100% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.22% | -0.02% | 0.13% | 100% | 100% |
| Class E | -0.46% | 0.39% | -0.28% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.30% | -0.90% | -0.55% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This crosscheck report presents the simulation results obtained for both implementations of the method described in contribution JVET-AD0152 and confirms the match of the partial results obtained with the ones reported in proposal JVET-AD0152.

# References

- R.-L. Liao, J. Chen, Y. Ye, and X. Li, “EE2-2.2: AMVP with SbTMVP mode,” JVET-AD0152, April 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

# Patent rights declaration(s)

Ofinno LLC does not have any current or pending patent rights relating to the technology described in this contribution.