| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0281-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AD0182 ("EE2-2.1: On Affine DMVR", Test EE2-2.1c and Test EE2-2.1g) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei Chen | Email: | chenwei06@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution is the cross-check of contribution JVET-AD0182 [1].

Partial results are obtained and match the coding results reported in JVET-AD0182.

Introduction

EE2-Test2.1 [1] is proposed to evaluate the performance of the affine DMVR extension, CPMV search, CPMV refinement and , affine parameter refinement,  several simplification aspects are also included: distortion-based early termination for affine DMVR and Affine DMVR usage adaptation according to CU size and shape.

In test 2.1a, CPMV search with simplification is tested.

In test 2.1b, CPMV refinement with simplification is tested.

In test 2.1c, affine DMVR extension is tested. Affine DMVR extension includes applying affine DMVR and affine regression to affine MMVD mode and adaptive BM merge mode.

In test 2.1d, affine parameter refinement is tested.

In test 2.1e, affine parameter refinement with simplification is tested.

In test 2.1g, a combination test of 2.1a and 2.1c with simplification is performed, wherein the affine DMVR adaptation according to CU sizes and shapes is applied to the extensions as well.

The source code was downloaded from the EE2 repository and was verified and confirmed to be consistent with the description in JVET-AD0182. The crosscheck results are perfectly matched with the simulation results from the proponent.

Simulation results

Test2.1c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.01% | -0.12% | 102% | 102% |
| Class C | -0.05% | -0.03% | -0.01% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.20% | 0.02% | 102% | 102% |
| Class F |  |  |  |  |  |

Test2.1g

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.03% | -0.14% | 103% | 103% |
| Class C | -0.10% | -0.12% | -0.05% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | -0.04% | -0.12% | 103% | 103% |
| Class F |  |  |  |  |  |

Reference

- M. Blestel, M. Radosavljević (Xiaomi), Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm), J. Chen, R. -L. Liao, X. Li, Y. Ye (Alibaba), “EE2-2.1: On Affine DMVR”, JVET-AD0182, Apr. 2023.

Patent rights declaration(s)

Kwai Inc. does not have any current or pending patent rights relating to the technology described in this contribution.