| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0282-v1 |
| --- | --- |

| Title: | Crosscheck of EE1-1.6 (In-Loop Filter with Wide Activation and Large Receptive Field) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | Yun Li | Email: | yli30@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-1.6 (JVET-AD0106 [1]), in which two tests have been conducted for the evaluation of the multi-scale feature extraction backbone network proposed in JVET-AC0178 [2]. This cross-check was carried out with both training and inference test.

The training procedure has been investigated, and stage one of the data extraction has been partially completed.

The results of the inference cross-check match those reported in JVET-AD0106 for the AI and RA configuration (only 0.01 percent difference is observed for one test with AI configuration).

# Introduction

In EE1-1.6 two tests have been conducted for the evaluation of the multi-scale feature extraction backbone network proposed in JVET-AC0178. Test 1 is performed with an input set of reconstruction, prediction, QP, and slice type, and Test 2 is conducted with an extra input of BS. For both tests, the NN intra prediction is enable. The experiment shows the performance of adding extra BS data for the proposed network.

The training of EE-1.6 consists of three stages, and iterative training for improving the filtering performance for inter slices has been utilized. In the first stage, the training data is extracted from the VTM, and the intra model is trained. In the second stage, the intra model is used for the filtering to extract the training data, and the inter model is then trained. In the final stage, both the intra model and inter model obtained from the previous stages are utilized to extract the training data, with which the intra&inter model is trained.

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology [3]. In the test, the NnlfOption is set to 2, the NNIntraPred is on, and the inference is conducted with the trained models provided by the proponent.

The BD-rate savings for the Y, Cb and Cr components, respectively, in the AI and RA configurations compared to NNVC 4.0 anchor are shown in Tables 1 and 2 for Test 1 and Test 2, respectively.

Table 1 Test 1 results vs NNVC-4.0

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -10.91% | -21.01% | -20.88% | -11.99% | -20.79% | -23.27% |
| Class A2 | -9.89% | -21.31% | -17.36% | -12.71% | -22.44% | -19.01% |
| Class B | -10.13% | -20.39% | -21.03% | -11.93% | -25.63% | -24.12% |
| Class C | -11.23% | -21.36% | -23.47% | -12.98% | -24.72% | -25.29% |
| Class E | -14.99% | -24.67% | -25.35% |  |  |  |
| Overall | -11.27% | -21.58% | -21.66% | -12.38% | -23.78% | -23.24% |
| Class D | -10.81% | -21.29% | -23.29% | -14.35% | -25.70% | -26.28% |
| Class F | -6.79% | -15.24% | -15.25% | -6.71% | -15.84% | -14.91% |

Table 2 Test 2 results vs NNVC-4.0

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -10.96% | -21.58% | -21.76% | -12.11% | -21.23% | -23.02% |
| Class A2 | -9.89% | -21.51% | -17.72% | -12.83% | -22.76% | -19.59% |
| Class B | -10.15% | -21.72% | -22.56% | -12.04% | -26.45% | -24.28% |
| Class C | -11.30% | -21.71% | -23.44% | -13.11% | -25.21% | -24.66% |
| Class E | -15.03% | -24.61% | -26.05% |  |  |  |
| Overall | -11.31% | -22.14% | -22.40% | -12.50% | -24.34% | -23.19% |
| Class D | -10.87% | -21.92% | -23.47% | -14.23% | -25.99% | -25.65% |
| Class F | -6.47% | -15.17% | -15.35% | -6.81% | -16.90% | -15.63% |

# Conclusion

The inference results with the provided models for both tests match the ones reported by the proponent for the AI and RA configuration (only 0.01 percent difference is observed for Test 1 with AI configuration).

# References

- EE1-1.6: In-Loop filter with wide activation and large receptive field, JVET-AD0106, Y. Li, K. Zhang, L. Zhang.

- EE1-related: In-Loop filter with wide activation and large receptive field, JVET-AC0178, Y. Li, K. Zhang, L. Zhang.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.