| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0287-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0085 (EE2-1.24a Combinations of MPM related tests) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of EE2-1.24a of JVET-AD0085. The simulation results reportedly match those provided by the proponents.

# Verification

EE2-1.24a of JVET-AD0085 [1] presents a combination test of EE2 Test 1.21c and 1.22 which are MPM related modifications.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The coding results of EE2-1.24a under the ECM CTC are summarized in Table 1.

Table 1: Performance of EE2-1.24a over ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | -0.10% | 101% | 102% |
| Class A2 | -0.17% | -0.18% | -0.11% | 102% | 100% |
| Class B | -0.22% | -0.23% | -0.16% | 101% | 103% |
| Class C | -0.27% | -0.25% | -0.27% | 101% | 102% |
| Class E | -0.16% | -0.30% | -0.28% | 101% | 102% |
| Overall | -0.19% | -0.20% | -0.19% | 101% | 102% |
| Class D | -0.20% | -0.21% | -0.11% | 100% | 105% |
| Class F | -0.17% | -0.26% | -0.20% | 100% | 102% |
| Class TGM | -0.09% | -0.10% | -0.03% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.02% | 100% | 101% |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.13% | -0.10% | 100% | 100% |
| Class C | -0.17% | -0.20% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | 0.07% | -0.14% | 100% | 99% |
| Class F | -0.10% | -0.50% | -0.35% | 100% | 100% |
| Class TGM | -0.03% | 0.02% | -0.04% | 101% | 100% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

L. Xu, Y. Yu, H. Yu, D. Wang, C. Zhou, Z. Lv, J. Zhang, H. Wang, Y. Chang, V. Seregin, M. Karczewicz, “EE2-1.24: Combinations of MPM related tests”, JVET-AD0085, April 2023.