| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0288-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0108 (AHG12: Cross-component residual model (CCRM) for inter prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AD0108. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-AD0108 [1] presents a cross-component residual model (CCRM) based chroma prediction to improve compression efficiency of inter slices. The proposed method uses 8-tap convolutional filter to map reconstructed luma into an improved chroma prediction when the CCRM prediction mode is activated by a TU level flag. The filter input consists of 6 spatial luma samples, a nonlinear term, and a bias term. Filter coefficients are derived for each block separately using the prediction signals and the filters are applied to the reconstructed luma signal.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The coding results of JVET-AD0108 under the ECM CTC are summarized in Table 1.

Table 1: Performance over ECM-8.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.89% | -0.93% | 102% | 99% |
| Class C | 0.04% | -0.74% | -0.69% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.09% | -0.49% | -0.45% | 102% | 101% |
| Class F | -0.03% | -0.72% | -0.68% | 104% | 102% |
| Class TGM | -0.31% | -0.79% | -0.86% | 103% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.04% | -2.20% | -2.98% | 105% | 99% |
| Class E | 0.05% | -0.16% | -0.75% | 102% | 101% |
| Overall |  |  |  |  |  |
| Class D | 0.09% | -3.32% | -3.47% | 105% | 98% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E | 0.01% | -1.36% | -1.01% | 103% | 101% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -3.14% | -2.99% | 105% | 106% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

P. Astola, J. Lainema, “AHG12: Cross-component residual model (CCRM) for inter prediction”, JVET-AD0108, April 2023.