| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0302-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0133 (AHG10: Lambda-QP Relationship Fix for Slice-level Multi-QP Optimization) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | N. Yan | Tel: Email: | yanning03@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This document reports the crosschecking results of JVET-AD0133 on multi-QP optimization. The source code provided from the proponent has been verified and confirmed to be consistent with the description in JVET-AD0133. The BR-rate results in this document are matched with the simulation results of JVET-AD0133.

Introduction

In JVET-AD0133, it is proposed to fix the slice-level multi-QP optimization in the VTM software. The main modifications this contribution proposes to the slice-level multi-QP optimization of the original version in VTM are as follows:

- This contribution proposes to modify the QP-lambda relationship in slice-level multi-QP RDO.

- This contribution proposes to fix the slice-level lambda while traversing the QP within a predefined range.

- This contribution proposes to disable CU-level mode reuse when multi-QP optimization is enabled.

Cross-check results

Table 1: cross-check results of JVET-AD0133

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM19.2-MultiQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.18% | 2.85% | -1.59% | 96% | 93% |
| Class A2 | -9.38% | -4.13% | -4.38% | 96% | 89% |
| Class B | -8.31% | -7.76% | -6.98% | 96% | 92% |
| Class C | -7.20% | -9.09% | -8.54% | 98% | 93% |
| Class E | -7.51% | -7.77% | -7.03% | 96% | 91% |
| Overall | -8.25% | -5.68% | -6.00% | 97% | 92% |
| Class D | -7.61% | -8.35% | -8.60% | 98% | 90% |
| Class F | -7.20% | -4.09% | -3.99% | 99% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM19.2-MultiQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -12.46% | -6.66% | -8.11% | 109% | 72% |
| Class A2 | -9.29% | -7.87% | -8.17% | 108% | 73% |
| Class B | -11.15% | -6.58% | -6.58% | 109% | 97% |
| Class C | -10.49% | -7.26% | -9.33% | 110% | 99% |
| Class E |  |  |  |  |  |
| Overall | -10.86% | -7.03% | -7.94% | 109% | 87% |
| Class D | -10.20% | -7.54% | -7.61% | 108% | 94% |
| Class F | -6.59% | -8.04% | -7.43% | 106% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM19.2-MultiQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.22% | -6.97% | -6.04% | 117% | 101% |
| Class C | -3.48% | -5.86% | -7.32% | 117% | 97% |
| Class E | -2.78% | 3.61% | 6.49% | 115% | 101% |
| Overall | -5.28% | -3.96% | -3.34% | 116% | 100% |
| Class D | -4.81% | -3.40% | -4.08% | 114% | 100% |
| Class F | -3.05% | -5.62% | -5.81% | 113% | 97% |

Conclusion

It is reported that the BD-rate results matched what provided by the proponent of JVET-AD0133.

Patent rights declaration(s)

Kwai Inc. does not have any current or pending patent rights relating to the technology described in this contribution.