| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0307 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0122 ([AHG8] Configuration changes for random access and low delay) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zizheng Liu | Tel: Email: | zizhengliu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the results of crosschecking JVET-AD0122 proposed by Ericsson.

The update of the configuration files had been reviewed and confirmed consistent with the proposal description. The simulation results match the reported results in JVET-AD0122.

Verification

JVET-AD0122 presents the QP offset change for LD and RA configurations. The experiments are conducted following the AhG8 CTC [2].

The bitrate matches exactly with the results reported in JVET-AD0122. The mAP and MOTA differ slightly due to different platforms. The simulation results are as follows:

Table 1 Simulation results (Random Access, JVET-AD0122 vs anchor)

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | EncT | DecT |
| SFU-HW | Class A | -4.25% | 2.77% | -4.25% | 79.66% | 99.57% |
|  | Class B | #NUM! | 0.65% | -0.02% | 72.58% | 98.35% |
|  | Class C | -7.83% | -5.48% | -7.83% | 67.24% | 90.85% |
|  | Class D | -15.14% | -15.86% | -15.14% | 67.83% | 85.88% |
|  | Average | #NUM! | -6.15% | -7.40% | 69.93% | 92.14% |
|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | EncT | DecT |
| TVD | TVD-01_1 | -2.25% | -1.72% | -2.25% | 71.90% | 96.66% |
|  | TVD-01_2 | -1.09% | -1.42% | -1.09% | 71.24% | 96.18% |
|  | TVD-01_3 | -8.35% | -6.25% | -8.35% | 67.58% | 94.26% |
|  | TVD-02_1 | -9.49% | -10.28% | -9.49% | 76.23% | 92.54% |
|  | TVD-03_1 | -0.72% | 0.77% | -0.72% | 69.04% | 98.29% |
|  | TVD-03_2 | -4.48% | 0.40% | -4.48% | 69.16% | 96.62% |
|  | TVD-03_3 | -22.41% | -10.28% | -9.49% | 67.85% | 86.37% |
|  | Average | -6.97% | -5.97% | -7.21% | 70.37% | 94.34% |

Table 2 Simulation results (Low-delay, JVET-AD0122 vs anchor)

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | EncT | DecT |
| SFU-HW | Class A | -1.90% | -3.40% | -1.90% | 83.36% | 96.22% |
|  | Class B | -7.42% | -7.74% | -7.42% | 77.83% | 95.64% |
|  | Class C | -8.57% | -8.81% | -8.57% | 74.22% | 88.39% |
|  | Class D | -14.53% | -15.11% | -14.53% | 72.79% | 83.37% |
|  | Average | -9.53% | -10.00% | -9.53% | 75.54% | 89.53% |
|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | EncT | DecT |
| TVD | TVD-01_1 | ############ | ############ | -8.96% | 76.31% | 88.93% |
|  | TVD-01_2 | -14.45% | -22.07% | -14.45% | 77.22% | 89.01% |
|  | TVD-01_3 | -16.13% | -22.41% | -16.13% | 73.38% | 93.94% |
|  | TVD-02_1 | -32.23% | -22.57% | -32.23% | 79.88% | 88.28% |
|  | TVD-03_1 | -8.40% | -8.17% | -8.40% | 75.50% | 93.16% |
|  | TVD-03_2 | ############ | ############ | 6.75% | 75.09% | 87.19% |
|  | TVD-03_3 | -21.90% | -23.43% | -22.35% | 73.83% | 80.36% |
|  | Average | #NUM! | #NUM! | -13.68% | 75.86% | 88.60% |

References

[1] C. Hollmann, R. Sjöberg, J. Ström (Ericsson), “[AHG8] Configuration changes for random access and low delay”, JVET-AD0122, Apr. 2023.

[2] S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content” JVET-AC2031, Jan. 2023.

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.