| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0308 |
| --- | --- |

| Title: | Non-EE2: Extension of unified PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath Thierry Dumas Fabrice Le Leannec Kevin Reuzé | Tel: Email: | gagan.rath@interdigital.com thierry.dumas@interdigital.com fabrice.leleannec@interdigital.com kevin.reuze@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution refines the unified  PDPC in intra prediction by first introducing a combination between PDPC and gradient PDPC. Specifically, for a positive directional intra prediction mode with angular scale value greater than -2 and less than 2, a combination of PDPC and gradient PDPC is applied. In the second refinement, the secondary reference samples are computed with a 4-tap filter instead of the nearest neighbor (in PDPC) or  linear interpolation (gradient PDPC). In the third refinement, for the current block, the unified PDPC is disabled if the neighbor block on the side of the secondary reference sample is not available.

It is reported that, on top of ECM-8.0, the proposed unified PDPC yields -0.05%, -0.10%, -0.10% and -…%, -…%, -…% in AI and RA configurations respectively.

# Introduction

In ECM-8.0 [1], for intra prediction with a positive directional mode, either PDPC or gradient PDPC [2],  is applied to the first prediction values. For any given target block, an angular scale value is computed using the inverse angle parameter corresponding to the prediction direction, and the height or width of the block depending on if the prediction direction is vertical or horizontal. If the scale value is greater than or equal to 0, then the normal PDPC is applied to the first prediction samples. On the contrary, if the scale value is negative, the gradient PDPC is enabled, the scale value is recomputed using only the height and width of the block, and the gradient PDPC is applied to the first prediction values. Instead of  using a hard boundary of scale value to make an either-or decision, this contribution proposes to  combine PDPC and gradient PDPC in a more graceful manner by allowing an overlap where for scale value in the range from -1 to 1 both gradient PDPC and PDPC are combined with suitable relative weights.

In PDPC, the secondary reference sample is localized by extending the prediction direction till the secondary reference array and is computed as the nearest reference sample. In this contribution we propose to use a 4-tap filter to increase the accuracy of the secondary reference sample. Similarly, in gradient PDPC, to compute the gradient at the secondary reference sample on a row (for vertical modes) or column (for horizontal modes), we propose to use a 4-tap filter to compute the primary reference sample corresponding to the secondary reference sample.

In either PDPC or in gradient PDPC, the aim of modifying the first prediction values is to avoid discontinuities on the side of the secondary reference array. When the neighbor block on that side is not available, the reference samples are usually filled up using padding. Since the reference samples may not be correlated to the initial prediction values, we propose to disable the PDPC or gradient PDPC in cases when the neighbor block on the side of secondary reference is not available.

# Combination of PDPC and gradient PDPC

Referring to Fig. 1, where the reference samples at (x,y) is denoted as R(x,y), the first prediction value Px,y is modified in PDPC as

Px,y=clipPx,y+wL×G-1,y'+wLG-1,y-G-1,y'+32≫6 where, the gradients G-1,y' and G(-1,y) are computed as

G-1,y'=R-1,y'-Px,y

G-1,y=R-1,y-Rx'',-1,

and the weight wL is computed as in ECM 8.0 and the weight wL is computed as

wL=wL≫(2*scale+2).

Figure 1: combination of PDPC and gradient PDPC for a positive directional mode with scale >= 0.

Similarly, referring to Fig. 2, the first prediction value Px,y is modified in gradient PDPC as

Px,y=clipPx,y+wL×G-1,y+wLG-1,y'-G-1,y+32≫6, where the gradients are computed as:

G-1,y'=R-1,y'-P0,y

G-1,y=R-1,y-Rx'',-1,

and the weight wL is computed as in ECM 8.0 and the weight wL is computed as

wL=wL≫(2-2*pdpcScale),

where pdpcScale is the original angular scale value having a negative value.

Figure 2: combination of PDPC and gradient PDPC for a positive directional mode with scale < 0.

Note that, when pdpcScale= -1, the secondary reference sample R-1,y'  is available for targets pixels on the first column for all block sizes.

## Interpolation accuracy and dependency on the availability of secondary reference samples.

For the current coding unit predicted with a positive directional intra prediction mode, the proposed unified PDPC is also improved in two manners.

- For a predicted sample post-processed via the unified PDPC, a weak 4-tap interpolation filter is applied to compute the secondary reference sample R-1,y' and the reference sample Rx'',-1.

- If the positive directional mode is vertical and none of the neighbor blocks on the left side of the current CB is available, or, if the positive directional mode is horizontal and none of the neighbor blocks above the current CB is available, the unified PDPC is disabled.

# Experimental results

The AI test results  with CTC are presented below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.09% | 100% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.05% | -0.05% | -0.09% | 100% | 98% |
| Class C | -0.04% | -0.09% | -0.17% | 103% | 103% |
| Class E | -0.05% | -0.06% | 0.00% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | -0.12% | -0.07% | 101% | 104% |
| Class F | -0.04% | -0.11% | -0.05% | 99% | 97% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

On top of ECM-8.0, it is reported that the proposed unified PDPC provides -…%, -…%, -…% and -…%, -…%, -…% in AI and RA configurations respectively for encoding and decoding runtimes of …% and …% respectively.

It is suggested to study the unified PDPC during the next round of EE2.

# References

[1] Algorithm description of Enhanced Compression Model 8 (ECM 8). M. Coban, F. L. Leléannec, R. Liao, K. Naser, J. Ström, L. Zhang. 29th JVET meeting, 11 to 20 January 2023, Teleconference.

[2] Unified PDPC for Angular Intra Modes. B. Ray, G. V. der Auwera, M. Karczewicz. JVET-Q391, 17th JVET Meeting, 7 to 17 January 2020, Brussels, Belgium.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).