| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0310-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0125 (EE2-1.12: Intra TMP with sub-pel precision) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fangjun Pu | Tel: Email: | fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of EE2 Test 1.12 for intra TMP with sub-pel precision. In Test 1.12, sub-pel precision is supported in Intra TMP. Specifically, three precisions (1/2-pel, 1/4-pel and 3/4-pel) in eight directions around the integer-pel position obtained by template matching are further enabled to Intra TMP. The eight directions are further sorted by template cost for each CU and only the first four direction can be used. The selected precision and direction are signaled in CU level. Four-tap DCT-IF interpolation filters are used for sub-pel interpolation. It is confirmed that the implementation matches the proposal and crosschecking results for all intra configuration match with the proponent’s results.

# Crosscheck results

The methods are implemented on top of ECM-8.0 and are tested under CTC. This crosscheck report tests with all intra configuration for EE2-1.12. Results are show in Table 1.

Table 1. Test 1.12

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.06% | 101% | 100% |
| Class A2 | -0.26% | -0.27% | -0.28% | 101% | 101% |
| Class B | -0.20% | -0.18% | -0.22% | 100% | 101% |
| Class C | -0.06% | -0.11% | -0.13% | 100% | 101% |
| Class E | -0.48% | -0.54% | -0.46% | 100% | 99% |
| Overall | -0.19% | -0.23% | -0.22% | 100% | 100% |
| Class D | -0.04% | -0.24% | -0.04% | 100% | 101% |
| Class F | 0.02% | -0.14% | 0.02% | 101% | 101% |
| Class TGM |  |  |  |  |  |

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.