| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0314-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0163 (AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Yang Liu Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution reports on the inference crosscheck of JVET-AD0163. JVET-AD0163 describes the usage of the most recent NNPFC and NNPFA SEI messages for NNPF in NNVC 4.0. Due to the time limit, we have only provided test results for Classes B, C, D, and F. The cross-check is reported as a success given that there is well match between the proponent’s results and the cross-checker’s. Overall, the deviations are 0.00%, and we will provide complete test results in version 2.

Introduction

The content-adaptive neural network post filter (NNPF) adopted to NNVC 4.0, JVET-AC0055 [1], makes use of outdated NNPFC and NNPFA SEI messages, JVET-Z2044 [2]. In JVET-AD0163, the latest NNPFC and NNPFA SEI messages have been implemented on top of NNVC 4.0, and the characteristics signalled within the NNPFC SEI message are used to correctly apply the post-filter at inference stage. This proposal crosschecks the performance of JVET-AD0163 in RA configuration.

Simulation results

The baseline code is the NNVC-4.0 and the evaluation is done in the JVET CTC RA configuration. The following tables present the cross-checker’s results when the anchor is NNVC-4.0 and the NNPF is OFF.

Table I. Cross-checker’s results of new NNPF inference on top of NNVC 4.0 and NNPF OFF

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 NNPF OFF |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
| Class B | -5.11% | -20.60% | -16.94% | -3.55% | -21.00% | -16.77% |
| Class C | -4.09% | -19.40% | -16.53% | -3.20% | -18.13% | -16.40% |
| Class E |  |  |  |  |  |  |
| Overall | 37.21% | 27.96% | 29.95% | 37.96% | 28.17% | 30.03% |
| Class D | -5.17% | -19.71% | -18.84% | -2.25% | -18.30% | -17.44% |
| Class F | -1.43% | -6.51% | 2.11% | -1.87% | -12.99% | -11.14% |

The following tables present the cross-checker’s results over the proponent’s:

Table II. Coding performance of our Simulation results on top of JVET-AD0163

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 NNPF OFF |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 40.00% | 40.00% | 40.00% | 40.00% | 40.00% | 40.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

Conclusion

The inference cross-check was successful. From the current results, a perfect match has been achieved between the proponent’s and cross-checker’s results.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).