| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0315-v2 |
| --- | --- |

| Title: | Non-EE2: Directional DC modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyungyong (Kyle) Kim Dongcheol Kim Ju-Hyung (John) Son Jin Sam Kwak | Email: | kyungyong.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two additional directional DC modes (Above DC, Left DC) which generate predicted samples with a constant value based on the above and left samples, respectively. They can be used regardless of whether the current block is a square or non-square block, like the directional planar mode. Compared to ECM-8.0, the proposed methods achieve -0.03% gain in AI configurations with 105% encoder and 101% decoder run time in AI configurations of common test conditions.

AI: {-0.03%, -0.02%, 0.03%, 105%, 101%}

Introduction

As in VVC, the DC intra prediction mode in ECM-8.0 [1] uses different average calculations depending on whether the current block is a square or rectangular block, as shown in Fig. 1. For square blocks, the DC intra prediction mode generates predictions using the mean sample value of the above and left reference samples. For non-square blocks, the DC intra prediction mode uses only the reference samples along the longer side of the block to calculate the mean value.

Figure 1. Example of average calculation in DC mode

Proposal

This contribution proposes two additional DC modes, similar to the directional planar mode in ECM-8.0. As shown in Fig 2, if the directional mode of the current block is Above DC mode, the mean sample value of the above neighboring samples of the current block is used for prediction generation. Otherwise, if the directional mode of the current block is Left DC mode, the mean sample value of the left neighboring samples of the current block is used for prediction generation. Otherwise, the existing DC mode, which depends on whether the current block is a square block or not, is used for prediction generation. By using two additional DC modes, it becomes possible to enable the cases shown in Fig. 3.

Figure 2. flowchart for proposed method

Figure 3. Case enabled by two additional DC modes

Encoder tests two additional DC modes (Above DC, Left DC) regardless of whether the current block is a square or non-square block. In the encoder side, the best DC mode is selected in terms of RDO. The existing DC mode has not been changed. To avoid confusion with existing DC mode and MPM list construction, the two additional DC modes are signalled with a sub-mode of the directional planar mode, as shown in Table. 1.

Table 1. Binarization of additional DC mode

| Directional mode | Binarization |
| --- | --- |
| Planar | 0 |
| Horizontal Planar | 10 |
| Vertical Planar | 110 |
| Above DC | 1110 |
| Left DC | 1111 |

Test Result

The proposed methods were implemented on top of the ECM-8.0 [1]. Simulations were performed by following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.05% | -0.02% | 103% | 100% |
| Class A2 | -0.02% | -0.03% | 0.02% | 103% | 100% |
| Class B | -0.03% | -0.01% | 0.00% | 105% | 101% |
| Class C | -0.03% | -0.06% | 0.03% | 106% | 101% |
| Class E | -0.02% | -0.03% | 0.13% | 106% | 101% |
| Overall | -0.03% | -0.02% | 0.03% | 105% | 101% |
| Class D | -0.06% | -0.22% | 0.06% | 107% | 101% |
| Class F | 0.05% | -0.13% | -0.08% | 102% | 101% |
| Class TGM | 0.01% | 0.02% | 0.08% | 102% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | -0.12% | 0.00% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.14% | 0.05% | 103% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

This contribution proposes two additional DC modes. Compared to ECM-8.0, the proposed methods achieve -0.03% gain in AI configurations with 105% encoder and 101% decoder run time in AI configurations of common test condition.

Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, January 2023.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).