| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April. 2023 | Document: JVET-AD0325-v2 |
| --- | --- |

| Title: | Cross check of JVET-AD0172 (AHG12: Adaptation of LFNST/NSPT for low-delay configuration) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Radosavljević | Tel: Email: | milosr@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This is cross-check of the contribution JVET-AD0172. In the contribution, it is proposed to activate LFNST/NSPT for the lower temporal Id (TiD) slices in LDB configuration. The proposed adaptation is introduced to balance the coding gain and run time.

The following results are obtained compared to ECM-8.0 in LDB configuration:

Overall:	-0.59%	-1.03%	-1.16%	(Y Cb Cr)	102% (Enc)	98% (Dec)

The results reported by the proponents are confirmed during the cross-check (in the v1 of the cross-check only partial results are provided).

Version 2 of this report updates results.

# Introduction

In ECM CTC, LFNST/NSPT is enabled only for All Intra and Random Access configurations. Proponents tested configuration when LFNST is enabled for LDB configuration with some modifications. LFNST is enabled for the lower TiD slices only. It is proposed to limit LFNST/NSPT usage for slices with TiD lower than 3. A picture header flag is signaled to indicate LFNST/NSPT usage for certain pictures. If disabled, lfnst_idx is not signalled in the bitstream.

The proposal introduces normative changes to the ECM. However, those normative changes provide solid gain with reasonable complexity for LDB configuration. On top of that, LFNST is known and well studied.

Proposed changes reduce run-time while maintaining most of the gain compared to straightforward LFNST/NSPT ON/OFF scenario.

# Experimental Results

Partial experimental results match those provided by the proponents. Although, our encoder/decoder run-time is not reliable due to heterogeneous cluster.

# Conclusion

The proposed adaptation offers good average BD-rate gain with reasonable encoder/decoder runtime compared to ECM-8.0 in LDB. On top of that, compared with straightforward ON/OFF configuration, most of the gain is preserved and runtime is reduced by using proposed modifications.

Cross-checkers propose to consider to directly adopt this proposal.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.