| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0335-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0236 (Non-EE2: On Classification of In-Loop Filters) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Vladislav Shchukin, Jacob Ström | Email: | vladislav.shchukin@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The contribution reports the results of crosscheck of JVET-AD0236 proposed by Bytedance Inc. In JVET-AD0236 authors proposed additional rules for 3 in-loop filters: adaptive loop filter (ALF), sample-adaptive offset (SAO) and bilateral filter (BIF).

For ALF and SAO, the deblocking filter-boundary strength (DBF-BS) is introduced. 2 levels of DBF-BS are considered that yields doubling of the number of classes used in ALF and SAO.

For BIF, the variance of samples in transform unit (TU) is the base of classification into 5 classes.

# Verification

Source code with changes over ECM-8.0 was provided by Bytedance.

The BD-rate results for finished classes match the results reported in JVET-AD0236. The simulation results are presented below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.20% |  |  |
| Class A2 | -0.03% | 0.17% | 0.17% |  |  |
| Class B | 0.00% | -0.08% | -0.04% |  |  |
| Class C | -0.03% | -0.11% | -0.01% |  |  |
| Class E | -0.04% | -0.03% | -0.04% |  |  |
| Overall | -0.01% | -0.03% | 0.04% |  |  |
| Class D | -0.03% | -0.11% | -0.03% |  |  |
| Class F | 0.04% | -0.20% | -0.21% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.24% | 0.01% | 0.14% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | 0.11% | -0.19% |  |  |
| Class F | -0.16% | -0.44% | -0.17% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E | -0.27% | 1.97% | -0.57% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.07% | -0.52% | 0.74% |  |  |
| Class F |  |  |  |  |  |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.