| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0346-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0107 (EE1-related: Simplified parameter selection for filter set #1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Tel: Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the inference crosscheck results for JVET-AD0107, which reduces the number of parameter selection from 3 to 2 for NNVC filter set 1. It is reported that the crosscheck results can match exactly the results in JVET-AD0107.

# Introduction

In the current NNVC-4.0 filter set 1, the parameter selection at the encoder side allows three conditional parameters {Param_1, Param_2, Param_3}[1]. Denoting the sequence level QP as q, for low temporal layers, Param_1 = q, Param_2 = q-5, Param_3 = q-10. For high temporal layers, Param_1 = q, Param_2 = q-5, Param_3 = q+5.

JVET-AD0107[2] proposes to reduce the number of parameter selection from 3 to 2, i.e., {Param_1, Param_2}. In addition, for low temporal layers, use Param_1 = q, Param_2 = q-5. For high temporal layers, use Param_1 = q, Param_2 = q+5.

The modification changes only the inference code. No retraining is needed.

# Results

Two simulations are crosschecked, the proposed method with NnIntra and NnFilterSet1 enabled, and the reference original parameter selection with NnIntra and NnFilterSet1 enabled.

The compiler is GCC 8.2.0. Results are on top of NNVC-4.0. Simulations were run with integer models.

The proposed method + NnIntra + NnFilterSet1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.52% | -17.54% | -21.94% | 175% | 30706% |
| Class A2 | -11.00% | -20.62% | -16.65% | 171% | 29043% |
| Class B | -10.54% | -22.75% | -23.55% | 181% | 30123% |
| Class C | -11.98% | -24.03% | -25.19% | 164% | 25706% |
| Class E |  |  |  |  |  |
| Overall | -11.01% | -21.62% | -22.28% | 173% | 28776% |
| Class D | -13.89% | -24.65% | -27.91% | 164% | 27303% |
| Class F | -6.45% | -14.96% | -13.13% | 222% | 13033% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.83% | -20.08% | -20.96% | 163% | 27606% |
| Class C | -9.47% | -21.78% | -21.86% | 156% | 27740% |
| Class E | -8.66% | -17.72% | -20.33% | 247% | 19900% |
| Overall | -8.59% | -20.06% | -21.10% | 178% | 25478% |
| Class D | -11.21% | -24.97% | -27.35% | 144% | 24313% |
| Class F | -5.57% | -13.80% | -10.82% | 207% | 13255% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.19% | -19.78% | -21.27% | 269% | 23346% |
| Class A2 | -9.17% | -21.15% | -17.07% | 242% | 19374% |
| Class B | -9.40% | -21.05% | -23.91% | 249% | 19460% |
| Class C | -10.10% | -21.71% | -24.47% | 217% | 12942% |
| Class E | -13.88% | -23.81% | -25.97% | 242% | 20501% |
| Overall | -10.40% | -21.46% | -22.80% | 243% | 18468% |
| Class D | -9.81% | -21.40% | -25.43% | 206% | 12946% |
| Class F | -7.14% | -16.85% | -15.75% | 181% | 15414% |

The reference original parameter selection + NnIntra + NnFilterSet1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.53% | -17.71% | -22.20% | 200% | 30754% |
| Class A2 | -11.01% | -20.76% | -16.93% | 195% | 29642% |
| Class B | -10.54% | -23.00% | -23.69% | 198% | 31967% |
| Class C | -11.98% | -24.24% | -25.39% | 178% | 25944% |
| Class E |  |  |  |  |  |
| Overall | -11.02% | -21.82% | -22.49% | 192% | 29553% |
| Class D | -13.89% | -25.07% | -28.38% | 179% | 27335% |
| Class F | -6.41% | -15.05% | -13.28% | 255% | 13488% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.81% | -20.15% | -21.10% | 187% | 28685% |
| Class C | -9.52% | -22.02% | -22.29% | 174% | 28414% |
| Class E | -8.68% | -17.61% | -19.52% | 307% | 21220% |
| Overall | -8.60% | -20.14% | -21.10% | 207% | 26519% |
| Class D | -11.15% | -24.97% | -27.84% | 165% | 24703% |
| Class F | -5.58% | -14.05% | -11.31% | 255% | 13996% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.19% | -19.78% | -21.27% | 273% | 23849% |
| Class A2 | -9.17% | -21.15% | -17.07% | 247% | 19158% |
| Class B | -9.40% | -21.05% | -23.91% | 246% | 19573% |
| Class C | -10.10% | -21.71% | -24.47% | 219% | 12902% |
| Class E | -13.88% | -23.81% | -25.97% | 227% | 20291% |
| Overall | -10.40% | -21.46% | -22.80% | 241% | 18484% |
| Class D | -9.81% | -21.40% | -25.43% | 196% | 13054% |
| Class F | -7.14% | -16.85% | -15.75% | 179% | 15345% |

Encoding time and decoding time are not reliable since these were not run on the same machine.

# References

[1] Description of algorithms and software in neural network-based video coding (NNVC) version 2, JVET-AC2019, Y. Li, H. Wang, L. Wang, F. Galpin, J. Ström, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023

[2] EE1-related: Simplified parameter selection for filter set #1, JVET-AD0107, Y. Li, K. Zhang, L. Zhang, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 30th Meeting, Antalya, TR, 21–28 April 2023.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.