| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0347-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0211 (EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6.) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Tel: Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the inference crosscheck results for JVET-AD0211, which proposes a combined network structure of EE1-1.3.5 and EE1-1.6 for the NN inloop filter. It is reported that the crosscheck results can match the results in JVET-AD0211.

# Introduction

In EE1-1.3.5 JVET-AD0205[1], a 3x3 convolution is separated in to a 3x1 convolution and a 1x3 convolution to reduce the computations. In EE1-1.6 JVET-AD0106[2], residual blocks with multi-scale features are used.

The proposed network structure in JVET-AD0211[3] combines the multi-scale features from JVET-AD0106 and the separated 3x1 and 1x3 convolutions from JVET-AD0205.

Four new networks were trained in JVET-AD0211, and this report crosschecked the inference of the int16 networks.

# Results

One simulation with NnFilterSet1 is enabled. The compiler is GCC 7.4.0. Results are on top of NNVC-4.0. Simulation was run with integer models.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.79% | -18.69% | -21.12% | 454% | 136669% |
| Class A2 | -10.81% | -23.47% | -26.77% | 433% | 132883% |
| Class B | -9.86% | -26.46% | -26.04% | 457% | 139144% |
| Class C | -10.99% | -25.53% | -26.16% | 341% | 120783% |
| Class E |  |  |  |  |  |
| Overall | -10.34% | -24.06% | -25.23% | 418% | 132287% |
| Class D | -12.51% | -26.85% | -27.81% | 330% | 118731% |
| Class F | -6.01% | -17.95% | -16.84% | 673% | 61768% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.96% | -21.93% | -20.51% | 368% | 121509% |
| Class E | -9.84% | -22.71% | -23.79% | 950% | 98494% |
| Overall |  |  |  |  |  |
| Class D | -11.22% | -23.72% | -23.39% | 319% | 99736% |
| Class F | -5.58% | -15.31% | -12.29% | 699% | 59982% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.99% | -20.25% | -23.06% | 368% | 101033% |
| Class A2 | -7.06% | -22.72% | -22.88% | 251% | 83301% |
| Class B | -7.35% | -24.01% | -23.71% | 235% | 83911% |
| Class C | -8.31% | -19.13% | -22.23% | 167% | 50723% |
| Class E | -10.72% | -23.81% | -24.64% | 247% | 85127% |
| Overall | -8.02% | -22.05% | -23.29% | 240% | 77481% |
| Class D | -8.09% | -18.19% | -21.95% | 157% | 50042% |
| Class F | -6.27% | -19.26% | -17.82% | 183% | 64317% |

Encoding time and decoding time are not reliable since these were not run on the same machine.

# References

[1] EE1-1.3: Reduced complexity CNN-based in-loop filtering, JVET-AD0205, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 30th Meeting, Antalya, TR, 21–28 April 2023.

[2] EE1-1.6: In-loop filter with wide activation and large receptive field, JVET-AD0106, Y. Li, K. Zhang, L. Zhang, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 30th Meeting, Antalya, TR, 21–28 April 2023.

[3] EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6, JVET-AD0211, Y.Li, S.Eadie, D.Rusanovskyy, M.Karczewicz, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 30th Meeting, Antalya, TR, 21–28 April 2023.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.