| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0355-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0188 (EE2-1.6a: Non-local cross-component prediction and cross-component merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo | Tel: Email: | cheweikuo@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

Introduction

This document reports the crosschecking results of JVET-AD0355 which presents a joint solution of cross-component prediction (CCP) merge mode for chroma intra coding in EE2-1.6 from JVET-AC0176 and JVET-AC0315. The CCP merge mode implies that cross-component model parameters of the current chroma block are inherited from its spatial adjacent, spatial non-adjacent, history-based, or default CCP models. These CCP models are collected from the blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes, and further reordered based on the sum of absolute difference (SAD) cost, which is derived from a neighbouring template of the current block. In Test 1.6a, the merge list includes spatial adjacent, non-adjacent, and default candidates.

Results over ECM-8.0 are crosschecked. It is reported BD-rate points and runtime are matched with the results provided from the proponents.

Table 1 Simulation Results of Test #1.6a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -2.06% | -3.13% | 101% | 101% |
| Class A2 | -0.32% | -3.07% | -3.21% | 102% | 103% |
| Class B | -0.12% | -2.15% | -2.13% | 101% | 102% |
| Class C | -0.07% | -0.68% | -0.84% | 102% | 102% |
| Class E | -0.06% | -1.29% | -0.80% | 101% | 102% |
| Overall | -0.16% | -1.82% | -1.97% | 101% | 102% |
| Class D | -0.07% | -1.00% | -0.69% | 101% | 102% |
| Class F | -0.14% | -1.19% | -1.23% | 101% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -1.05% | -2.43% | 101% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.03% | -1.71% | -1.51% | 101% | 101% |
| Class C | -0.05% | -0.29% | -0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.24% | -0.65% | 101% | 101% |
| Class F | -0.14% | -1.30% | -1.15% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Patent rights declaration(s)

Kwai Inc. does not have any current or pending patent rights relating to the technology described in this contribution.