| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0365-v1 |
| --- | --- |

| Title: | AHG14: The extension of SADL library |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijie Bao, Ding Ding, Jiwei Wang, Jianghao Jia, Zhengzhong Chen | Email: | {baoweijie, dingding, wangjiwei, jiajh2021, zzchen}@whu.edu.cn |
| Source: | Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents extensions in the Small AdHoc Deep Learning (SADL) library version 3.0. Extended features include Conv2DTranspose, Slicing, and PReLU. New features include GirdSample, ScatterND, and Resize (Bilinear Interpolation).

Introduction

The SADL library version 3.0 supports some common features, which contributes to the advancement of NNVC. However, the supported features are limited, such as those related to optical flow calculation in JVET-AC0114 [1]. This contribution extends the SADL library version 3.0 and verifies the extended SADL library.

SADL Extensions

Overview

Table 1 below summarizes the extensions in the SADL library version 3.0.

Table 1: The overview of extensions in the SADL library version 3.0.

| Language | Pure C++, header only |
| --- | --- |
| Footprint | ~1000 LOC, library ~40KB, no dependency |
| Compatibility | Update Onnx to SADL converter |
| Extended features | Conv2DTranspose, Slicing, PReLU |
| New features | GirdSample, ScatterND, Resize (Bilinear Interpolation) |
| Type support | float |

Changelog

Changes include the extensions of original features and the addition of new features:

- Extend Conv2DTranspose layer for 4×4 kernel

- Extend Slicing layer on height and width dimensions

- Extend PReLU layer with multiple α parameters

- Add GridSample layer

- Add ScatterND layer

- Add Resize layer, which currently supports bilinear interpolation

- Update convert.py file to include conversions of the above features

Results

We use the extended SADL library to implement a simplified network structure of JVET-AC0114, which includes all the extended and new features. To ensure the extended SADL library covers the PyTorch features used in JVET-AC0114, we replace some unimplemented features, such as replacing Div with Mul and rewriting flow_warp function to avoid some features. Table 2 shows the performance comparison between PyTorch and SADL models under the same simplified network structure. Table 2 indicates the consistency of the extended and new features between PyTorch and SADL.

Table 2: performance comparison between PyTorch and SADL models (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over PyTorch Implementation on Class D |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| BasketballPass | 0.00% | -0.10% | 0.01% | -0.01% | -0.06% | 0.04% | 245% | 742% |
| BQSquare | 0.01% | 0.02% | 0.03% | -0.01% | 0.00% | 0.03% | 310% | 667% |
| BlowingBubbles | 0.00% | -0.07% | 0.06% | 0.00% | -0.04% | 0.02% | 237% | 653% |
| RaceHorses | -0.01% | 0.00% | -0.18% | -0.05% | -0.05% | -0.12% | 189% | 628% |
| Class D | 0.00% | -0.04% | -0.02% | -0.02% | -0.04% | -0.01% | 242% | 671% |

Conclusions

This contribution presents extensions in the Small AdHoc Deep Learning (SADL) library version 3.0. The experiment indicates the consistency of the extended and new features between PyTorch and SADL.

Reference

[1] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11-20 January 2023.

Patent rights declarations

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.