| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0336-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0167(EE1-1.7-related: Combination test for NNVC based on filter set 0) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AD0167[1] (EE1-1.7-related: Combination test for NNVC based on filter set 0), which tests several combinations of NnIntra, EncOpt, temporal filter and filter set#0.

Test a.1, test a.2, test b.1 and test b.2 have been inference crosschecked. The crosschecker’s results can match proponent’s.

Simulation results

All the tests are carried out under common test condition[2].

Test a.1: NnIntra+NnlfSet0+EncOpt+TemporalFilter vs. NNVC-4.0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.53% | -18.74% | -20.47% | -11.86% | -20.55% | -21.93% | 158% | 43828% |
| Class A2 | -12.43% | -22.03% | -17.72% | -11.96% | -21.61% | -15.59% | 157% | 41247% |
| Class B | -11.45% | -23.05% | -23.51% | -11.05% | -22.48% | -23.00% | 161% | 43173% |
| Class C | -12.50% | -24.68% | -24.49% | -11.73% | -21.66% | -22.24% | 145% | 36177% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.94% | -22.42% | -22.01% | -11.58% | -21.70% | -21.10% | 155% | 40934% |
| Class D | -14.23% | -24.59% | -25.82% | -11.15% | -22.69% | -21.24% | 140% | 34699% |
| Class F | -6.37% | -14.84% | -13.78% | -7.01% | -16.11% | -14.80% | 177% | 28436% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.09% | -19.95% | -22.16% | -12.70% | -22.78% | -22.41% | 276% | 33619% |
| Class A2 | -9.82% | -20.13% | -16.86% | -10.56% | -19.96% | -15.07% | 237% | 28214% |
| Class B | -10.16% | -19.58% | -20.85% | -10.14% | -21.09% | -22.36% | 229% | 26508% |
| Class C | -11.20% | -20.98% | -23.25% | -10.97% | -22.25% | -24.00% | 202% | 19387% |
| Class E | -14.88% | -22.52% | -23.47% | -15.21% | -21.68% | -24.30% | 228% | 30088% |
| Overall | -11.27% | -20.53% | -21.37% | -11.67% | -21.54% | -21.84% | 231% | 26550% |
| Class D | -10.88% | -19.80% | -23.01% | -10.02% | -21.63% | -23.45% | 188% | 16864% |
| Class F | -7.12% | -15.33% | -15.07% | -6.96% | -17.63% | -17.42% | 156% | 21567% |

Test a.2: NnIntra+NnlfSet0 vs. NNVC-4.0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.37% | -19.98% | -22.72% | -13.77% | -22.25% | -23.46% | 162% | 55954% |
| Class A2 | -11.69% | -22.09% | -16.95% | -11.58% | -21.73% | -14.98% | 154% | 52449% |
| Class B | -11.11% | -23.54% | -23.92% | -10.72% | -23.38% | -23.45% | 158% | 54765% |
| Class C | -11.70% | -25.68% | -25.69% | -11.27% | -22.24% | -23.10% | 140% | 47161% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.63% | -23.11% | -22.76% | -11.65% | -22.52% | -21.66% | 153% | 52397% |
| Class D | -12.77% | -25.03% | -26.60% | -10.22% | -22.31% | -23.23% | 138% | 45121% |
| Class F | -5.79% | -15.58% | -14.49% | -6.27% | -16.26% | -16.19% | 185% | 24071% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.78% | -19.85% | -22.19% | -12.42% | -22.73% | -22.57% | 246% | 32934% |
| Class A2 | -9.39% | -20.07% | -16.68% | -10.14% | -19.99% | -14.96% | 207% | 27900% |
| Class B | -9.73% | -19.78% | -20.91% | -9.71% | -21.15% | -22.46% | 204% | 26222% |
| Class C | -10.76% | -21.44% | -23.44% | -10.53% | -22.36% | -24.02% | 177% | 19212% |
| Class E | -14.41% | -22.41% | -23.33% | -14.72% | -21.63% | -24.14% | 205% | 30461% |
| Overall | -10.86% | -20.65% | -21.38% | -11.25% | -21.57% | -21.86% | 205% | 26332% |
| Class D | -10.42% | -20.15% | -23.25% | -9.49% | -21.61% | -23.75% | 162% | 16346% |
| Class F | -6.79% | -15.32% | -14.96% | -6.53% | -17.64% | -17.42% | 150% | 22703% |

Test b.1: NnIntra+NnlfSet0(tf)+EncOpt+TemporalFilter vs. NNVC-4.0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.07% | -24.11% | -25.23% | -12.48% | -27.15% | -27.42% | 141% | 39061% |
| Class A2 | -13.34% | -29.56% | -31.27% | -12.74% | -29.70% | -30.50% | 130% | 36684% |
| Class B | -12.26% | -33.73% | -30.03% | -11.50% | -32.11% | -29.87% | 182% | 47760% |
| Class C | -12.85% | -27.05% | -26.62% | -12.18% | -25.12% | -25.38% | 255% | 62935% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -12.60% | -29.19% | -28.41% | -12.12% | -28.77% | -28.31% | 177% | 46842% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.14% | -25.01% | -28.76% | -13.84% | -28.13% | -29.93% | 341% | 40109% |
| Class A2 | -10.91% | -29.13% | -31.14% | -11.67% | -29.94% | -28.94% | 277% | 30614% |
| Class B | -11.25% | -28.24% | -27.17% | -11.11% | -29.25% | -28.80% | 265% | 35360% |
| Class C | -11.75% | -24.24% | -26.53% | -11.64% | -25.78% | -27.71% | 231% | 26373% |
| Class E | -15.96% | -28.41% | -29.11% | -15.85% | -27.72% | -30.33% | 260% | 37926% |
| Overall | -12.24% | -26.99% | -28.28% | -12.57% | -28.15% | -29.03% | 269% | 33417% |
| Class D | -11.06% | -24.03% | -26.17% | -10.30% | -26.32% | -27.01% | 218% | 26704% |

Test b.2: NnIntra+NnlfSet0(tf) vs. NNVC-4.0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.07% | -24.11% | -25.23% | -12.48% | -27.15% | -27.42% | 141% | 39061% |
| Class A2 | -13.34% | -29.56% | -31.27% | -12.74% | -29.70% | -30.50% | 130% | 36684% |
| Class B | -12.26% | -33.73% | -30.03% | -11.50% | -32.11% | -29.87% | 182% | 47760% |
| Class C | -12.85% | -27.05% | -26.62% | -12.18% | -25.12% | -25.38% | 255% | 62935% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -12.60% | -29.19% | -28.41% | -12.12% | -28.77% | -28.31% | 177% | 46842% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.14% | -25.01% | -28.76% | -13.84% | -28.13% | -29.93% | 341% | 40109% |
| Class A2 | -10.91% | -29.13% | -31.14% | -11.67% | -29.94% | -28.94% | 277% | 30614% |
| Class B | -11.25% | -28.24% | -27.17% | -11.11% | -29.25% | -28.80% | 265% | 35360% |
| Class C | -11.75% | -24.24% | -26.53% | -11.64% | -25.78% | -27.71% | 231% | 26373% |
| Class E | -15.96% | -28.41% | -29.11% | -15.85% | -27.72% | -30.33% | 260% | 37926% |
| Overall | -12.24% | -26.99% | -28.28% | -12.57% | -28.15% | -29.03% | 269% | 33417% |
| Class D | -11.06% | -24.03% | -26.17% | -10.30% | -26.32% | -27.01% | 218% | 26704% |

Reference

[1] R. Chang, L. Wang, X. Xu, S. Liu. “EE1-1.7-related: Combination test for NNVC based on filter set 0,” JVET-AD0167, April. 2023

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.