| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0370-v1 |
| --- | --- |

| Title: | AHG9: Editorial improvement for signalling of NNPFC purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes editorial changes to nnpfc_purpose signalling. Instead of singalling different meaningful bits in a single syntax element, it is proposed to split the syntax into several separate flags. This allows the specification to refer to a meaningful syntax element name instead of and and operation of the syntax element and a mask value.

# Introduction

In the current NNPFC syntax, purpose as a signalled as a single 16-bit syntax element:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| […] |  |

The in semantics the syntax element is interpreted as a bit field, with each bit indicating a different purpose:

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |

The value of 0 has the special meaning that the purpose is to be determined by the application.

When the specification needs to refer to a specific purpose, it always has to refer to a bitwise and operation on nnpfc_purpose, e.g. ( nnpfc_purpose & 0x02 )  !=  0 for chroma upsampling.

The reference to that bitwise and operation does not supply any context to the reader of the specification and may be easy to confuse, e.g. ( nnpfc_purpose & 0x02 ) and ( nnpfc_purpose & 0x20 ).

# Proposed Syntax and Semantics

To improve readability, it is proposed to replace the syntax element nnpfc_purpose with a separate flag for each purpose:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_general_visual_quality_improvement_flag | u(1) |
| nnpfc_chroma_upsampling_flag | u(1) |
| nnpfc_resolution_upsampling_flag | u(1) |
| nnpfc_picture_rate_upsampling_flag | u(1) |
| nnpfc_bit_depth_upsampling_flag | u(1) |
| nnpfc_colourization_flag | u(1) |
| nnpfc_purpose_reserved_zero_10bits | u(10) |
| […] |  |

The syntax is equivalent with the existing syntax, but the order of bits is reversed. If the order of the syntax elements in the table above is reversed, the proposed change is bit exact and thus purely editorial.

Semantics:

The variable nnpfcPurposeIndicated is derived as follows:

- If one or more of the following conditions is true, nnpfcPurposeIndicated is set equal to TRUE:

- nnpfc_general_visual_quality_improvement_flag is equal to 1;

- nnpfc_chroma_upsampling_flag is equal to 1;

- nnpfc_resolution_upsampling_flag is equal to 1;

- nnpfc_picture_rate_upsampling_flag is equal to 1;

- nnpfc_bit_depth_upsampling_flag is equal to 1;

- nnpfc_colourization_flag is equal to 1;

- nnpfc_purpose_reserved_zero_10bits is not equal to 0;

- Otherwise, nnpfcPurposeIndicated is set equal to FALSE.

When nnpfcPurposeIndicated is equal to FALSE, the purpose of the NNPF is determined by the application.

nnpfc_general_visual_quality_improvement_flag equal to 1 indicates the purpose of the NNPF as general visual quality improvement. When nnpfcPurposeIndicated is equal to FALSE and nnpfc_general_visual_quality_improvement_flag is equal to zero, it is indicated the purpose of the NNPF is not general visual quality improvement.

nnpfc_chroma_upsampling_flag equal to 1 indicates the purpose of the NNPF as chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format). When nnpfcPurposeIndicated is equal to FALSE and nnpfc_chroma_upsampling_flag is equal to zero, it is indicated the purpose of the NNPF is not chroma upsampling.

When ChromaFormatIdc is equal to 3, nnpfc_chroma_upsampling_flag shall be equal to 0.

nnpfc_resolution_upsampling_flag equal to 1 indicates the purpose of the NNPF as resolution upsampling. When nnpfcPurposeIndicated is equal to FALSE and nnpfc_resolution_upsampling_flag is equal to zero, it is indicated the purpose of the NNPF is not resolution upsampling.

nnpfc_picture_rate_upsampling_flag equal to 1 indicates the purpose of the NNPF as picture rate upsampling. When nnpfcPurposeIndicated is equal to FALSE and nnpfc_picture_rate_upsampling_flag is equal to zero, it is indicated the purpose of the NNPF is not picture rate upsampling.

nnpfc_bit_depth_upsampling_flag equal to 1 indicates the purpose of the NNPF as bit depth upsampling (increasing the luma bit depth or the chroma bit depth). When nnpfcPurposeIndicated is equal to FALSE and nnpfc_bit_depth_upsampling_flag is equal to zero, it is indicated the purpose of the NNPF is not bit depth upsampling.

nnpfc_colourization_flag equal to 1 indicates the purpose of the NNPF as colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format). When nnpfcPurposeIndicated is equal to FALSE and nnpfc_colourization_flag is equal to zero, it is indicated the purpose of the NNPF is not colourization.

When ChromaFormatIdc or nnpfc_chroma_upsampling_flag is not equal to 0, nnpfc_colourization_flag shall be equal to 0.

nnpfc_purpose_reserved_zero_10bits shall be equal to 0 in bitstreams conforming to this edition of this document. Other values are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose_reserved_zero_10bits not equal to 0.

NOTE 2– When a reserved value of nnpfc_purpose_reserved_zero_10bits is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

In the remaining text all occurrences all occurrences of “( nnpfc_purpose & 0xXX )” shall be replaced by the appropriate flag and occurrences of “nnpfc_purpose is equal to zero” shall be replaced by “nnpfcPurposeIndicated is equal to FALSE”.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).