| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0373-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0239 (Non-EE2: Fixes for IntraTMP template availability) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Ruiz Coll | Tel: Email: | duizcoll@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AD0239 (Fixes for IntraTMP template availability), which proposes to fix two bugs in the IntraTMP implementation in the current ECM-8.0. The first bugfix is related to checking the availability of the left template of the current block. The second bugfix concerns the reference block templates located at picture boundaries. The BD-rate results available in this crosscheck report match those provided by the proponent.

# Introduction

IntraTMP was introduced in JVET-V0130 “EE2: Intra Template Matching” [1], which was enabled if the L-shape template of both the current block and reference block were available.

In JVET-W0069 “On Intra TMP Boundary Conditions” [2] an extension using either left or above templates when only one of them is available was proposed.

The two following problems have been asserted in JVET-AD0239:

1.	The left template of the current block should be chosen in case all the adjacent CUs to the left are available. Instead, a condition of top-left CU availability is applied. Consequently, the IntraTMP for the case where the left template of the current block is only available, is performed only if the template of the top-left CU is availability.

2.	When only left or above template is used (it is derived by the template availability of the current block), the reference block may be located at the picture boundary having available only left or above template accordingly, however in the current design both templates have to be available meaning that the reference block cannot be located at the picture boundary.

A Crosscheck of the source code and simulation results has been conducted. Inspected source code follows the description in the proposal. The crosscheck has been performed on top of ECM-8.0 in All Intra and Random Access configurations. Two tests have been performed:

- Test-1: Fixes to both bugs have been enabled and test results are reported on top of the ECM-8.0.

- Test-2: Fixes to both bugs have been enabled and test results are reported on top of the EE2-1.20j

The crosscheck results are provided in the tables below and match the test results provided by the proponent. Note that coding time may not be accurate. Below are the results for Test-1 and Test-2:

Test-1 (Bugfixes 1 and 2 on top of ECM-8.0)

Table I. Proposed method (bugfixes 1 & 2) on top of ECM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | 0.06% | 96% | 98% |
| Class A2 | -0.01% | -0.01% | 0.02% | 100% | 93% |
| Class B | 0.00% | -0.02% | 0.00% | 99% | 99% |
| Class C | -0.01% | -0.03% | 0.07% | 101% | 112% |
| Class E | 0.00% | -0.06% | 0.09% | 99% | 103% |
| Overall | 0.00% | -0.04% | 0.05% | 99% | 101% |
| Class D | 0.01% | 0.03% | -0.11% | 101% | 121% |
| Class F | -0.01% | 0.00% | 0.00% | 99% | 99% |
| Class TGM | -0.01% | 0.02% | 0.03% | 99% | 83% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.01% | 102% | 101% |
| Class A2 | -0.01% | 0.09% | 0.12% | 102% | 102% |
| Class B | 0.00% | -0.01% | -0.02% | 102% | 102% |
| Class C | -0.01% | 0.05% | 0.11% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.05% | 102% | 102% |
| Class D | -0.03% | -0.09% | -0.31% | 102% | 101% |
| Class F | -0.01% | -0.05% | 0.06% | 102% | 101% |
| Class TGM | 0.00% | -0.04% | 0.00% | 99% | 97% |

Test-2 (Bugfixes 1 and 2 on top of EE2-1.20j)

Table II. The proposed method (bugfixes 1 & 2) on top of EE2-1.20j

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.20j |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.13% | 102% | 95% |
| Class A2 | 0.00% | 0.03% | 0.03% | 101% | 100% |
| Class B | -0.04% | -0.07% | -0.06% | 102% | 95% |
| Class C | -0.01% | -0.20% | -0.04% | 100% | 88% |
| Class E | -0.05% | 0.06% | -0.13% | 103% | 93% |
| Overall | -0.02% | -0.05% | -0.06% | 101% | 94% |
| Class D | -0.04% | -0.12% | -0.08% | 99% | 79% |
| Class F | 0.00% | 0.09% | 0.00% | 98% | 100% |
| Class TGM | -0.04% | -0.05% | -0.05% | 104% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.20j |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Conclusion

Results reported in JVET-AD0239 (Fixes for IntraTMP template availability) have been confirmed. It is recommended that both bugfixes will be adopted in the ECM-9.0.

# Reference

[1]	K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, “EE2: Intra Template Matching,” JVET-V0130, April 2021.

[2]	K. Naser, T. Poirier, F. Le Leannec, G. Martin-Cocher, “AHG12: On Intra TMP Boundary Conditions,” JVET-W0069, July 2021.

[3]	ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[4]	V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023.

[5]	M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration

Ofinno LLC does not have any current or pending patent rights relating to the technology described in this contribution.