| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0377-v1 |
| --- | --- |

| Title: | AHG11: On unified NN filter design for NNVC high performance tier |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Junru Li, Chaoyi Lin, Kai Zhang, Li Zhang (Bytedance) Dmytro Rusanovskyy, Samuel Eadie, Yun Li, Marta Karczewicz (Qualcomm) | Tel: Email: | yue.li@bytedance.com lijunru@bytedance.com linchaoyi.cy@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com dmytror@qti.qualcomm.com seadie@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Bytedance Inc., Qualcomm Inc. |  |  |

_____________________________

- Abstract

This contribution presents a unified design proposal for high performance tier of the NNVC loop filter. Proposed design incorporates elements major NNVC loop filter technologies studied in EE1 for this meeting. For the purpose of practicality, and to reduce uncertainty of the results, it is proposed to employ an existing training and inference software package developed and cross-checked in the EE1-1.6 submission. Additionally, elements of other perspective designs studied in EE1 for meeting are integrated into existing and verified SW package.

It is asserted that the proposed solution has following benefits:

- Simplifying the R&D and design evaluation, since existing package is known to community and being deployed/cross-checked by multiple participants.

- Increase possibility of achieving stable result in next meeting

- Eliminating an uncertainty potentially originated with an arbitrary selecting of network architecture and training strategies.

- Minimizing the efforts on adjustment/calibration/finetuning/harmonization of numerous tricky and interacted aspects (learning rate, channel numbers, layers, optimizer, training logic)

Introduction

NNVC common software includes two sets of deep in-loop filter, known as filter set #1 and filter set #0 respectively. The basic building blocks for the two filter sets are shown in Fig.1 (a) and Fig. 1 (b). It can be observed that the residual block in Fig. 1 (a) has relatively larger receptive field while the residual block in Fig. 1 (b) has relatively wider activation. To combine the strengths of both residual blocks, JVET-AD0106 [1] (EE1-1.6) proposes a NN filter design based on a new type of residual block with both wide activation and large receptive field as shown in Fig. 2. Currently, EE1-1.6 achieves the highest performance among those solutions that have been fully crosschecked in EE1 and implemented using SADL int16, with a reasonable complexity as shown in Fig. 3.

Fig. 1. (a) Residual block in filter set #1. (b) Residual block in filter set #0, M > K.

Fig. 2. Architecture of the proposed deep in-loop filter.

Fig. 3. Filter architecture tests & NNVC-4.0 tools. These solutions have been fully training cross-checked, implemented in Int16 SADL. Combined YUV metrics are obtained by bd-rate (6Y+U+V)/8, complexity is measured with YUV 4:2:0 kMAC/pixel

Proposed network architecture

A list of design elements that have been understood as vital for network architecture design, and their usage in EE1 tests, are shown in the columns of Table 1.

This contribution proposes two test models under consideration (TMuC) as unified filter design.

Table 1. Elements proven to be vital for NN filter network architecture

|  | Separate Y, U, V FS1 derivatives | Unified IPB model FS1 derivatives | Long Skip FS0 FS1 | Long/wide Activation FS0 EE1-1.1 EE1-1.3 EE1-1.6 | RBs | Pred | BS FS1 EE1-1.3 | baseQP | sliceQP | CONV for side info | Slice Type | IPB EE1-1.4 | Channels redistribution EE1-1.4 | Transformers EE1-1.7 | CONV decompos. EE1-1.1 EE1-1.3 | Multi-scale feature extraction EE1-1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EE1-1.6 | √ | √ | √ | √ | √ | √ | √ | √ | × | × | √ | × | × | × | × | √ |
| TMuC1 | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | × | √ | √ |
| TMuC2 | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |

TMuC1

The proposed unified filter design without transformer is shown in Fig 4. The missing key elements except transformer block have been integrated into EE1-1.6 network architecture. Specifically, network head part is modified to enable features extraction from newly added inputs, i.e. sliceQp (EE1-1.7) and IPB (EE1-1.4). Redistributed channel numbers are used for inputs of different importance’s (EE1-1.4). Note larger sized "conv" in Fig. 4 indicates more channels, and vice versa. In addition, 3×3 layer in the backbone block is decomposed into 1×3 and 3×1 layers (EE1-1.3). A feasibility of combination of multi-scale feature extraction and convolution decomposition have been demonstrated in EE-Related input: JVET-AD0211.

Fig. 4. Architecture of the proposed unified filter design without transformer. Elements newly added on top of EE1-1.6 have been highlighted.

TMuC2

Additionally, acknowledging preliminary performance demonstrated by transform usage in EE1-1.7c, an alternative test mode design can be considered. The proposed unified filter design with transformer is shown in Fig 5. Compared with TMuC1, TMuC2 additionally introduces transformer block.

Fig. 6. Architecture of the proposed unified filter design with transformer. Elements newly added on top of EE1-1.6 have been highlighted.

Proposed NN filter interface

The elements that have been proven to be vital for NN filter interface design are shown in the columns of Table 2. It can be observed that EE1-1.6 filter interface contains most vital elements. It is therefore proposed to integrate additional missing elements on top of the interface used for EE1-1.6, to facilitate an easier, safer, and faster implementation.

|  | Pre-processing and post-processing of luma | Pre-processing and post-processing of chroma | Enable DBF/SAO | Blending with DBF/SAO | Combination with SAO | Slice-level/Block-level blending with adaptive scale factor | Slice-level/Block-level blending with fixed scale factor | Separate adaptive scale factor for U and V | BaseQP adjustment | Encoder-only Optimization | Temporal filter | Adaptive inference granularity | configurable inference size | configurable block extension size | configuration QP params number | block-level QP adaptation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EE1-1.6 | × | × | DBF only | DBF | × | √ | × | × | √ (-5, -10, +5) | √ | √ | √ | √ | √ | √ | √ |
| TMuC1 | × | × | DBF, SAO | DBF | √ | √ | √ | √ | √ (-5, -10, +5) | √ | √ | √ | √ | √ | √ | √ |
| TMuC2 | × | × | DBF, SAO | DBF | √ | √ | √ | √ | √ (-5, -10, +5) | √ | √ | √ | √ | √ | √ | √ |

Conclusions

This contribution presents a unified design proposal for high performance tier of the NNVC loop filter. Proposed design incorporates elements of major NNVC loop filter technologies studied in EE1 for this meeting. For the purpose of practicality, and to reduce uncertainty of the results, it is proposed to employ an existing training and inference software package developed and cross-checked in the EE1-1.6 submission. Additionally, elements of other perspective designs studied in EE1 for meeting are integrated into existing and verified SW package.

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.6: In-Loop Filter with wide activation and large receptive field, JVET-AD0106, Antalya, TR, April 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).