| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0379-v1 |
| --- | --- |

| Title: | AHG11: Unified NNLF solution for high operation point |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes the unified NN-based loop filter solution for high operation point. Both the unified network structure solution and interface solution are proposed. In the unified network structure aspects, all the existing advantages of network architecture in NNVC-4.0 or EE proposals are combined tentatively. In the unified interface aspects, the existing different interfaces are combined to form a more comprehensive interface.

Introduction

In this contribution, two unified network solutions and one unified interface solution are proposed. Specifically, several effective elements from NNVC-4.0 or EE proposals are integrated to combine a new network architecture in the first unified network solution. Considering the effectiveness of transformer, another unified network solution with transformer is also proposed. For the interface aspects, a comprehensive interface is proposed based on existing different interfaces.

Proposed method

The unified network implementation with SADL and quantization is firstly presented in Section 2.1. Section 2.2 presents the unified network implementation with transformer. The unified interface implementation is provided in Section 2.3.

Unified network implementation with SADL and quantization

The network structure of unified network implementation with SADL and quantization is shown in Fig. 1.

Several effective elements are integrated into the network containing input part, head part, body part, tail part and output part. In the input part, reconstruction, prediction, boundary strength (BS), sliceQP and baseQP are fed into the network. The sliceType or IPB is also used to unify the intra and inter models. For the resBlock in the body part, several effective elements including long/wide activation, large receptive field and convolution with decomposition are used to build a new effective structure.

Fig. 1: Network architecture of unified network implementation with SADL and quantization

Unified Network Implementation with Transformer

Since the advanced network structure is beneficial to the exploration of NNVC and transformer shows very promising performance according to JVET-AC-EE1-1.7 [], the unified network implementation with transformer is also proposed and shown in Fig. 2.

Fig. 2: Network architecture of unified network implementation with transformer

The unified network solution with transformer is implemented on the top of the unified network solution with SADL and quantization. Specifically, the transformer block is added into the resblock to build the resTransBlock and placed after the last convolution layer.

Unified Interface Implementation

In this contribution, it is recommended that define a unified interface can infer both the high complexity and low complexity networks. There are three main interfaces used in this meeting, so some general NN-filter methods without re-training need to spend much more effort to verify performance on different interfaces, which is not conducive to the development of AHG11. Therefore, the unified interface implementation is proposed as follows.

|  | Filter set#0 | Filter set#1 | EE1-1.1 | EE1-1.3 | EE1-1.4 | EE1-1.5 | EE1-1.6 | EE1-1.7 | Unified |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-processing and post-processing of luma | × | × | √ |  |  |  |  |  | √ |
| Pre-processing and post-processing of chroma | √ | × | × |  |  |  |  |  | √ |
| Enable DBF/SAO | both | DBF only | both |  |  |  |  |  | both |
| Blending with the DBF/SAO | SAO | DBF | DBF |  |  |  |  |  | SAO |
| Combination with SAO | √ | × | × |  |  |  |  |  | √ |
| Slice-level/Block-level blending with adaptive scale factor | √ | √ | Slice-level only |  |  |  |  |  | √ |
| Slice-level/Block-level blending with fixed scale factor | √ | × | × |  |  |  |  |  | √ |
| Separate adaptive scale factor for U and V | √ | × | √ |  |  |  |  |  | √ |
| Base QP adjustment | (-5, +5) | (-5, -10, +5) | × |  |  |  |  |  | (-5, -10, +5) |
| Encoder-only Optimization | √ | √ | × |  |  |  |  |  | √ |
| Temporal filter | × | √ | × |  |  |  |  |  | √ |
| Adaptive inference granularity | × | √ | × |  |  |  |  |  | √ |
| Model selection | × | × | √ |  |  |  |  |  | √ |
| configurable inference size | x | √ | × |  |  |  |  |  | √ |
| configurable block extension size | x | √ | × |  |  |  |  |  | √ |
| configuration QP params number | x | √ | × |  |  |  |  |  | √ |
| Block-level QP adaptation | x | √ | × |  |  |  |  |  | √ |

Conclusions

In this contribution, two unified solutions for the network structure aspects and one unified solution for the interface aspects are proposed to facilitate the development of AHG11, especially for the high operation point. In the network structure aspects, the unified network implementation with SADL and quantization, and the unified network implementation with transformer are proposed to combine the existing effective elements for the network structure. In the interface aspects, the unified interface implementation is proposed to combine three different existing interfaces.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.7: Optimization of training and network for NNVC filter set 0. JVET-AD0166, 30th Meeting, Antalya, TR, 21–28 April 2023.