| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0382-v2 |
| --- | --- |

| Title: | Results of expert viewing for testing of FGC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien RWTH Aachen University Aachen Germany | Tel: Email: | +49-241-8027867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document reports the results of an on-site expert viewing performed at the 30th JVET meeting in Antalya. The evaluation targets at goal 1 of the test plan document JVET-AC2022, i.e., the assessment of subjective quality improvement with and without film grain synthesis. This assessment evaluates the benefits of film grain synthesis as a tool for improving the visual quality of a compressed video sequence. The results of two tests are reported: A DCR test of test points selected to cover the MOS range, and an A-B comparison in the high-quality range where the film grain is just preserved by coding or synthetically added.

The results of this assessment indicate a potentially significant visual benefit by applying film grain synthesis for improving the visual quality of a compressed video sequence. In the evaluated set of bitrates, gains are specifically observer for mid- to high-bitrate ranges. At low bitrates, the capability of masking compression artifacts by FGS is decreasing. In the DCR test, it is observed that the impact of FGS is strongly dependent on the test sequence. For CrowdRun (scanned from film), a tendency towards improvement is observed but the confidence intervals overlap for all rate points. OldTownCross shows early saturation when FGS is applied. The distance of the MOS curves indicates significant improvements for this sequence. The test sequence DinnerScene has be acquired by a digital camera. For this sequence, significant improvements are indicated with the confidence intervals between the two curves being just non-overlapping for the two middle rate points.

The A-B comparison of the uncompressed sequence against a low QP for each sequence indicates that the preservation of the original filmgrain has been achieved. For all three sequences, both were considered equal within the confidence interval by the viewers. The comparison of the selected rate points with FGS indicate a bitrate saving of more than 90% at a marginal subjective quality loss.

# Introduction

This document reports the results of an on-site expert viewing performed at the 30th JVET meeting in Antalya. The evaluation targets at goal 1 of the test plan document JVET-AC2022, i.e., the assessment of subjective quality improvement with and without film grain synthesis. This assessment evaluates the benefits of film grain synthesis as a tool for improving the visual quality of a compressed video sequence.

# Test setup

## Logistics

The setup at the meeting site included a PC with a Decklink video board for HDMI connection and SSD drives capable of stable playout of the raw YUV data at the required frame rate.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Samsung 65” S95B, HDMI (3840×2160), 10bit input |
| Viewing distance | 3 viewers sitting at 1.5H, 2 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 20 (6 female, 14 male) |

Participants confirmed visual acuity and normal colour vision.

The setup with three viewers sitting and two viewers standing right behind them at about the same distance was chosen to maximize the number of collected scores per session. Two groups of five viewers each alternated in the viewing task. The viewers entered their scores into the test database in the break after each viewing session. This process was repeated two times resulting in a total of 20 viewers.

## Test sequences and quantizer settings

For evaluation, the test sequences and quantizer settings listed in JVET-AC2022 were tested. The data was made available on the JVET ftp site ftp.ient.rwth-aachen.de in the directory ./ahg/viewingtests/JVET-AC2022 (regular JVET login). The directory includes bitstreams, a pre-compiled tool to apply the film grain characteristics parameters, corresponding configuration files and a script to apply the FG parameters. The tool is available in the JVET gitlab (https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt/vfgs).

Table 1 – Candidate test sequences and QP settings for “quality improvement” category [1]

| Test sequence & format | QPs | Thumbnail |
| --- | --- | --- |
| CrowdRun (from SVT FairyTale) 3840×2160, 50fps, 500 frames Kodak Vision 250D film | 16 (grain just preserved) 26 (additional low QP point with FGS) 29 34 38 42 |  |
| OldTownCross (from SVT FairyTale) 3840×2160, 50fps, 500 frames Kodak Vision 250D film | 16 (grain just preserved) 26 (additional low QP point with FGS) 29 34 38 42 |  |
| DinnerScene (from Netflix Chimera) 4096×2160, 60fps, 600 frame RED Epic Dragon digital camera | 14 (grain just preserved) 21 (additional low QP point with FGS) 29 32 36 40 |  |

## Test method and test design

The viewers participated in two different test sessions. First, an assessment of the four highest QPs of each test sequences was conducted using the Degradation Category Rating (DCR) method [2]. Both, the reconstructed video with and without application of FGS were evaluated. In a second session, A-B comparisons are show, comparing the lowest QP point without applying FGS a) to the uncompressed sequence, and b) to the lowest QP using FGS as listed in Table 1.

### Degradation category rating

#### Session setup

The test sequences were evaluated using the 11-grade scale as specified in Rec. ITU-R BT.500-14[3], shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale as specified in Rec. ITU-R BT.500-14 Table 2-4

The test session included 31 basic test cells (BTCs) showing the uncompressed video sequence and one processed video sequence. Each BTC was structured as followed (PVS denotes the processed video sequence under evaluation):

“Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS] (10sec) – “Vote <N>” (5sec)

The session had a duration of approx. 15min. It included a stabilization phase and three trapping BTCs where uncompressed sequences were to be scored.

#### Training

The three test sequences were shown at different compression rates, to provide an overall indication of the impairments, including versions with and without synthesized film grain. Before the presentation of the training session, the experts were instructed on the meaning of the impairment scale. The viewers were advised to score the PVSs according to their impression relative to the impairment scale. Any occurring requests and questions on the test procedure or the scale were answered. The training session included 5 BTCs. The participants were advised to calibrate their personal voting scale during the training session and apply it in the actual test sessions. The first BTC showed the uncompressed sequence as the PVS. After presentation of this BTC, the viewers were asked about their impression and their score and the fact that the original has been presented was revealed. After this, four BTCs were shown covering the expected impairment range, employing two sequences with and two sequences without film grain synthesis.

### A-B comparison

#### Session setup

The test sequences were evaluated using a 5-grade scale as shown in Figure 2 below. The scale was explained as one being much better, or better, or equal to the other presented PVS.

Figure 2: Example for the scoring sheet used for scoring the A-B comparison

The test session included 9 BTCs showing the uncompressed video sequence and two processed video sequences. The allocation of PVSs to slot A or slot B was randomized. The uncompressed sequence was shown to provide the viewers with the impression of the undistorted sequence. Since subtle differences between the PVSs were to be assessed, the presentation of the test sequences was repeated. Thereby, the viewers were enabled to confirm their impression gained after the first comparison. Each BTC was structured as followed:

“Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS A] (10sec) – [PVS B] (10sec) “Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS A] (10sec) – [PVS B] (10sec) – “Vote <N>” (5sec)

The session included a stabilization phase of three BTCs.

#### Training

In the training session, the viewers were familiarized with the structure of the video session including presentation of an example BTC. The scoring scale was explained. The viewers were advised to rate the PVSs according to their overall impression of the presented content. Any occurring requests and questions on the test procedure or the scale were answered.

# Results and analysis

## Data processing

### DCR evaluation

As a first step, the complete set of votes experts was screened for the trapping sequences. In a total of 3 cases, viewers were voting below score of 8 for the original. The results of the affected viewers not regarded in the evaluation. As a second step, isolated outliers -considered to be obvious errors- were removed.

### A-B comparison

For numerical evaluation, the five scores (A>>B, A>B, A==B, A<B, A<<B) were mapped to the values (2, 1, 0, -1, -2) and the random A-B assignment was reverted.

Three of the BTCs in this evaluation were shown once in the stabilization phase and once for evaluation. Besides a single deviation in the first BTC for one viewer, the scores for all viewers were within minor deviation between the stabilization phase and the evaluation phase. It was decided not to apply further processing to the scores collected in the evaluation phase.

## Subjective results

### DCR evaluation

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated.

Figure 3: MOS-over-rate plots for the DCR experiment

### A-B comparison

The measured CMOS values comparing the PVs are plotted on the 5-grade scale for each sequence. The ±95% confidence intervals for the MOS values are indicated. Table 1 provides a comparison of the bitrates of the bitstreams in the reported comparison.

Figure 3: Result of the A-B comparison

In the comparison of the uncompressed sequence to the lowest QP without FGS, positive numbers indicate the compressed version to be scored better than the uncompressed sequence, negative numbers indicate the compressed version to be scored worse than the uncompressed sequence. In the comparison of the sequence with and without FGS, positive numbers indicate the FGS version to be scored better, negative numbers indicate the FGS version to be scored worse than the low QP bitstream without FGS.

Table 1: Bitrate comparison for the rate points in the A-B comparison

|  | no FGS |  | FGS |  |  |
| --- | --- | --- | --- | --- | --- |
|  | low QP | bitrate [kbit/s] | QP | bitrate [kbit/s] | rate delta |
| CrowdRun | 16 | 522183.50 | 26 | 37733.85 | -92.77% |
| DinnerScene | 14 | 359735.93 | 21 | 12935.66 | -96.40% |
| SVT_OldTownCross | 16 | 129379.85 | 26 | 12380.28 | -90.43% |

# Discussion

The results of this assessment indicate a potentially significant visual benefit by applying film grain synthesis for improving the visual quality of a compressed video sequence. In the evaluated set of bitrates, gains are specifically observer for mid- to high-bitrate ranges. At low bitrates, the capability of masking compression artifacts by FGS is decreasing. In the DCR test, it is observed that the impact of FGS is strongly dependent on the test sequence. For CrowdRun (scanned from film), a tendency towards improvement is observed but the confidence intervals overlap for all rate points. OldTownCross shows early saturation when FGS is applied. The distance of the MOS curves indicates significant improvements for this sequence. The test sequence DinnerScene has be acquired by a digital camera. For this sequence, significant improvements are indicated with the confidence intervals between the two curves being just non-overlapping for the two middle rate points.

The A-B comparison of the uncompressed sequence against a low QP for each sequence indicates that the preservation of the original filmgrain has been achieved. For all three sequences, both were considered equal within the confidence interval by the viewers. The comparison of the selected rate points with FGS indicate a bitrate saving of more than 90% at a marginal subjective quality loss.

# References

- P. de Lagrange, W. Husak, M. Radosavljević, M. Wien, “Draft plan for subjective quality testing of FGC SEI message,” Doc. JVET-AC2022, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 29th meeting, online, Jan. 2022.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.