| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0383-v1 |
| --- | --- |

| Title: | AHG9: Modification to the signalling of output picture size in NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Sachin Deshpande | Email: | dr.hendry@lge.com yekui.wang@bytedance.com sdeshpande@sharplabs.com |
| Source: | LG Electronics Inc., Bytedance Inc., and Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution provides modification to the signalling of output picture size that is present when the purpose of the neural-network post-filter (NNPF) includes spatial-resolution upsampling after discussion based on contributions JVET-AD0067 item 5, JVET-AD0091 item 3, and JVET-AD0143 item 2 and Ballot Comment #013.

# Introduction

Contributions JVET-AD0067, JVET-AD0091, and JVET-AD0143 and Ballot Comment #013 suggest that signalling of picture output size from an neural-network post-filter (NNPF) when the purpose of the NNPF includes spatial-resolution upsampling is based on ratio of the input picture size (e.g., 2x, 1.5x, etc) to the filter. It is asserted that such signalling allows one spatial-resolution-upsampling NNPF to be used by different (sets of) cropped decoded output pictures with different spatial resolutions within the same CLVS.

Based on the discussion in the Break Out Group, the following design points were recommended:

- Replace the signalling with ratio.

- Signal the syntax elements for the ratio using “minus1” term.

- Code the syntax elements for the ratio with ue(v) coding.

- Ensure that calculated spatial resolution based on the ratio is rounded correctly.

# Proposal

The proposed text changes are provided below:

    - Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_pic_width_num_minus1 plus 1 and nnpfc_pic_width_denom_minus1 plus 1 specify the numerator and denominator for the ratio of output picture width relative to CroppedWidth. The value of (nnpfc_pic_width_num_minus1 + 1) ÷ (nnpfc_pic_width_denom_minus1 + 1) shall be in the range of 1 to 16, inclusive. When nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are not present, the value of nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are inferred to be equal to 0, respectively.

The value of the variable nnpfcOutputPicWidth, representing the width of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicWidth = Ceil( (CroppedWidth * (nnpfc_pic_width_num_minus1 + 1)) ÷ (nnpfc_pic_width_denom_minus1 + 1) )	(X)

nnpfc_pic_height_num_minus1 plus 1 and nnpfc_pic_height_denom_minus1 plus 1 specify the numerator and denominator for the ratio of output picture height relative to CroppedHeight. The value of (nnpfc_pic_height_num_minus1 + 1) ÷ (nnpfc_pic_height_denom_minus1 + 1) shall be in the range of 1 to 16, inclusive. When nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are not present, the value of nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are inferred to be equal to 0, respectively.

The value of the variable nnpfcOutputPicHeight, representing the height of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicHeight = Ceil( (CroppedHeight * (nnpfc_pic_height_num_minus1 + 1)) ÷ (nnpfc_pic_height_denom_minus1 + 1) )	(Y)

When nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, nnpfc_pic_height_denom_minus1 are present, at least one the following is true:

–	The value of (nnpfc_pic_width_num_minus1 + 1) ÷ (nnpfc_pic_width_denom_minus1 + 1) is not equal to 1.

–	The value of (nnpfc_pic_height_num_minus1 + 1) ÷ (nnpfc_pic_height_denom_minus1 + 1) is not equal to 1.

…

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples  &&  xY < nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples  && xY < nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

…

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, and outPatchCHeight are derived as follows:

outPatchWidth = (nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth	(86)

outPatchHeight = (nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight	(87)

horCScaling = SubWidthC / outSubWidthC		(88)

verCScaling = SubHeightC / outSubHeightC		(89)

outPatchCWidth = outPatchWidth * horCScaling		(90)

outPatchCHeight = outPatchHeight * verCScaling		(91)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfcOutputPicWidth nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfcOutputPicHeight nnpfc_pic_height_in_luma_samples * inpPatchHeight.

…

# Patent rights declaration(s)

Bytedance Inc., LG Electronics Inc., and Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).