| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0386-v1 |
| --- | --- |

| Title: | AHG9: Common text for the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Yong He Sean McCarthy | Email: | miska.hannuksela @nokia.com yonghe@qti.qualcomm.com sean.mccarthy@dolby.com |
| Source: | Nokia, Qualcomm Inc., Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

The BoG on NNPF and SEI processing order SEI messages during the current meeting of JVET requested the proponents of JVET-AD0059, JVET-AD0061, and JVET-AD0079 develop a common text for the SEI processing order to be included in the output of this JVET meeting (JVET-AC0362). This contribution proposes such text based on the JVET-AD0059 syntax as agreed in the BoG. The base text is JVET-AC2027.

# Proposal

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0, b = 0; b < payloadSize; i++, b += 4 ) { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(15) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| b += po_num_prefix_bytes[ i ] + 1 |  |
| } |  |
| po_sei_processing_order[ i ] | u(16) |
| } |  |
| } |  |

D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS.

When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal.

The SEI processing order SEI message can carry one or more SEI prefix indications of a particular payloadType. Each SEI prefix indication is a byte string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, and may be followed by bits that do not represent any complete syntax element of the SEI payload.

These SEI prefix indications should provide sufficient information to determine the specific processing order for SEI messages having the same value of payloadType but different preferred processing order.

po_sei_prefix_flag[ i ] equal to 1 specifies that po_num_prefix_bytes[ i ] is present. po_sei_prefix_flag[ i ] equal to 0 specifies that po_num_prefix_bytes[ i ] is not present.

po_sei_payload_type[ i ] specifies the payloadType value of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different non-negative integer values of m and n, the values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical unless both po_sei_prefix_flag[ m ] and po_sei_prefix_flag[ n ] are both equal to 1.

SeiProcessingOrderSeiList is set to consist of the payloadType values specified in clause D.2.1, except the values 137, 144, 147, 148, 179, 180, 200, 201, 208, and 213. The value of po_sei_prefix_flag[ i ] shall be equal to 0 when po_sei_payload_type[ i ] is not equal to any value among SeiProcessingOrderSeiList.

po_num_prefix_bytes[ i ], when present, specifies the number of bytes associated with the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, the value of po_num_prefix_bytes[ i ] is inferred to be equal to 0.

po_prefix_byte[ i ][ j ], when present, specifies the j-th byte value of the i-th SEI message.

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present, bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present, bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes[ n ] − 1, inclusive, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes [ i ], and bytes po_prefix_ byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes [ i ] − 1, inclusive, they shall have the same value of po_sei_processing_order[ i ].

# Open issues

- Should the SEI prefix be required to consist of a number of complete syntax elements only?

  - If yes, should the SEI prefix length be indicated in bytes or bits?

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).