| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0388-v1 |
| --- | --- |

| Title: | AHG9: Output flags in the NNPFA SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies Tampere, Finland Ye-Kui Wang Bytedance Inc. San Diego, CA, USA | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com yekui.wang@bytedance.com |
| Source: | Nokia, Bytedance Inc. |  |  |

_____________________________

# Abstract

The NNPF BOG recommended to disallow outputting a picture multiple times for any particular output order or output time instance. It is asserted that an NNPF combining picture rate upsampling and another purpose as activated by consecutive NNPFA SEI messages may result into a violation of the BOG agreement, or may require to achieve the same functionality through multiple NNPFs. This contribution proposes to add nnpfa_output_flag[ i ] syntax elements in the NNPFA SEI message to avoid violations of the BOG agreement and to avoid the need of multiple NNPFs.

# Proposal

## NNPFC SEI message

(Note: these changes are presented on top of JVET-AD0056 item 5 text.)

The variables numInputPics, specifying the number of pictures used as input for the NNPF, NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ] specifying the input picture index of the idx-th picture that is present in the output tensor of the NNPF and has a corresponding input picture, and numOutputPics, specifying the total number of pictures resulting from the NNPFpresent in the output tensor of the NNPF, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_output_flag[ i ] ) {
		InpIdx[ numOutputPics ] = i
		numOutputPics++
	}
NumInpPicsInOutputTensor = numOutputPics
if( ( nnpfc_purpose & 0x08 )  !=  0 )
	for( i = 0; i  <=  numInputPics − 2; i++ )		(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]

…

The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {	(93)
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

An NNPF-generated picture with index i contains sample arrays FilteredYPic[ i ], FilteredCbPic[ i ], and FilteredCrPic[ i ], when present, that are derived by Formula (93).

The NNPF process consists of the process defined by Formula (93) followed by outputting NNPF-generated pictures in their increasing index order, where all NNPF-generated pictures that were interpolated by the NNPF are output and those NNPF-generated pictures that correspond to any input pictures to the NNPF are output as specified in the semantics of the NNPFA SEI message.

## NNPFA SEI message

### Option 1

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| } |  |
| } |  |

nnpfa_num_output_entries specifies the number of nnpfa_output_flag[ i ] syntax elements. The value of nnpfa_num_output_entries shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive.

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive.

### Option 2

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_output_entries_present_flag | u(1) |
| if( nnpfa_output_entries_present_flag ) { |  |
| nnpfa_num_output_entries_minus1 | ue(v) |
| for( i = 0; i <= nnpfa_num_output_entries_minus1; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Nokia and Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).