| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0396-v1 |
| --- | --- |

| Title: | EE2-related: IBC MBVD for coding of camera captured content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Pavel Nikitin, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of IBC MBVD for camera captured content. The anchor is ECM-8.0 and the test is EE2-3.2. For both anchor and test, IBC is enabled. It reports all intra configuration results as follows:

AI : -0.05% (Y), -0.04% (U), -0.05 (V), 100% (EncT), 100% (DecT)

# Proposal

In ECM-8.0, IBC currently tailored to screen content coding. In JVET-AD0208, it proposes to adapt the IBC to coding of camera captured content. In JVET-AD0199, it proposes to improve the MBVD performance for screen content coding. In this contribution, it provides additional results of enabling the method for camera captured content.

# Simulation results

The simulation uses ECM-8.0 as anchor and the test is EE2-3.2. Both anchor and test have IBC enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.07% | 100% | 100% |
| Class A2 | -0.07% | -0.08% | -0.01% | 100% | 100% |
| Class B | -0.04% | 0.02% | 0.00% | 100% | 99% |
| Class C | -0.04% | -0.12% | -0.04% | 101% | 101% |
| Class E | -0.10% | -0.01% | -0.16% | 100% | 101% |
| Overall | -0.05% | -0.04% | -0.05% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.06% | 101% | 100% |
| Class F | -0.41% | -0.40% | -0.32% | 101% | 104% |
| Class TGM | -0.68% | -0.64% | -0.58% | 101% | 105% |

It suggests to study the method in the next EE2.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).