| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD1003-v2 |
| --- | --- |

| Title: | Coding-independent code points for video signal type identification (Draft 2 for 3rd edition) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Walt Husak Alexandros M. Tourapis | Email: Email: Email: | gary.sullivan@outlook.com wjh@dolby.com atourapis@apple.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for the specification of additional colour type identifiers for CICP (Rec. ITU-T H.273 | ISO/IEC 23091-2). Text modifications are provided for specification of code point identifiers for YCgCo-R colour representation with equal luma and chroma bit depths and for Draft SMPTE ST 2128. The new code points for YCgCo-R are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Draft SMPTE ST 2128 specifies a colour representation referred to as IPT-PQ-C2. The affected sections are provided relative to the corresponding basis text of ISO/IEC DIS 23091-2:202x (3rd Ed.). Change marks are included to show the changes since the January output (JVET-AC1008), which reflect a renumbering of the new MatrixCoefficients values and some minor refinements and corrections since the previous draft.

# Changes to the specification text of CICP in subclause 8.3

Replace subclause 8.3 and Table 5 with the following:

## 8.3	Matrix coefficients

Type: Unsigned integer, enumeration

Range: 0 – 255, plus associated flag

MatrixCoefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red, or X, Y, and Z primaries, as specified in Table 5 and the formulae below.

A flag, VideoFullRangeFlag, may be supplied with this code point (see below).

VideoFullRangeFlag specifies the scaling and offset values applied in association with the MatrixCoefficients. When not present or not specified, the value 0 for VideoFullRangeFlag would ordinarily be inferred as the default value for video imagery.

An 8-bit field should be adequate for representation of the MatrixCoefficients code point.

Certain values of MatrixCoefficients may be disallowed, depending on the application and the characteristics and format of the signal, e.g. with regard to combinations of the chroma format sampling structure and the values of BitDepthY and BitDepthC.

The interpretation of MatrixCoefficients is specified by the following formulae. ER, EG, and EB are defined as "linear-domain" real-valued signals based on the indicated colour primaries (see 8.1) before applying the transfer characteristics (see 8.2).

For purposes of the YZX representation when MatrixCoefficients is equal to 0, the symbols R, G, and B are substituted for X, Y, and Z, respectively, in the following descriptions of Formulae (12) to (14), Formulae (15) to (17), Formulae (25) to (27), and Formulae (31) to (33).

Nominal peak white is specified as having ER equal to 1, EG equal to 1, and EB equal to 1.

Nominal black is specified as having ER equal to 0, EG equal to 0, and EB equal to 0.

The application of the transfer characteristics function is denoted by ( x )′ for an argument x.

| — | If MatrixCoefficients is not equal to 14 or 15, the signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as per Formulae (12) to (14): |
| --- | --- |

E′R = (ER)′								(12)

E′G = (EG)′								(13)

E′B = (EB)′								(14)

|  | In this case, the range of E′R, E′G, and E′B is specified as follows: |  |  |
| --- | --- | --- | --- |
|  | — | If TransferCharacteristics is equal to 11 or 12, or TransferCharacteristics is equal to 13 and MatrixCoefficients is not equal to 0, E′R, E′G, and E′B are real numbers with values that have a larger range than the range of 0 to 1, inclusive, and their range is not specified in this document. |  |
|  | — | Otherwise, E′R, E′G, and E′B are real numbers in the range of 0 to 1. |  |
| — | Otherwise (MatrixCoefficients is equal to 14 or 15), the signals E′L, E′M, and E′S are determined by the following ordered steps: |  |  |
|  | a) | The "linear-domain" real-valued signals EL, EM, and ES are determined as follows: |  |
|  |  | — | If MatrixCoefficients is equal to 14, Formulae (15) to (17) apply: |

EL = ( 1 688 * ER + 2 146 * EG + 262 * EB ) ÷ 4 096	(15)

EM = ( 683 * ER + 2 951 * EG + 462 * EB ) ÷ 4 096	(16)

ES = ( 99 * ER + 309 * EG + 3 688 * EB ) ÷ 4 096	(17)

|  |  | — | Otherwise (MatrixCoefficients is equal to 15), Formulae (18) to (20) apply: |
| --- | --- | --- | --- |

EL = ( 1 747 * ER + 2 169 * EG + 180 * EB ) ÷ 4 096	(18)

EM = ( 673 * ER + 3 029 * EG + 394 * EB ) ÷ 4 096	(19)

ES = ( 50 * ER + 207 * EG + 3 839 * EB ) ÷ 4 096	(20)

|  | b) | The signals E′L, E′M, and E′S are determined by application of the transfer characteristics function as per Formulae (21) to (23): |
| --- | --- | --- |

E′L = (EL)′								(21)

E′M = (EM)′								(22)

E′S = (ES)′								(23)

When MatrixCoefficients is equal to 0, 8, 16, or 17, the variables BitDepthRGB and MaxValRGB are derived using the following ordered steps:

| a) | The variable BitDepthRGB is derived as follows: |  |
| --- | --- | --- |
|  | — | If MatrixCoefficients is equal to 0 or 8, the following applies: |

BitDepthRGB = BitDepthY					(24)

|  | — | Otherwise, if MatrixCoefficients is equal to 16, the following applies: |
| --- | --- | --- |

BitDepthRGB = BitDepthY + 2				(25)

|  | — | Otherwise (MatrixCoefficients is equal to 17), the following applies: |
| --- | --- | --- |

BitDepthRGB = BitDepthY + 1				(26)

| b) | The variable MaxValRGB is derived as follows: |
| --- | --- |

MaxValRGB = ( 1 << BitDepthRGB ) − 1		(27)

The interpretation of MatrixCoefficients is specified as follows.

| — | If VideoFullRangeFlag is equal to 0, the following applies: |  |
| --- | --- | --- |
|  | — | If MatrixCoefficients is equal to 0, 8, 16, or 17, Formulae (28) to (30) apply: |

R = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) * ( 219 * E′R + 16 ) )	(28)

G = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) * ( 219 * E′G + 16 ) )	(29)

B = Clip3( 0, MaxValRGB, ( 1 << ( BitDepthRGB − 8 ) ) * ( 219 * E′B + 16 ) )	(30)

|  | — | Otherwise, if MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, Formulae (31) to (33) apply: |
| --- | --- | --- |

Y = Clip1Y ( Round( ( 1 << (BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(31)

Cb = Clip1C ( Round( ( 1 << (BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(32)

Cr = Clip1C ( Round( ( 1 << (BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(33)

|  | — | Otherwise, if MatrixCoefficients is equal to 2, the interpretation of the MatrixCoefficients code point is unknown or is determined by the application. |
| --- | --- | --- |
|  | — | Otherwise (MatrixCoefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, or 17), the interpretation of the MatrixCoefficients code point is reserved for future definition by ITU-T ¦ ISO/IEC. |
| — | Otherwise (VideoFullRangeFlag is equal to 1), the following formulae apply: |  |
|  | — | If MatrixCoefficients is equal to 0, 8, 16, or 17, Formulae (34) to (36) apply: |

R = Clip3( 0, MaxValRGB, MaxValRGB * E′R )	(34)

G = Clip3( 0, MaxValRGB, MaxValRGB * E′G )	(35)

B = Clip3( 0, MaxValRGB, MaxValRGB * E′B )	(36)

|  | — | Otherwise, if MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, Formulae (37) to (39) apply: |
| --- | --- | --- |

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(37)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PB + ( 1 << ( BitDepthC − 1 ) ) ) )	(38)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PR + ( 1 << ( BitDepthC − 1 ) ) ) )	(39)

|  | — | Otherwise, if MatrixCoefficients is equal to 2, the interpretation of the MatrixCoefficients code point is unknown or is determined by the application. |
| --- | --- | --- |
|  | — | Otherwise (MatrixCoefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, or 17), the interpretation of the MatrixCoefficients code point is reserved for future definition by ITU-T ¦ ISO/IEC. |

When MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, or 13, the constants KR and KB are specified as follows:

| — | If MatrixCoefficients is not equal to 12 or 13, the constants KR and KB are specified in Table 5. |
| --- | --- |
| — | Otherwise (MatrixCoefficients is equal to 12 or 13), the constants KR and KB are computed as follows, using the chromaticity coordinates (xR, yR), (xG, yG), (xB, yB), and (xW, yW) specified by Table 3 for the ColourPrimaries code point for the red, green, blue, and white colour primaries, respectively: |

(40)

(41)

|  | where the values of zR, zG, zB, and zW, are given by Formulae (42) to (45): |
| --- | --- |

zR = 1 − ( xR + yR )						(42)

zG = 1 − ( xG + yG )						(43)

zB = 1 − ( xB + yB )						(44)

zW = 1 − ( xW + yW )						(45)

The variables E′Y, E′PB, and E′PR (for MatrixCoefficients not equal to 0, 8, 16, or 17) or Y, Cb, and Cr (for MatrixCoefficients equal to 0, 8, 16, or 17) are specified as follows.

| — | If MatrixCoefficients is not equal to 0, 8, 10, 11, 13, 14, 15, 16, or 17, Formulae (46) to (48) apply: |
| --- | --- |

E′Y = KR * E′R + ( 1− KR − KB ) * E′G + KB * E′B	(46)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )			(47)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )			(48)

|  | E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When TransferCharacteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1. When TransferCharacteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5. When TransferCharacteristics is equal to 11 (IEC 61966-2-4), or 12 (Rec. ITU-R BT.1361-0 extended colour gamut system), E′Y, E′PB, and E′PR are real numbers with a larger range not specified in this document. |
| --- | --- |
| — | Otherwise, if MatrixCoefficients is equal to 0, Formulae (49) to (51) apply: |

Y = Round( G )							(49)

Cb = Round( B )							(50)

Cr = Round( R )							(51)

| — | Otherwise, if MatrixCoefficients is equal to 8 and BitDepthC is equal to BitDepthY, Formulae (52) to (54) apply: |
| --- | --- |

Y = Round( 0.5 * G + 0.25 * ( R + B ) )		(52)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + (1 << ( BitDepthC − 1 ) )	(53)

Cr = Round( 0.5 * (R − B ) ) + (1 << ( BitDepthC − 1 ) )	(54)

|  | For purposes of the YCgCo nomenclature used in Table 5, Cb and Cr of Formulae (53) and (54) may be referred to as Cg and Co, respectively. An appropriate inverse conversion for Formulae (52) to (54) is given in Formulae (55) to (58): |
| --- | --- |

t = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )	(55)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(56)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(57)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(58)

|  | — | Otherwise, if MatrixCoefficients is equal to 8, 16, or 17, Formulae (59) to (62) apply: |
| --- | --- | --- |

Cr = Round( R ) − Round( B ) + (1 << ( BitDepthC − 1 ) )	(59)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(60)

Cb = Round( G ) − t + (1 << ( BitDepthC − 1 ) )	(61)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(62)

|  | For purposes of the YCgCo nomenclature used in Table 5, Cb and Cr of Formulae (58) and (56) may be referred to as Cg and Co, respectively. An appropriate inverse conversion for Formulae (59) to (62) is given in Formulae (63) to (66): |
| --- | --- |

t = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(63)

G = Clip3( 0, MaxValRGB, t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(64)

B = Clip3( 0, MaxValRGB, t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(65)

R = Clip3( 0, MaxValRGB, B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(66)

| — | Otherwise, if MatrixCoefficients is equal to 10 or 13, the signal E′Y is determined by application of the transfer characteristics function as given in Formulae (67) and (68), and Formulae (69) to (76) apply for specification of the signals E′PB and E′PR: |
| --- | --- |

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB	(67)

E′Y = ( EY )′								(68)

|  | In this case, EY is defined from the "linear-domain" signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are real values with the value 0 associated with nominal black and the value 1 associated with nominal white. |
| --- | --- |

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB ) for −NB <= E′B − E′Y <= 0	(69)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB ) for 0 < E′B − E′Y <= PB	(70)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR ) for −NR <= E′R − E′Y <= 0	(71)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR ) for 0 < E′R − E′Y <= PR	(72)

|  | where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as given in Formulae (73) to (76): |
| --- | --- |

NB = ( 1 − KB )′							(73)

PB = 1 − ( KB )′							(74)

NR = ( 1 − KR )′							(75)

PR = 1 − ( KR )′							(76)

| — | Otherwise, if MatrixCoefficients is equal to 11, Formulae (77) to (79) apply: |
| --- | --- |

E′Y = E′G									(77)

E′PB = ( 0.986 566 * E′B − E′Y ) ÷ 2.0		(78)

E′PR = ( 0.991 902 * E′Y ) ÷ 2.0				(79)

|  | In this case, for purposes of the Y′D′ZD′X nomenclature used in Table 5, E′PB may be referred to as D′Z and E′PR may be referred to as D′X. |  |
| --- | --- | --- |
| — | Otherwise, if MatrixCoefficients is equal to 14, the following applies: |  |
|  | — | If TransferCharacteristics is not equal to 18, Formulae (80) to (82) apply: |

E′Y = 0.5 * ( E′L + E′M )					(80)

E′PB = ( 6 610 * E′L − 13 613 * E′M + 7 003 * E′S ) ÷ 4 096	(81)

E′PR = ( 17 933 * E′L − 17 390 * E′M − 543 * E′S ) ÷ 4 096	(82)

|  | — | Otherwise, Formulae (83) to (85) apply: |
| --- | --- | --- |

E′Y = 0.5 * ( E′L + E′M )					(83)

E′PB = ( 3 625 * E′L − 7 465 * E′M + 3 840 * E′S ) ÷ 4 096	(84)

E′PR = ( 9 500 * E′L − 9 212 * E′M − 288 * E′S ) ÷ 4 096	(85)

|  | In these cases, for purposes of the ICTCP nomenclature used in Table 5, E′Y, E′PB, and E′PR of Formulae (80), (81), and (82) or Formulae (83), (84), and (85) may be referred to as I, CT, and CP, respectively. Formulae (80), (81), and (82) were designed specifically for use with TransferCharacteristics equal to 16 (PQ), and Formulae (83), (84), and (85) were designed specifically for use with TransferCharacteristics equal to 18 (HLG). |  |
| --- | --- | --- |
| — | Otherwise (MatrixCoefficients is equal to 15), the following applies: |

E′Y = ( 1 638 * E′L + 1 638 * E′M + 820 * E′S ) ÷ 4 096	(86)

E′PB = ( 18 248 * E′L − 19 870 * E′M + 1 622 * E′S ) ÷ 4 096	(87)

E′PR = ( 3 300 * E′L + 1 463 * E′M − 4 763 * E′S ) ÷ 4 096	(88)

|  | In this case, for purposes of the IPT nomenclature used in Table E.5, E′Y, E′PB, and E′PR of Formulae (86), (87), and (88) may be referred to as I, P, and T, respectively. |
| --- | --- |

# Table 5 — Interpretation of the matrix coefficients (MatrixCoefficients) value

| Value | Matrix coefficients | Informative remarks |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ); IEC 61966-2-1 sRGB Society of Motion Picture and Television Engineers ST 428-1 (2019) See Formulae (49) to (51) |
| 1 | KR = 0.2126; KB = 0.0722 | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B See Formulae (46) to (48) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) See Formulae (46) to (48) |
| 5 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.470-6 System B, G (historical) Rec. ITU-R BT.601-7 625 Rec. ITU-R BT.1358-0 625 (historical) Rec. ITU-R BT.1700-0 625 PAL and 625 SECAM IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) See Formulae (46) to (48) |
| 6 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.601-7 525 Rec. ITU-R BT.1358-1 525 or 625 (historical) Rec. ITU-R BT.1700-0 NTSC Society of Motion Picture and Television Engineers ST 170 (2004) (functionally the same as the value 5) See Formulae (46) to (48) |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers ST 240 (1999) See Formulae (46) to (48) |
| 8 | YCgCo or YCgCo-R | See Formulae (52) to (58) for YCgCo (when BitDepthC is equal to BitDepthY) See Formulae (59) to (66) for YCgCo-R (when BitDepthC is equal to BitDepthY + 1) |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 (non-constant luminance) Rec. ITU-R BT.2100-2 Y′CbCr See Formulae (46) to (48) |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 (constant luminance) See Formulae (67) to (76) |
| 11 | Y′D′ZD′X | Society of Motion Picture and Television Engineers ST 2085 (2015) See Formulae (77) to (79) |
| 12 | See Formulae (40) to (45) | Chromaticity-derived non-constant luminance system See Formulae (46) to (48) |
| 13 | See Formulae (40) to (45) | Chromaticity-derived constant luminance system See Formulae (67) to (76) |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Formulae (80) to (82) for TransferCharacteristics value 16 (PQ) See Formulae (83) to (85) for TransferCharacteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Formulae (86) to (88) |
| 16 | YCgCo-Re | See Formulae (59) to (66) |
| 17 | YCgCo-Ro | See Formulae (59) to (66) |
| 18–255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

NOTE	In a previous edition of this document, the IEC 61966-2-1 sYCC representation was identified as corresponding to MatrixCoefficients equal to 1. Closer study later determined that this representation should correspond to MatrixCoefficients equal to 5 instead (which is functionally the same as the value 6). This document was therefore revised to correct the error.