| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0023_v2 |
| --- | --- |

| Title: | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang and Z. Xie | Tel: Email: | elena.alshina@huawei.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the JVET-AD and JVET-AE meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences have been conducted during this meeting circle.

# Introduction

In total seventeen test been planned for this EE1 round, but only seven were completed. Major reason for the delay is unified filter architecture for high operation point (HOP) and unified training procedure (both greed at JVET-AD meeting and documented in the JVET-AD0380 and JVET-AD2023) implemented from scratch and trained in parallel by multiple companies with careful cross-check of all intermediate steps and performance tracking. In addition, many unfinished EE tests are supposed to be conducted on top of the HOP model, which is not fully ready yet.

EE1 conducted two teleconferences jointly with AhG11 and AhG14 to discuss training of HOP unified filter (EE1-0). The teleconferences were held on June 14 and 28, 2023 and attended by approximately 20 participants.

In JVET-AD meeting low complexity operation point NN-based filter was agreed to be default configuration of NNVC-5.0. Also it was agreed to enable NN-Intra by default in NNVC-5.0 anchor. EE1 tests participant targeting for adoption to Common SW base NNVC are requested to report performance relatively to NNVC-5.0 default configuration.

In order to track NNVC work progress results relatively to VVC (NNVC-5.0 with all tools disabled) are shown in Figure 1.

Figure 1 Complexity performance trade-off of each NNVC tool in common SW base (old filters shown for the last time).

# Unified Filter Training

## EE1-0 Agreed unified filter design for High Operation Point

JVET-AE0191 AhG11: EE1-0 High Operation Point model F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei)

Figure 2 Unified filter architecture (HOP).

Unified filter architecture (EE1.0) was documented in JVET-AD2023 and is shown in Figure 2. Computational complexity: 477 kMAC/pxl; Total Number of parameters: 1.45 M;

Unified training procedure of EE1.0 was also documented in JVET-AD2023 and consisted of following stages

- Training Model stage I (Intra model)

  - All Intra coding of DIV2K data coding with vanilla VTM (NN Tools off)

  - Training data dumping and DIV2K unified database creation size of 2.6TB

  - Model stage I training from scratch for Intra data (40 epochs, 10 days)

- Training Model stage II (Intra & Inter model)

  - Random Access coding of BVI and TVD data with NNVC5.0 (NN Tools off, HOP Stage I model enabled for Intra Slices)

  - Training data dumping, BVI and TVD data unified databases creation size of 1.5TB

  - Model stage II training with DIV2K, BVI and TVD databases (20 epochs, 10 days)

  - Testing (5 days)

- Training stage III (Intra & Inter model)

  - Random Access coding of BVI and TVD data with NNVC5.0 (NN Tools off, HOP Stage II model enabled for Intra and B slices)

  - Training data dumping, BVI and TVD data unified databases creation size of 1.5TB

  - Model stage III training with DIV2K, BVI and TVD databases (20 epochs, 10 days)

It must be noted that collaborative development of HOP filter (EE1-0) allows achieving significant improvement for complexity-performance trade-off of NN-based filter at high operation point.

| HOP filters: past and future | Parameters, M | kMAC /pixel | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | Y | Cb | Cr |
| EE1-0 (Stage I) | 1.5 | 477 | -4.2% | -17.4% | -16.2% | -7.8% | -19.2% | -20.2% |
| EE1-0 (Stage II) | 1.5 | 477 | -9.5% | -22.4% | -22.1% | -7.8% | -18.8% | -20.0% |
| filter set#0 (fully trained) | 1.9 | 615 | -9.8% | -21.1% | -20.5% | -7.5% | -16.8% | -17.4% |
| filter set#1 (fully trained) | 3.1 | 673 | -9.6% | -20.9% | -21.5% | -7.4% | -18.3% | -19.9% |

Only after two (out of three) stages training unified HOP filter (EE1-0) achieves almost the same performance with ste#0/set#1 filters in NNVC 4.0 with more than 20% lower computational complexity. Roughly 1% gain is expected after completion Stage II training (expected to finish during JVET-AE meeting).

Filter architecture EE1.0 and unified dataset and training process are recommended for adoption to the next version of NNVC common SW base.

New finding during joint training of HOP are recommended to be presented with details (JVET-AE0191).

# Unified Filter Architectural changes

## EE1-1.2 Complexity-performance tradeoff of decomposition

JVET-AE0164 EE1-1.2 Complexity-performance tradeoff of decomposition D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

Figure 3 Back Bone Block: (above) EE1-0 design, all 24 BBB sare the same; (below) proposed design: every odd BBBs are T1, even BBBs are T2.

Proposed modification for unified filter architecture is shown in Figure 3. Type 1 and Type 2 Back Bone Blocks feature reduced complexity, comparing to the anchor BBB, and are alternating (T1 and T2) in order. The model was trained following HOP training procedure and test set. Results for Stage1 have been provided, training for latter stages of HOP is ongoing, results to be provided once available.

Computational complexity: 426 kMAC/pxl (11% reduction compared to EE1-0); Total Number of parameters: 1.3M;

In all Intra cfg performance difference compare to EE1-0 (after same stage of training) is 0.0% (Y), 0.2%(Cb), 0.1%(Cr).

Recommendation: review in the BOG to plan study in EE1, e.g. provide results with 3 stages training, perform training cross-check.

## EE1-1.5 Optimization for complexity-performance trade-off of network

JVET-AE0160 EE1-1.5: Optimization for complexity-performance trade-off of HOP network R. Chang, L. Wang, X. Xu, S. Liu (Tencent)

Proposed modification for unified filter architecture is shown in Figure 4.

Figure 4 Proposed split Luma – Chroma filter architecture. Key changes: long-skip connection, Back Bone Block are changed to Residual Block (skip connection added).

Three variants of residual block are proposed.  They are shown on Figure 5.

Figure 5 Proposed split variants of  Residual Block.

Based on filter0 training scripts and dataset, retrain the EE1-0 residual block and two improved residual blocks in Test 1, and retrain the EE1-0 architecture and split architecture in Test 2. Compared to the EE1-0 residual block, 11% kMAC/pixel reduction with only 0.2% luma gain loss for AI. Compared to the EE1-0 architecture, 34% kMAC/pixel reduction with only 0.3% luma gain loss for AI.

| Test | Parameters, M | kMAC /pixel | Random Access vs NNVC-5.0 anchor |  |  | All Intra vs NNVC-5.0 anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-1.5.1-subtest 1 | 1.4 | 461 |  |  |  | -3.1% | -12.4% | -13.5% |
| NNIntra & EE1-1.5.1-subtest 2 | 1.2 | 402 | -4.8% | -14.9% | -14.1% | -2.8% | -12.0% | -13.0% |
| NNIntra & EE1-1.5.1-subtest 3 | 1.3 | 408 | -5.0% | -14.2% | -13.8% | -2.9% | -11.8% | -13.0% |
| NNIntra & EE1-1.5.2-subtest 1 | 1.5 | 480 | -3.6% | -12.1% | -13.7% | -2.3% | -11.4% | -12.2% |
| NNIntra & EE1-1.5.2-subtest 2 | 0.9 | 316 |  |  |  | -2.0% | -10.6% | -11.5% |

| Test | Subtest | Architecture | Back Bone Block | Training strategy | Training set | Interface | kMAC/pxl |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EE1-0 (HOP) |  | HOP | HOP resblock | HOP | HOP | HOP | 477 |
| EE1-1.5.1 | subtest1 (*) | Filter set#0 | Same as HOP resblock | Filter set#0 | Filter set#0 | Same as HOP | 461 |
|  | subtest2 |  | HOP resblock with depth-wise separable |  |  |  | 408 |
|  | subtest3 |  | HOP resblock with group convolution |  |  |  | 402 |
| EE1-1.5.2 | subtest1 (**) | Same as HOP | Same as HOP resblock |  |  |  | 480 |
|  | subtest2 | HOP with Split |  |  |  |  | 316 |

Recommendation: review in the BOG to plan study in EE1, e.g. utilizing HOP unified dataset and training process.

# Training set modification

No specific study was conducted in this category. Test EE1-1.5 uses different training data and training strategy.

# Unified filter usage aspects

No tests are possible at this point, since HOP filter is not yet available.

Recommendation to conduct scheduled tests in the next EE round.

# Improvement for Low complexity NN-filter

## EE1-4.1 Neural-network loop filters with further complexity reduction

JVET-AE0067 EE1-4.1: Neural-network loop filters with further complexity reduction J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby)

Filter architecture of low operation point (LOP) filter in NNVC-5.0 is shown in Figure 6. Computational complexity is 16 kMAX/pxl. Number of parameters 0.21 M.

Figure 6 Low Operation Point (LOP)   filter in NNVC-5.0.

Proposed modification for LOP are  shown in Figure 7. . Parameters of the architecture are following: number of input feature sets M=64, K=24, R=24. Computational complexity is 10 kMAX/pxl. Number of parameters 0.13 M.

Figure 7 Proposed modification for LOP filter.

Complexity reduced drastically, performance degradation is 0.3...0.4% (performance gain of LOP filter is ~5%). Only float point model is provided. Performance test results is summarized in  Table 1 .

Table 1 Performance of low-complexity filter EE1-4.1 in comparison with LOP filter in NNVC-5.0

| Test | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-4.1 | -6.2% | -10.9% | -9.4% | -7.5% | -11.0% | -10.8% |
| Anchor(NNIntra & LOP NNVC-5.0) | -6.6% | -7.7% | -7.3% | -7.8% | -8.6% | -8.9% |
| NNIntra | -1.8% | -0.5% | -0.9% | -3.6% | -3.2% | -3.3% |
| LOP NNVC5.0 | -5.2% | -7.5% | -6.6% | -4.6% | -5.7% | -5.7% |

Recommendation: to continue study in EE1, provide quantized model and cross-check training.

## EE1-4.4 Low complexity NN filter with design elements of UF and EE1-1.2 and EE1-1.3

JVET-AE0165 EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3  Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) (Dolby)

In this contribution, Unified Filter Architecture of HOP was adjusted to meet complexity constraints of LOP. This was achieved by reducing complexity of the headblock, backbone, and tailblock and utilizing the CP4 decomposition in the backbone blocks. The model was trained End-to-End following HOP training procedure and test set. Results for Stage1 have been provided, training for latter stages of HOP is ongoing, results to be provided once available.

Training for latter stages of HOP is ongoing, results to be provided once available. Proposed filter, comparing to the EE1.0 anchor is shown in Figure 8 (above: EE1-0 filter, below: proposed low complexity filter).

Achieved model complexity in kMAC/pixel: 14.8 (frame-wise); 16.7 (block-wise of block 256x256); 18.7 (block-wise of block 128x128), total amount of parameters is 0.05 M (compared to 1.45 for HOP).

Figure 8 Low complexity filter following concept of unified HOP.

Performance test results are available for all intra configuration only, shown in Table 2. Training procedure follows EE1 recommendation, so results easily comparable with the rest of EE1 tests.

Table 2 Performance of low-complexity filter EE1-4.4 in comparison with LOP filter in NNVC-5.0

| Test | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-4.4 |  |  |  | -7.7% | -13.6% | -14.3% |
| Anchor(NNIntra & LOP NNVC-5.0) | -6.6% | -7.7% | -7.3% | -7.8% | -8.6% | -8.9% |

Channels number in some point of processing pipeline have been selected to match complexity/number of channels of the LOP anchor. Due to this, they are multiple of 16 (even not multiple of 8), which could be sub-optimal for some implementations

Recommendation: review in the BOG to plan study in EE1, e.g for architecture optimization, provide results with HOP 3 stages training, perform training cross-check.

# NN-based Inter

## EE1-5.1 NN-based inter prediction with lower complexity

JVET-AE0112 EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)

The framework of the proposed DRF method is depicted in Figure 9. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Figure 9 position of NN-Inter in video codec (test EE1-5)

Changes of design compared to previous EE1 tests: removal of PFRB module and ResBlocks, IFRNet small IFRNet, SADL implementation (32 float).

Computational complexity is 504 kMAC/pxl (was 986 before), total number of parameters is 14.9 M (was 19.3 M).

Performance in random access configuration test (compared to previous design) is shown Table 3.

Table 3 Performance of reduced complexity NN-Inter (EE1-5) compared to previously studied design.

Performance after complexity reduction (EE1-5): kMAC/xpl 504, Number of parameters 14.9M.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-5.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -3.2% | -7.3% | -6.6% | -3.9% | -9.3% | -8.4% | 700% | 1030222% |
| Class C | -4.5% | -7.0% | -7.2% | -6.0% | -9.7% | -10.0% | 441% | 761956% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | -5.5% | -5.9% | -6.5% | -5.7% | -10.4% | -10.1% | 751% | 1300136% |

Performance demonstrated in JVET-AD0162: kMAC/xpl 989, Number of parameters 19.3M.

|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.7% | -8.0% | -8.1% | -4.1% | -9.4% | -9.0% | 157% | 65726% |
| Class A2 | -5.4% | -11.7% | -9.4% | -6.2% | -11.5% | -9.2% | 147% | 60992% |
| Class B | -4.1% | -10.3% | -9.8% | -4.7% | -11.2% | -10.8% | 159% | 79960% |
| Class C | -5.7% | -13.4% | -13.6% | -7.3% | -13.5% | -14.3% | 123% | 47989% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.7% | -11.0% | -10.4% | -5.6% | -11.5% | -11.0% | 146% | 63562% |
| Class D | -7.2% | -14.2% | -15.6% | -6.8% | -14.8% | -15.2% | 113% | 32008% |

Reduction of computational complexity roughly twice leads to ~1 % compression performance degradation.

# NN-based intra

## EE1-6.1 Neural network-based intra prediction with reduced complexity

JVET-AE0144  EE1-6.1: neural network-based intra prediction with reduced complexity 
T. Dumas, F. Galpin, P. Bordes (Interdigital)

NN-Intra is enabled by default in NNVC-5.0 Design of NN-Intra is show in Figure 10.

Note that all the mentioned layers are fully-connected. No convolutional layers exist in the presented neural networks. Changes compared to NN-Intra in common SW base (NNVC0) are summarized in Table 4. The percentage of non-zero weights in the layer returning repIdx becomes 10% for the neural network predicting blocks of size 4×4, and 50% for the other six neural networks.

Computational complexity is different for different block sizes, the worst case is 4.8 kMAC/pixel (7.8 in NNVC). Total number of parameters is 1.3 M (1.5 in NNVC).

In AI, full test results show ~0.2% BD-rate performance degradation compared to NN-intra in NNVC-5.0. In RA, partial test results (full test results will be available by start of the meeting) show ~0.1% (RA) BD-rate performance degradation compared to NN-intra in NNVC-5.0.

Figure 10 : prediction of the current w×h block from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the neural network-based intra prediction mode. Here, w=8 and h=4. The dimensions of the context of reference samples, “preprocessing”, “postprocessing”, repIdx, grpIdx1, grpIdx2 are explained in NNVC description document.

Table 4 architecture policy for each neural network in common SW base NNVC and proposed solution (EE1-6).

| architecture | NN-Intra in common SW base | EE1-6 |
| --- | --- | --- |
| Number of hidden layers | For 16x16, 3. For the other six neural networks, 2. | Same. |
| Number of neurons per hidden layer | 1216. | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four neural networks, 1216. |
| Biases | For all layers. | Only for the layer returning repIdx and the layer returning grpIdx1 and grpIdx2. |

Recommendation: looking at Bytedance’s training cross-check to decide.

# Recommendation:

- Present unified HOP filter (JVET-AE0191) in details

- Adopt HOP filter from JVET-AE0191 (after training is finished) to NNVC-6.0.

- Continue investigating pending EE tests of this round (supposed to be conducted on top of the HOP model) in the next round of EE.

- Conduct BoG to review input contributions and plan EE1 tests, identify potential for joint training in the next EE1 round (could be two designs for LOP and HOP).

- Conduct training cross-check for low-complexity NN-Intra (EE1-6)