| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0051-v2 |
| --- | --- |

| Title: | AHG9: On persistent NNPF activation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that when an NNPF outputs multiple filtered input pictures or interpolates pictures between multiple pairs of input pictures, the NNPF cannot be persistently activated with nnpfa_persistence_flag equal to 1. This is because the persistent activation invokes the NNPF for each picture in output order, which would cause overlapping NNPF output pictures resulting from subsequent NNPF activations, which is disallowed.

It is proposed to enable persistent activation of an NNPF that outputs multiple filtered input pictures or interpolates pictures between multiple pairs of input pictures as follows:

- In the NNPFA SEI semantics of the VSEI text: Derivation of the pictures for which the NNPF is activated when nnpfa_persistence_flag is equal to 1 at the smallest interval such that the same picture is not output by the NNPF process more than once in subsequent NNPF activations and interpolated pictures are not generated between the same pair of input pictures in subsequent NNPF activations.

- In the NNPF SEI use of the VVC text: The repeated inference to generate interpolated pictures up to the end of the bitstream is performed at the smallest interval that does not cause interpolated pictures to be generated between the same pair of input pictures in subsequent NNPF inferences.

# Introduction

nnpfa_persistence_flag equal to 1 specifies that the NNPF is applied persistently for each picture in the current layer in output order as follows:

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

When the NNPF process of an NNPF outputs multiple filtered input pictures and/or the NNPF process of a picture rate upsampling NNPF takes more than two pictures as input and outputs interpolated pictures between two or more pairs of input pictures, nnpfa_persistence_flag cannot be set equal to 1, because the following constraint in subclause 8.28.1.1 would be violated:

Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance.

It is asserted that NNPFs that would violate the constraint cited above are not uncommon and a design principle for the NNPFA SEI message has been to enable persistent activation of the NNPF. Consequently, it is proposed to enable persistent activation of an NNPF for non-consecutive pictures in output order. The proposal can be summarized as follows:

- In the NNPFA SEI semantics of the VSEI text: Derivation of the pictures for which the NNPF is activated when nnpfa_persistence_flag is equal to 1 in a manner that the same picture is not output by the NNPF process more than once in subsequent NNPF activations and interpolated pictures are not generated between the same pair of input pictures in subsequent NNPF activations.

- In the NNPF SEI use of the VVC text: The repeated inference to generate interpolated pictures up to the end of the bitstream is performed at the smallest interval that does not cause interpolated pictures to be generated between the same pair of input pictures in subsequent NNPF inferences.

# Proposed VSEI changes

## Overview

In this proposal, the variable NnpfaActivationDelta, specifying that the target NNPF is activated for every NnpfaActivationDelta-th picture of the current layer in output order relative to the current picture, is derived. The value of NnpfaActivationDelta is such that the same picture is not output by the NNPF process more than once in subsequent NNPF activations and interpolated pictures are not generated between the same pair of input pictures in subsequent NNPF activations.

## Proposed VSEI text changes

nnpfa_persistence_flag specifies the persistence of the target NNPF for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target NNPF may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order as specified below.

[Ed. (MH): The semantics of nnpfa_num_output_entries and nnpfa_output_flag[ i ] have been moved up.]

nnpfa_num_output_entries specifies the number of nnpfa_output_flag[ i ] syntax elements present in the NNPFA SEI message. The value of nnpfa_num_output_entries shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive.

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive.

When nnpfa_persistence_flag is equal to 1, the variable NnpfaActivationDelta, specifying that the target NNPF is activated for every NnpfaActivationDelta-th picture of the current layer in output order relative to the current picture, is derived as follows:

–	If NumInpPicsInOutputTensor is greater than 0 and nnpfa_output_flag[ i ] is equal to 1 for at least one value of i in the range of 0 to NumInpPicsInOutputTensor − 1, the following applies

–	Let lastFiltered be equal to InpIdx[ i ] for the greatest value of i in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, such that nnpfa_output_flag[ i ] is equal to 1.

–	Let firstFiltered be equal to InpIdx[ i ] for the smallest value of i in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, such that nnpfa_output_flag[ i ] is equal to 1.

–	filteringActivationDelta is set equal to lastFiltered − firstFiltered + 1

–	Otherwise, filteringActivationDelta is set equal to 1.

–	If nnpfc_purpose & 0x08 is greater than 0 (i.e., if nnpfc_purpose indicates picture rate upsampling), the following applies:

–	Let firstInterpolated be equal to the smallest value of i in the range of 0 to numInputPics − 2, inclusive, such that nnpfc_interpolated_pics[ i ] is equal to 1.

–	Let lastInterpolated be equal to the greatest value of i in the range of 0 to numInputPics − 2, inclusive, such that nnpfc_interpolated_pics[ i ] is equal to 1.

–	interpolationActivationDelta is set equal to lastInterpolated − firstInterpolated + 1.

–	Otherwise, interpolationActivationDelta is set equal to 1.

–	NnpfaActivationDelta is set equal to Max( filteringActivationDelta, interpolationActivationDelta ).

The list NnpfaTargetPictures, which includes the pictures for which the target NNPF is activated by the current NNPFA SEI message in output order, is derived as follows:

–	If nnpfa_persistence_flag is equal to 0, NnpfaTargetPictures is set to include the current picture only.

–	Otherwise (nnpfa_persistence_flag is equal to 1), NnpfaTargetPictures is set to include the current picture and every NnpfaActivationDelta-th picture, if any, in the current layer following the current picture in output order until the bitstream ends or up to but excluding the picture for which one or more of the following conditions are true, whichever is the earliest in output order:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

EXAMPLE – When NnpfaActivationDelta is equal to 2, the NnpfaTargetPictures contains every other picture in output order until the persistence of this NNPFA SEI message ends as specified above.

If the last picture in NnpfaTargetPictures is followed in output order by a CLVS of the current layer or a picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1, the variable NnpfaPersistenceUntilEOBFlag is set equal to 0. Otherwise, the variable NnpfaPersistenceUntilEOBFlag is set equal to 1.

Let the nnpfcTargetPictures be the set of pictures to which the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the current NNPFA SEI message in decoding order pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in NnpfaTargetPictures shall also be included in nnpfcTargetPictures.

# Proposed VVC changes

## Overview

The VVC text for the use of NNPF SEI messages includes repeated inference at the end of a bitstream to generate interpolated pictures up to the end of the bitstream. It is asserted to be reasonable to generalize the repeated inference to not only apply to persistent activation for each picture. Thus, the following changes are proposed

- to perform the repeated inference at intervals of NnpfaActivationDelta, and

- to take into account that when an NNPFA SEI message persists until the end of a bitstream, the last picture for which the picture rate upsampling NNPF is activated might not be the last picture in the bitstream, because the activation happens at intervals of NnpfaActivationDelta.

Note the rationale for the following proposed change has been given in JVAT-AE0049 item 2:

–	If pictureRateUpsamplingFlag is equal to 1 nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling), …

## Proposed VVC text changes

The variable numInferences is derived as follows:

–	If pictureRateUpsamplingFlag is equal to 1 nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling), the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId currPic is the last picture among NnpfaTargetPictures, and NnpfaPersistenceUntilEOBFlag is equal to 1, the variable numPostRoll is set equal to the smallest value of i such that nnpfc_interpolated_pics[ i ] is greater than 0, the variable numFollPicUntilEOB is set equal to the count of pictures that follow currPic in output order in the current layer within the same bitstream, and the variable numInferences is set equal to 1 + ( numPostRoll + numFollPicUntilEOB ) / NnpfaActivationDelta.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures within the bitstream, respectively, are specified as follows:

–	When j is greater than 0, the variable latestPic is set to be currPic and for each value of k in the range of 0 to j * NnpfaActivationDelta − 1, inclusive, the following applies in decreasing order of k:

–	If there is a picture that follows latestPic in output order in the current layer within the same bitstream, inputPic[ k ] is set to be the first picture in output order that follows latestPic in output order in the current layer within the same bitstream and subsequently latestPic is set equal to inputPic[ k ] and inputPicPresentFlag[ k ] is set equal to 1.

–	Otherwise, inputPic[ k ] is set to be currPic latestPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j * NnpfaActivationDelta ], is set equal to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j * NnpfaActivationDelta  + 1 to numInputPics − j − 1, inclusive, in increasing order of i:

–	If pictureRateUpsamplingFlag is equal to 1, currPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if pictureRateUpsamplingFlag is equal to 0 and there is a picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if currPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).