| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0054-v1 |
| --- | --- |

| Title: | AHG2/AHG9: Editorial improvements of annotated regions SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoko Aono Tomohiro Ikai | Tel: Email: | chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects and has been introduced in VSEI, HEVC and AVC. After reviewing the current their texts, several editorial issues have been found. They should be improved when revising VSEI, HEVC and AVC. This contribution proposes revised texts for each VSEI, HEVC and AVC.

Problem statement

The following problems exist in the texts of the annotated regions SEI message.

- The “ar_” prefix of several syntax elements is lacking.

- The offset variables for the conformance window are used in the semantics, but they are not needed because the input picture of the SEI message has been cropped.

- The name of the chroma format indicator differs between VSEI and HEVC/AVC.

- In the case of VVC, the offset variables of the conformance window are defined on PPS, but in the case of HEVC/AVC, they are defined on SPS.

- Many unknown characters are inserted in the test of Word versions of HEVC and AVC.

In the next section, we propose revised texts for each VSEI [1], HEVC [2] and AVC [3].

Proposal

VSEI

- 8.18.2	Annotated regions SEI message semantics

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects.

Use of this SEI message requires the definition of the following variables:

- A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- A conformance cropping window left offset, ConfWinLeftOffset

- A conformance cropping window top offset, ConfWinTopOffset

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

ar_object_cancel_flag equal to 1 cancels the persistence scope of the ar_object_idx[ i ]-th object. ar_object_cancel_flag equal to 0 indicates that parameters associated with the ar_object_idx[ i ]-th object are signalled.

ObjectTracked[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_object_idx[ i ]-th object is tracked. ObjectTracked[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_object_idx[ i ]-th object is not tracked.

ar_bounding_box_cancel_flag equal to 1 cancels the persistence scope of the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], ar_bounding_box_height[ ar_object_idx[ i ] ]. ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ]. ar_bounding_box_cancel_flag equal to 0 indicates that ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] ar_bounding_box_height[ ar_object_idx[ i ] ] ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are signalled.

ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 1 indicates that the bounding box information of the ar_object_idx[ i ]-th object is signalled. ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 0 indicates that the bounding box information of the ar_object_idx[ i ]-th object is not signalled.

ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the ar_object_idx[ i ]-th object in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

The value of ar_bounding_box_left[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − 1, inclusive.

The value of ar_bounding_box_top[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − 1, inclusive.

The value of ar_bounding_box_width[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − ar_bounding_box_left[ ar_object_idx[ i ] ], inclusive.

The value of ar_bounding_box_height[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − ar_bounding_box_top[ ar_object_idx[ i ] ], inclusive.

The identified object rectangle contains the luma samples with horizontal picture coordinates from SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] ) to SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] + ar_bounding_box_width[ ar_object_idx[ i ] ] ) − 1, inclusive, and vertical picture coordinates from SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] ) to SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] + ar_bounding_box_height[ ar_object_idx[ i ] ] ) − 1, inclusive.

When ChromaArrayTypeChromaFormatIdc is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

ar_object_confidence[ ar_object_idx[ i ] ] indicates the degree of confidence associated with the ar_object_idx[ i ]-th object, in units of 2−( ar_object_confidence_length_minus1 + 1 ), such that a higher value of ar_object_confidence[ ar_object_idx[ i ] ] indicates a higher degree of confidence. The length of the ar_object_confidence[ ar_object_idx[ i ] ] syntax element is ar_object_confidence_length_minus1 + 1 bits. When not present, the value of ar_object_confidence[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CVS, if any.

HEVC

D.3.47	Annotated regions SEI message semantics

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects.

ar_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous annotated regions SEI message that is associated with one or more layers to which the annotated regions SEI message applies. ar_cancel_flag equal to 0 indicates that annotated regions information follows.

When ar_cancel_flag equal to 1 or a new CLVS of the current layer begins, the variables LabelAssigned[ i ], ObjectTracked[ i ], and ObjectBoundingBoxAvail are set equal to 0 for i in the range of 0 to 255, inclusive.

Let picA be the current picture. Each region identified in the annotated regions SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an annotated regions SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, and the semantics of the annotated regions SEI message for PicB cancels the persistence of the region identified in the annotated regions SEI message for PicA.

ar_not_optimized_for_viewing_flag equal to 1 indicates that the decoded pictures that the annotated regions SEI message applies to are not optimized for user viewing, but rather are optimized for some other purpose such as algorithmic object classification performance. ar_not_optimized_for_viewing_flag equal to 0 indicates that the decoded pictures that the annotated regions SEI message applies to may or may not be optimized for user viewing.

ar_true_motion_flag equal to 1 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to was selected with a goal of accurately representing object motion for objects in the annotated regions. ar_true_motion_flag equal to 0 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to may or may not be selected with a goal of accurately representing object motion for objects in the annotated regions.

ar_occluded_object_flag equal to 1 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object or a portion of an object that may not be visible or may be only partially visible within the cropped decoded picture. ar_occluded_object_flag equal to 0 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is entirely visible within the cropped decoded picture. It is a requirement of bitstream conformance that the value of ar_occluded_object_flag shall be the same for all annotated_regions( ) syntax structures within a CLVS.

ar_object_confidence_info_present_flag equal to 1 indicates that ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements are present. ar_object_‌confidence_info_present_flag equal to 0 indicates that ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_object_‌confidence_present_flag shall be the same for all annotated_regions( ) syntax structures within a CLVS.

ar_object_confidence_length_minus1 plus 1 specifies the length, in bits, of the ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements. It is a requirement of bitstream conformance that the value of ar_object_‌confidence_length_minus1 shall be the same for all annotated_regions( ) syntax structures within a CLVS.

ar_object_cancel_flag equal to 1 cancels the persistence scope of the ar_object_idx[ i ]-th object. ar_object_cancel_flag equal to 0 indicates that parameters associated with the ar_object_idx[ i ]-th object tracked object are signalled.

ObjectTracked[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_object_idx[ i ]-th object is tracked. ObjectTracked[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_object_idx[ i ]-th object is not tracked.

ar_object_label_update_flag equal to 1 indicates that an object label is signalled. ar_object_label_update_flag equal to 0 indicates that an object label is not signalled.

ar_object_label_idx[ ar_object_idx[ i ] ] indicates the index of the label corresponding to the ar_object_idx[ i ]-th object. When ar_object_label_idx[ ar_object_idx[ i ] ] is not present, its value is inferred from a previous annotated regions SEI message in output order in the same CLVS, if any. The value of ar_object_label_idx[ ar_object_idx[ i ] ] shall be in the range of 0 to 255, inclusive.

ar_bounding_box_update_flag equal to 1 indicates that object bounding box parameters are signalled.  ar_bounding_box_‌update_flag equal to 0 indicates that object bounding box parameters are not signalled.

ar_bounding_box_cancel_flag equal to 1 cancels the persistence scope of the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], ar_bounding_box_‌height[ ar_object_idx[ i ] ]. ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_‌confidence[ ar_object_idx[ i ] ]. ar_bounding_box_‌cancel_flag equal to 0 indicates that ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] ar_bounding_box_‌height[ ar_object_idx[ i ] ] ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements are signalled.

ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 1 indicates that the bounding box information of the ar_object_idx[ i ]-th object is signalled. ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 0 indicates that the bounding box information of the ar_object_idx[ i ]-th object is not signalled.

ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the ar_object_idx[ i ]-th object in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

Let croppedWidth and croppedHeight be the width and height, respectively, of the cropped decoded picture in units of luma samples, as specified by Equations D-28 and D-29.

The value of ar_bounding_box_‌left[ ar_object_idx[ i ] ] shall be in the range of 0 to croppedWidth / SubWidthC − 1, inclusive.

The value of ar_bounding_box_‌top[ ar_object_idx[ i ] ] shall be in the range of 0 to croppedHeight / SubHeightC − 1, inclusive.

The value of ar_bounding_box_‌width[ ar_object_idx[ i ] ] shall be in the range of 0 to croppedWidth / SubWidthtC − ar_bounding_box_‌left[ ar_object_idx[ i ] ], inclusive.

The value of ar_bounding_box_‌height[ ar_object_idx[ i ] ] shall be in the range of 0 to croppedHeight / SubHeightC − ar_bounding_box_‌top[ ar_object_idx[ i ] ], inclusive.

The identified object rectangle contains the luma samples with horizontal picture coordinates from SubWidthC * ( conf_win_left_offset + ar_bounding_box_‌left[ ar_object_idx[ i ] ] ) to SubWidthC * ( conf_win_left_offset + ar_bounding_box_‌left[ ar_object_idx[ i ] ] + ar_bounding_box_‌width[ ar_object_idx[ i ] ] ) − 1, inclusive, and vertical picture coordinates from SubHeightC * ( conf_win_top_offset + ar_bounding_box_‌top[ ar_object_idx[ i ] ] ) to SubHeightC * ( conf_win_top_offset + ar_bounding_box_‌top[ ar_object_idx[ i ] ] + ar_bounding_box_‌height[ ar_object_idx[ i ] ] ) − 1, inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

The values of ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] persist in output order within the CLVS for each value of ar_object_idx[ i ]. When not present, the values of  ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] or ar_bounding_box_‌height[ ar_object_idx[ i ] ] are inferred from a previous annotated regions SEI message in output order in the CLVS, if any.

ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is only partially visible within the cropped decoded picture. ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that may or may not be only partially visible within the cropped decoded picture. When not present, the value of ar_partial_object_flag[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CLVS, if any.

ar_object_confidence[ ar_object_idx[ i ] ] indicates the degree of confidence associated with the ar_object_idx[ i ]-th object, in units of 2−( ar_object_confidence_length_minus1 + 1 ), such that a higher value of ar_object_confidence[ ar_object_idx[ i ] ] indicates a higher degree of confidence. The length of the ar_object_confidence[ ar_object_idx[ i ] ] syntax element is ar_object_confidence_length_minus1 + 1 bits. When not present, the value of ar_object_confidence[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CLVS, if any.

AVC

D.2.38	Annotated regions SEI message semantics

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects.

The following variables are specified for the use of this SEI message:

–	CroppedWidth and CroppedHeight, set as specified by Equations D-30 and D-31, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset, set equal to CropUnitX * frame_crop_left_offset.

–	A conformance cropping window top offset, ConfWinTopOffset, set equal to CropUnitY * frame_crop_top_offset.

ar_occluded_object_flag equal to 1 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object or a portion of an object that may not be visible or may be only partially visible within the cropped decoded picture. ar_occluded_object_flag equal to 0 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is entirely visible within the cropped decoded picture. It is a requirement of bitstream conformance that the value of ar_occluded_object_flag shall be the same for all annotated_regions( ) syntax structures within a coded video sequence.

ar_object_confidence_info_present_flag equal to 1 indicates that ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements are present. ar_object_‌confidence_info_present_flag equal to 0 indicates that ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_object_‌confidence_present_flag shall be the same for all annotated_regions( ) syntax structures within a coded video sequence.

ar_object_confidence_length_minus1 plus 1 specifies the length, in bits, of the ar_object_‌confidence[ ar_object_idx[ i ] ] syntax elements. It is a requirement of bitstream conformance that the value of ar_object_‌confidence_length_minus1 shall be the same for all annotated_regions( ) syntax structures within a coded video sequence.

ar_object_cancel_flag equal to 1 cancels the persistence scope of the ar_object_idx[ i ]-th object. ar_object_cancel_flag equal to 0 indicates that parameters associated with the ar_object_idx[ i ]-th object tracked object are signalled.

ObjectTracked[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_object_idx[ i ]-th object is tracked. ObjectTracked[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_object_idx[ i ]-th object is not tracked.

ar_bounding_box_cancel_flag equal to 1 cancels the persistence scope of the ar_bounding_box_‌top[ ar_object_idx‌[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ], ar_bounding_box_‌height[ ar_object_idx[ i ] ]. ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ]. ar_bounding_box_‌cancel_flag equal to 0 indicates that ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] ar_bounding_box_‌height[ ar_object_idx[ i ] ] ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are signalled.

ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 1 indicates that the bounding box information of the ar_object_idx[ i ]-th object is signalled. ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] equal to 0 indicates that the bounding box information of the ar_object_idx[ i ]-th object is not signalled.

ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_‌object_idx[ i ] ], and ar_bounding_box_‌height[ ar_object_idx[ i ] ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the ar_object_idx[ i ]-th object in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

The value of ar_bounding_box_left[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − 1, inclusive.

The value of ar_bounding_box_top[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − 1, inclusive.

The value of ar_bounding_box_width[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − ar_bounding_box_‌left[ ar_object_idx[ i ] ], inclusive.

The value of ar_bounding_box_height[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − ar_bounding_box_‌top[ ar_object_idx[ i ] ], inclusive.

The identified object rectangle contains the luma samples with horizontal picture coordinates from SubWidthC * ( ConfWinLeftOffset + ar_bounding_box_‌left[ ar_object_idx[ i ] ] ) to SubWidthC * ( ConfWinLeftOffset + ar_‌bounding‌_box_‌left[ ar_object_idx[ i ] ] + ar_bounding_box_‌width[ ar_object_idx[ i ] ] ) − 1, inclusive, and vertical picture coordinates from SubHeightC * ( ConfWinTopOffset + ar_bounding_box_‌top[ ar_object_idx[ i ] ] ) to SubHeightC * ( ConfWinTopOffset + ar_bounding_box_‌top[ ar_object_idx[ i ] ] + ar_bounding_box_‌height[ ar_‌object_‌idx[ i ] ] ) − 1, inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

The values of ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_‌box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] persist in output order within the coded video sequence for each value of ar_object_idx[ i ]. When not present, the values of ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] or ar_bounding_box_‌height[ ar_object_idx[ i ] ] are inferred from a previous annotated regions SEI message in output order in the coded video sequence, if any.

ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is only partially visible within the cropped decoded picture. ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_bounding_box_‌top[ ar_object_idx[ i ] ], ar_bounding_box_‌left[ ar_object_idx[ i ] ], ar_bounding_box_‌width[ ar_object_idx[ i ] ] and ar_bounding_box_‌height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that may or may not be only partially visible within the cropped decoded picture. When not present, the value of ar_partial_object_flag[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the coded video sequence, if any.

ar_object_confidence[ ar_object_idx[ i ] ] indicates the degree of confidence associated with the ar_object_idx[ i ]-th object, in units of 2−( ar_object_confidence_length_minus1 + 1 ), such that a higher value of ar_object_confidence[ ar_object_idx[ i ] ] indicates a higher degree of confidence. The length of the ar_object_confidence[ ar_object_idx[ i ] ] syntax element is ar_object_confidence_length_minus1 + 1 bits. When not present, the value of ar_object_confidence[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the coded video sequence, if any.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

References

[1]	Recommendation ITU-T H.274 (V2) Versatile supplemental enhancement information messages for coded video bitstream, May 2022.

[2]	Recommendation ITU-T H.265 (V8) High efficiency video coding, Aug. 2021.

[3]	Recommendation ITU-T H.264 (V14) Advanced video coding for generic audiovisual services, Aug. 2021.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).