| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31th Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0057-v1 |
| --- | --- |

| Title: | AHG10/AHG12: MTT split modes early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | W. Ahmad, P. Wennersten, K. Andersson | Tel: Email: | +46761046057 {waqas.ahmad, per.wennersten, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents an early termination method that is claimed to reduce encoding runtime for  ECM and VTM. The cost of coding the CU without splitting the CU at all is compared to a threshold. If the cost is higher than the threshold, MTT split will not be considered, which is claimed to reduce encoding runtime. The proposal is implemented on top of ECM 9.0 software and compared to ECM-9.0, the average BDR loss and runtimes reported in this contribution are as follows:

Y 0.03%, U 0.06%,	V 0.11%,  EncTime 97%, DecTime 100% for the RA configuration,

Y x.xx%, U x.xx%, V x.xx %, EncTime xx%, DecTime xxx% for the LDB configuration,

For the worst-case RA encoding time, it is claimed that the proposed scheme reduces the runtime in line with these figures: For the slowest chunk in Parkrunning3 QP22, the run time is reported to be reduced by 3%.

The proposed scheme is also implemented on top of VTM 20.2 and compared to VTM 20.2 the average BDR loss, and runtimes reported in this contribution are as follows:

Y 0.02%, U 0.06%, V 0.05%, EncTime  96%, DecTime 100% for the RA configuration,

Y 0.06%, U 0.39%,	V 0.21%, EncTime 96%, DecTime 100% for the LDB configuration,

It is proposed to include the method in ECM and VTM software for RA camera captured content.

# Introduction

Generally, for every CU, ECM evaluates the no-split mode, followed by QT and MTT splitting modes. The MTT modes contains BTH, BTV, TTH, and TTV splitting modes. The QT and MTT splitting modes in ECM provide support for rectangular CUs as well as square CUs, which allows a better fit to local picture content characteristics and thus improves coding gain. However, with the variety of splitting modes in ECM, on the one hand coding gain is improved, but on the other hand, it increases the encoding time. This contribution proposes to add a criterion to decide when to early terminate the MTT splitting modes.

# Proposed method

In ECM, the no-split cost represents the minimum rate-distortion cost found without splitting the CU with either QT split mode or MTT split modes. In this contribution, we propose that a condition is applied on the no-split cost of 64x64 luma CUs. If the no-split cost is greater than a threshold, MTT split mode will not be investigated, saving runtime. An exception is made if MTT split has already been selected in parents to the 64x64 CU. This is because in ECM, after an MTT split mode has been used, QT splitting is not allowed for the subsequent CUs. Hence, early termination of MTT split modes for such CUs would hurt performance unduly. The threshold is adaptively set based on QP value as follows

TQP=120-QP-22×106.

Here, QP represents value of a base QP and TQP represents the threshold value for the corresponding QP.

# Results

In the RA configuration, the encoding time reduction for ClassB, ClassC and ClassD is estimated by sequentially running the proposed and anchor scheme on a same machine. The order of test and anchor scheme is randomly selected. For ClassA1 and A2, the test and anchor schemes are run in parallel at same time without restriction on having same host machine.

In the LDB configuration, for ClassB, ClassC, ClassE, and ClassD the test and anchor schemes are run on the same machine in sequential manner. The order of test and anchor scheme is randomly selected.

The results are presented in the following subsections.

## ECM 9.0

The proposed scheme is implemented on top of the ECM-9.0 [1] and experiments are performed by following the common test conditions for ECM9.0 [2]. An encoding time reduction is obtained for RA configuration with limited degradation in coding efficiency.

Table 1 Coding performances of the proposed modifications in ECM-9.0 for RA configuration.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | 0.07% | 99.7% | 99.8% |
| Class A2 | 0.04% | 0.01% | -0.02% | 97.0% | 99.6% |
| Class B | 0.04% | 0.16% | 0.18% | 96.5% | 100.3% |
| Class C | 0.02% | 0.06% | 0.15% | 95.8% | 99.8% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.06% | 0.11% | 97.0% | 99.9% |
| Class D | -0.04% | -0.01% | 0.06% | 97.3% | 99.9% |

Table 2 Coding performances of the proposed modifications in ECM-9.0 for LDB configuration.

|  | Low delay B 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C | 0.04% | -0.16% | -0.19% | 96.3% | 99.6% |
| Class E | 0.04% | -0.03% | -0.60% | 98.7% | 100.2% |
| Overall |  |  |  |  |  |
| Class D | -0.14% | 0.11% | 0.61% | 98.1% | 99.9% |

### Worst-case encoding time for RA

In the worst case for parallel encoding of RA, which corresponds to the sequences DaylightRoad2 and ParkRunning3, the encoding time per intra period is reduced by 12% and 3%, respectively with the proposed MTT early termination scheme implemented in ECM-9.0.

Table 3: Impact of the proposed scheme on ECM-9.0 random access

| ECM 9.0 Random Access, QP=22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ECM Enc Time |  | Chunks | ECM Enc time/chunk |  |  | Enc time reduction |  |  |  | Enc time reduction/chunk |  |  |
| Sequence | [h] | [days] |  | [h] | [days] | [h] |  | [days] | [%] | [h] |  | [days] | [%] |
| Tango2 | 817.8 | 34.08 | 5 | 190.95 | 7.96 | -8.44 |  | -0.36 | -1 | 5.21 |  | 0.21 | 3 |
| FoodMarket4 | 549.21 | 22.88 | 5 | 163 | 6.79 | 52.11 |  | 2.18 | 9 | 1.65 |  | 0.07 | 1 |
| Campfire | 1289.31 | 53.72 | 10 | 150.02 | 6.25 | -121.64 |  | -5.07 | -9 | -1.53 |  | -0.06 | -1 |
| CatRobot1 | 652.03 | 27.17 | 5 | 158.79 | 6.62 | -32.19 |  | -1.34 | -5 | -10.77 |  | -0.45 | -7 |
| DaylightRoad2 | 949.34 | 39.56 | 5 | 235.32 | 9.81 | -80.9 |  | -3.37 | -9 | -27.36 |  | -1.14 | -12 |
| ParkRunning3 | 1117.29 | 46.55 | 5 | 251.06 | 10.46 | -43.61 |  | -1.81 | -4 | -8.48 |  | -0.35 | -3 |

## VTM 20.2

The proposed scheme is implemented on top of the VTM-20.2 [3] and tested according to the common test conditions for VTM [4]. The results for RA and LDB configurations are presented in the table below.

Table 4 Coding performances of the proposed modifications in VTM-20.2 for RA configuration.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 20.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.02% | 99.3% | 99.6% |
| Class A2 | 0.06% | 0.09% | 0.07% | 95.4% | 100.1% |
| Class B | 0.03% | 0.10% | 0.09% | 96.1% | 99.9% |
| Class C | 0.00% | 0.04% | 0.03% | 94.2% | 100.3% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.06% | 0.05% | 96.1% | 100.0% |
| Class D | 0.03% | -0.22% | 0.06% | 96.0% | 99.6% |

Table 5 Coding performances of the proposed modifications in VTM-20.2 for LDB configuration.

|  | Low delay B 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.07% | 0.45% | 0.51% | 95.6% | 100.1% |
| Class C | 0.04% | 0.33% | -0.07% | 94.7% | 99.7% |
| Class E | 0.05% | 0.38% | 0.06% | 99.5% | 99.6% |
| Overall | 0.06% | 0.39% | 0.21% | 96.3% | 99.8% |
| Class D | 0.03% | 0.62% | -0.45% | 96.1% | 100.0% |

### Worst-case encoding time for RA

In the worst case for parallel encoding of RA, which corresponds to the sequences DaylightRoad2 and ParkRunning3 at QP 22, the encoding time per intra period is reduced by 15% and 10%, respectively, with the proposed MTT early termination scheme implemented in VTM-20.2.

Table 6: Impact of the proposed scheme on VTM-20.2 random access

| VTM-20.2 Random Access, QP=22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ECM Enc Time |  | Chunks | ECM Enc time/chunk |  |  | Enc time reduction |  |  |  | Enc time reduction/chunk |  |  |
| Sequence | [h] | [days] |  | [h] | [days] | [h] |  | [days] | [%] | [h] |  | [days] | [%] |
| Tango2 | 121.18 | 5.05 | 5 | 29,06 | 1,21 | 5,31 |  | 0,22 | 4% | 1,39 |  | 0,06 | 5% |
| FoodMarket4 | 80.76 | 3.37 | 5 | 24,02 | 1 | 2,35 |  | 0,09 | 3% | 1,22 |  | 0,05 | 5% |
| Campfire | 243.17 | 10.13 | 10 | 30,58 | 1,27 | -34,77 |  | -1,45 | -14% | -5,18 |  | -0,21 | -17% |
| CatRobot1 | 100.64 | 4.19 | 5 | 22,05 | 0,92 | -12,44 |  | -0,51 | -12% | -2,5 |  | -0,11 | -11% |
| DaylightRoad2 | 136.78 | 5.7 | 5 | 33,34 | 1,39 | -13,18 |  | -0,55 | -10% | -4,91 |  | -0,21 | -15% |
| ParkRunning3 | 203.01 | 8.46 | 5 | 44,52 | 1,86 | -22,99 |  | -0,96 | -11% | -4,34 |  | -0,19 | -10% |

# Conclusion

In this contribution, MTT split modes early termination scheme is proposed. The method uses the no-split cost of the CU to determine whether to evaluate MTT split modes.  It has been shown that the proposed MTT split modes early termination scheme reduces encoding time with limited coding loss. In particular, the proposed scheme reduces the worst-case encoding time for RA by 3% in ECM and 10% in VTM. It is therefore suggested to include the proposed method in the ECM and VTM software for RA configuration.

# References

- ECM-9.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

- VTM-20.2, reference software:  https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-20.2

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).