| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0058-v1 |
| --- | --- |

| Title: | AHG12: Spatial CABAC tuning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Ramin G. Youvalari Döne Bugdayci Sansli | Tel: Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that the current CABAC context update mechanism may lead to “overfitting” the probability estimates to the statistical properties of the CTU that is preceding the current CTU in scan order. To help overcome such bias, it is proposed to update the CABAC contexts using syntax element values retrieved from the above CTU when starting to process a new CTU. A limited number of bins (e.g., 512) of syntax elements belonging to the bottom CUs of the above CTU are fed into the context update mechanism of the shorter window probability estimator to achieve this.

It is reported that the proposed approach has the following BD-rate and runtime impact when implemented in ECM-9:

AI { -0.04%, 0.00%, -0.01%, 99%, 97%}

RA { -0.06%, -0.18%, -0.02%, 100%, 100%}

It is further reported that an alternative configuration with larger maximum number of stored bins (1024 instead of 512) per CTU and updating both shorter and longer window probability estimators has the following BD-rate and runtime impact:

AI { -0.05%, -0.03%, -0.04%, 99%, 100%}

RA { -0.07%, -0.18%, -0.07%, 101%, 100%}

# Introduction

It is asserted that the current CABAC context update mechanism may lead to “overfitting” the probability estimates to the statistical properties of the CTU or couple of CTUs that are preceding the current CTU in scan order.

Figure 1 illustrates the processing order of the blocks and related syntax elements in ECM. CTUs are scanned in raster scan order, while CUs inside CTUs are scanned in nested tree order. When starting to process the current CTU (CTUC) the processing “jumps” from the bottom-right CU (yellow CU 8) of CTUL to the top-left CU of the current CTU (blue CU 0 of CTUC). At that stage, the context parameters of the CABAC engine can be expected to be finetuned for the bottom-right area of the previous CTU instead of the ideal case where the context parameters would be finetuned for the actual next CU to be processed.

It is proposed to use the bins of the syntax elements of the bottom CUs of the above CTU (the green CUs of CTUA in the example of Fig. 1) to tune the CABAC contexts when starting to process a new CTU. It was found that tuning the state parameters of the shorter window probability estimator is typically enough and relatively minor additional gains are achieved by tuning both probability estimators.

Figure 1: Example of processing order of the CUs in the left and above CTUs

# Proposed changes to ECM

It is proposed to buffer bins relating to syntax elements of bottom CUs of CTUs in one CTU line. The number of stored bins per CTU is limited to a given number (maximum of 512 and 1024 stored binds were used in the reported experiments) to keep the buffering and updating cost at low level. It is also proposed to limit the number of stored bins related to a single context to a relative low number to avoid situation where a dominant syntax element may alone reserve a significant portion of the total storage. In the experiments reported here, the limit for the number of stored bins for a single context was set to 32.

When starting to encode or decode a new CTU, the stored bins from the above CTU are retrieved and fed into the context update mechanism of the shorter window probability estimator (with “window 0”). The bins are fed to context update process in the reverse order with respect to their coding order to simulate encoding or decoding that approaches the top-left corner of the new CTU from the direction of bottom-right corner of the CTU above. Results are also provided for an alternative configuration where contexts of both the shorter window and longer window estimators are updated.

# Experiments results

## Test 1 results

Test 1 uses the following configuration:

- Maximum number of stored bins per CTU: 	512

- Updated probability estimates:				Short window estimates only

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04 % | -0.04 % | -0.05 % | 98 % | 96 % |
| Class A2 | -0.04 % | 0.01 % | 0.02 % | 97 % | 98 % |
| Class B | -0.04 % | 0.04 % | -0.05 % | 99 % | 98 % |
| Class C | -0.02 % | -0.06 % | -0.03 % | 100 % | 97 % |
| Class E | -0.06 % | 0.05 % | 0.11 % | 99 % | 96 % |
| Overall | -0.04 % | 0.00 % | -0.01 % | 99 % | 97 % |
| Class D | 0.00 % | -0.01 % | -0.04 % | 101 % | 92 % |
| Class F | -0.02 % | -0.10 % | 0.19 % | 99 % | 96 % |
| TGM | -0.07 % | -0.12 % | -0.07 % | 98 % | 99 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -0.34 % | 0.01 % | 101 % | 98 % |
| Class A2 | -0.06 % | -0.25 % | -0.07 % | 101 % | 97 % |
| Class B | -0.08 % | -0.07 % | 0.07 % | 100 % | 102 % |
| Class C | -0.03 % | -0.14 % | -0.12 % | 100 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.06 % | -0.18 % | -0.02 % | 100 % | 100 % |
| Class D | 0.07 % | -0.28 % | 0.07 % | 101 % | 101 % |
| Class F | -0.08 % | -0.34 % | -0.26 % | 100 % | 101 % |
| TGM | -0.03 % | 0.00 % | -0.03 % | 101 % | 101 % |

## Test 2 results

Test 2 uses the following configuration:

- Maximum number of stored bins per CTU: 	1024

- Updated probability estimates:				Both short and long window estimates

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -0.09 % | -0.08 % | 99 % | 101 % |
| Class A2 | -0.04 % | -0.08 % | -0.08 % | 99 % | 100 % |
| Class B | -0.06 % | 0.03 % | -0.03 % | 99 % | 99 % |
| Class C | -0.05 % | -0.01 % | -0.07 % | 99 % | 101 % |
| Class E | -0.06 % | -0.04 % | 0.08 % | 99 % | 99 % |
| Overall | -0.05 % | -0.03 % | -0.04 % | 99 % | 100 % |
| Class D | 0.01 % | 0.00 % | -0.03 % | 100 % | 99 % |
| Class F | -0.06 % | -0.13 % | -0.03 % | 99 % | 101 % |
| TGM | -0.08 % | -0.12 % | -0.08 % | 97 % | 99 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | -0.30 % | -0.02 % | 101 % | 97 % |
| Class A2 | -0.10 % | -0.08 % | -0.06 % | 100 % | 98 % |
| Class B | -0.06 % | -0.18 % | -0.04 % | 101 % | 102 % |
| Class C | -0.06 % | -0.17 % | -0.15 % | 101 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.07 % | -0.18 % | -0.07 % | 101 % | 100 % |
| Class D | 0.00 % | -0.35 % | 0.15 % | 102 % | 100 % |
| Class F | -0.09 % | -0.15 % | -0.13 % | 101 % | 101 % |
| TGM | 0.02 % | 0.00 % | 0.00 % | 101 % | 101 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).