| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0062-v1 |
| --- | --- |

| Title: | AHG9: On NNPF for DeInterlacing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ahmed Sidiya, Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is proposed to allow specifying deinterlacing as a new NNPF purpose. For this following is proposed:

- It is proposed to signal a flag that indicates the number of output pictures for each interlaced input picture to the NNPF.

- It is proposed to signal if the input needs to be flipped so that a neural network trained on top field first can work on bottom field first content and vice-versa.

# Introduction

Additional SEI messages for VSEI [1] specifies the syntax and semantics for Neural network post-filter characteristics (NNPFC) SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Some modifications are proposed to the neural-network post-filter characteristics SEI Message related to the auxiliary input information.

# Proposals

It is proposed to specify deinterlacing as a new NNPF purpose. For this following is proposed:

- It is proposed to add deinterlacing purpose and to signal the output picture height and width information (via the width and height scale factors).

- It is proposed to signal a flag that indicates the number of output pictures for each interlaced input picture to the NNPF. It is asserted that a deinterlacing-based neural network post-processing filter has the capability to generate multiple pictures for a single input picture. In the scenario where an interlaced input picture is formed by combining two fields, the neural-network post filter output may generate two full resolution pictures. As an example, the neural network filter described in [2] operates this way. The Figure 4 from [2] illustrates this behavior.

- It is asserted that some neural network post filters for deinterlacing are trained on top field first. It is proposed to signal if some of the input pictures need to be flipped so that a neural network trained on top field first can work on bottom field first input pictures (and vice-versa). It is asserted that a neural network trained to predict missing even lines in a picture of interlaced content will fail when tested on a picture that is missing odd lines (and vice versa). However, by applying a flipping operation to the input and output of the de-interlacing neural network, when necessary, it becomes possible to successfully predict both missing even and odd lines.

To validate this assertion, we conducted experiments using the model weights from [3] to predict missing odd lines in a picture. The model was trained to predict missing even lines. By applying the flipping operation, we were able to achieve a peak signal-to-noise ratio (PSNR) value comparable to the one obtained when predicting the correct missing lines. Without the use of the flipping operation, when necessary, a loss of more than 3 dB is incurred. Table 1 shows the results.

| ​ Missing Lines | Trained to predict even lines in an image​ | Trained to predict even lines in a picture and with the necessary flipping operation applied​ |
| --- | --- | --- |
| even lines missing​ | 35.80​ dB | 35.80​ dB |
| odd lines missing​ | 32.52​ dB | 35.82​ dB |

Table 1. PSNR results for the prediction from [3], when the necessary flipping operation is done (last column) the results are the best as can be seen in the last row.

The proposed modifications are shown highlighted below compared with JVET-AD2006-v1 [1].

    - Neural-network post-filter characteristics SEI message

      - Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( colourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( resolutionResamplingFlag ¦ ¦ deInterlacingFlag) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| nnpfc_per_output_deinterlaced _pics_flag | u(1) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ )​ |  |
| nnpfc_input_field_flip_flag[ i ]​ | u(1) |
| } |  |
| … |  |
| } |  |

      - Neural-network post-filter characteristics SEI message semantics

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application.

The value of nnpfc_purpose shall be in the range of 0 to 63 127, inclusive, in bitstreams conforming to this edition of this document. Values of 64 128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Deinterlacing |

The variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, and colourizationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, and colourization, respectively, are derived as follows:

chromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
resolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
pictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(76)
bitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
colourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
deinterlacingFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0

…

nnpfc_per_output_deinterlaced_pics_flag equal to 1 indicates that two output pictures are generated for each interlaced input picture. nnpfc_per_output_deinterlaced_pics_flag equal 0 indicates that one output picture is generated for each interlaced input picture.

nnpfc_input_field_flip_flag[ i ] equal to 1 indicates that the i-th input picture is rotated 180 degrees counterclock-wise in the DeriveInputTensors() and StoreOutputTensor() processes. nnpfc_input_field_flip_flag[ i ] equal to 0 indicates that the i-th input picture is not rotated in the DeriveInputTensors() and StoreOutputTensor() processes.

…

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the NNPF between the i-th and the ( i + 1 )-th picture used as input for the NNPF. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 63, inclusive. The value of nnpfc_interpolated_pics[ i ] shall be greater than 0 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1 − 1, inclusive.

The variables NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ] specifying the input picture index of the idx-th picture that is present in the output tensor of the NNPF and has a corresponding input picture, and numOutputPics, specifying the total number of pictures present in the output tensor of the NNPF, are derived as follows:

for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_output_flag[ i ] ) {

if(deinterlacingFlag){ 
          		 if(nnpfc_per_output_deinterlaced_pics_flag){ 
                  	InpIdx[ numOutputPics ] = i

InpIdx[ numOutputPics+1 ] = i
		     	numOutputPics+=2  
			}
           	 	 else{
		 		InpIdx[ numOutputPics ] = i
		 	 	numOutputPics++

}

}

else{
		   InpIdx[ numOutputPics ] = i
		   numOutputPics++

}
	}						(80)
NumInpPicsInOutputTensor = numOutputPics
if( pictureRateUpsamplingFlag )
	for( i = 0; i  <=  numInputPics − 2; i++ )
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]

…

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag )
                                if(nnpfc_input_field_flip_flag[ i ])
                                {

yPf = inpPatchHeight – 1 – yP

xPf = inpPatchWidth – 1 – xP

}
				inpVal = InpY( InpSampleVal( cTop + yPyPf, cLeft + xPxPf, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  1 )		                                                          (88)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag) 
                                if(nnpfc_input_field_flip_flag[ i ])
                                 {

yPf = inpPatchHeight – 1 – yP

xPf = inpPatchWidth – 1 – xP

}
				inpCbVal = InpC( InpSampleVal( cTop + yPyPf, cLeft + xPxPf, CroppedHeight /       SubHeightC, CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yPyPf, cLeft + xPxPf, CroppedHeight / SubHeightC, CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag)
                                if(nnpfc_input_field_flip_flag[ i ])
                                 {

yPf = inpPatchHeight – 1 – yP

xPf = inpPatchWidth – 1 – xP

}

				yY = cTop + yPyPf
				xY = cLeft + xPxPf
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag)
                                if(nnpfc_input_field_flip_flag[ i ])
                                 {

yPf = inpPatchHeight – 1 – yP

xPf = inpPatchWidth – 1 – xP

}
				yTL = cTop + yPyPf * 2
				xTL = cLeft + xPxPf * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yPyPf
				xC = cLeft / 2 + xPxPf
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ]
			}
}

…

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag) 
                                if(nnpfc_input_field_flip_flag[ InpIdx[i] ])
                                 {

yPf = outPatchHeight – 1 – yP

xPf =  outPatchWidth –1–xP

}
				yY = cTop * outPatchHeight / inpPatchHeight + yPyPf
				xY = cLeft * outPatchWidth / inpPatchWidth + xPxPf

if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP][ xP][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		                                                            (91)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag)
                                if(nnpfc_input_field_flip_flag[ InpIdx[i] ])
                                 {

yPf = outPatchHeight – 1 – yP

xPf =  outPatchWidth –1–xP

}
				xSrc = cLeft * horCScaling + xPxPf
				ySrc = cTop * verCScaling + yPyPf

if ( ySrc < nnpfcOutputPicHeight / outSubHeightC  &&
						xSrc < nnpfcOutputPicWidth / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag)
                                if(nnpfc_input_field_flip_flag[ InpIdx[i] ])
                                 {

yPf = outPatchHeight – 1 – yP

xPf =  outPatchWidth –1–xP

}
				yY = cTop * outPatchHeight / inpPatchHeight + yPyPf
				xY = cLeft * outPatchWidth / inpPatchWidth + xPxPf
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC

yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC

if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {

yPf = yP

xPf = xP

if(deinterlacingFlag)
                                if(nnpfc_input_field_flip_flag[ InpIdx[i] ])
                                 {

yPf = outPatchHeight – 1 – yP

xPf =  outPatchWidth –1–xP

}
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yPyPf
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xPxPf

if ( ySrc < nnpfcOutputPicHeight / 2  &&
						xSrc < nnpfcOutputPicWidth / 2 )
					if( !nnpfc_component_last_flag ) {
					FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
				FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
				FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
			     FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
				  FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
				  FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
				  FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
			FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
				FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
				FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposals in the next version of “Additional SEI messages for VSEI” text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-AD2006, Additional SEI Messages for VSEI (Draft 4), Online, April 2023

[2] Zhu, H., Liu, X., Mao, X., & Wong, T.-T. (2017). Real-time Deep Video Deinterlacing. CoRR, abs/1708.00187.

[3] Bernasconi, M., Djelouah, A., Hattori, S., Schroers, C. (2020). Deep Deinterlacing.

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.