| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0064-v1 |
| --- | --- |

| Title: | AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Dong-Gyu Gwak Hendry Jaehyun Lim Seung hwan KimName(s) Miska M. Hannuksela Francesco Cricri Honglei Zhang | Email: | chulkeun.kim@lge.com firstname.lastname@nokia.com |
| Source: | LG electronics and Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message for signalling encoder preprocessing type that is applied to video bitstream and indication whether the optimization is targeting for human viewing and / or machine analysis. The proposed SEI message contains the following information:

- Encoding pre-processing type applied to picture(s) associated with the SEI message.

- Indication whether the pre-processing for the picture(s) is optimized for human viewing.

- Indication whether the pre-processing for the pictures(s) is optimized for machine analysis.

The proposed SEI message is the result of further study on contributions made in previous JVET meetings that include JVET-AD0060 and JVET-AD0137.

# Introduction

Several contributions advocating the need for signalling pre-processing that have been applied at encoder side to video bitstream and also whether such pre-processing is for optimizing the decoding pictures for human viewing and / or machine analysis have been proposed in the previous JVET meetings. The contributions can be briefly summarized as follows:

- JVET-AB0274 and JVET-AB0275 discussed and proposed encoder pre-processing methods that may help optimize the bitstream for region of interest (ROI) applications.

- JVET-AC0077 and JVET-AD0060 discussed and proposed the case where decoded pictures may be sub-optimal for certain uses that makes it suitable / not suitable for human viewing and / or machine analysis. Furthermore, some methods for indicating such sub-optimality were proposed.

- JVET-AD0137 proposed SEI message for signalling of encoder pre-processing method that is applied to picture(s) associated with the SEI message.

This contribution is a result of further studies on the above contributions.

# Proposed SEI message

We propose an SEI message to signal the following information:

- Encoding pre-processing type applied to picture(s) associated with the SEI message.

- Indication whether the pre-processing for the picture(s) is optimized for human viewing.

- Indication whether the pre-processing for the pictures(s) is optimized for machine analysis.

The syntax and semantics of the proposed SEI message is as shown below:

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| } |  |
| } |  |

optimization_cancel_flag equal to 1 specifies that the persistence of the previous applied optimization is cancelled. optimization_cancel_flag equal to 0 indicates that optimization_persistence_flag and optimization_method follow.

optimization_persistence_flag specifies the persistence of the optimization the current layer. optimization_persistence_flag equal to 0 specifies that the optimization applies for the current picture only. optimization_persistence_flag equal to 1 specifies that the optimization applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder_optimization_info SEI message is output that follows the current picture in output order.

optimization_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. optimization_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

optimization_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. optimization_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

optimization_type indicates the types of optimization method as specified in Table 2.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

# Patent rights declaration(s)

LG Electronics Inc. and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).