| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0069-v1 |
| --- | --- |

| Title: | AHG9: On the signalling of output pictures in NNPFA SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG electronics |  |  |

_____________________________

# Abstract

It is asserted that the current signalling design of output pictures and their association to input pictures in NNPFA SEI message has a bug / an overlook. In the signalling of the output pictures association with input pictures, there are constraints specified in NNPFC SEI message to ensure that at the NNPF shall produce output picture(s). However, when the signalling of output pictures association with input pictures is updated in the NNPFA SEI message, it is possible that the activated NNPFA does not output any picture after the signalling of the output picture flags in the NNPFA SEI message.

To fix the asserted problem, this contribution proposes that the following constraint is added to the semantics of syntax element nnpfa_output_flag[ i ]:

“When nnpfc_purpose & 0x08 is equal to 0 for the associated NNPFC and nnpfa_num_output_entries is equal to NumInpPicsInOutputTensor, nnpfa_output_flag [ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfa_num_output_entries − 1, inclusive”

# Problem description

In the current design of neural-network post-filter characteristic (NNPFC) and neural-network post-filter activation (NNPFA) SEI messages, the design for signalling the output pictures and their association to the input pictures is as follows: it is first signalled in NNPFC SEI message and then it can be updated in NNPFA SEI message that activates the NNPFC. The following snippet show the syntax and semantics of the related design.

-------------------------------Start snippet from NNPFC and NNPFA SEI messages-------------------------------

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

…

nnpfc_num_input_pics_minus1 plus 1 specifies the number of pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When pictureRateUpsamplingFlag is equal to 1, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

The variable numInputPics, specifying the number of pictures used as input for the NNPF, is derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1	(77)

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture. When nnpfc_num_input_pics_minus1 is equal to 0, nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1. When pictureRateUpsamplingFlag is equal to 0 and nnpfc_num_input_pics_minus1 is greater than 0, nnpfc_input_pic_output_flag[ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1, inclusive.

…

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_target_base_flag | u(1) |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |  |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |  |
| nnpfa_output_flag[ i ] | u(1) |  |
| } |  |  |
| } |  |

…

nnpfa_num_output_entries specifies the number of nnpfa_output_flag[ i ] syntax elements present in the NNPFA SEI message. The value of nnpfa_num_output_entries shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive.

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive.

…

-------------------------------End snippet from NNPFC and NNPFA SEI messages-------------------------------

In the signalling of the output pictures association with input pictures, there are constraints established to ensure that at the NNPF shall produce output picture(s) as shown as yellow highlighted text above. However, when the signalling of output pictures association with input pictures is updated in the NNPFA SEI message, it is possible that the activated NNPFA does not output any picture after the signalling of the output picture flags in the NNPFA SEI message. This seems to be a bug / an overlook and constraint(s) that gives the similar effect as those specified in NNPFC SEI message should be added.

# Proposal and text implementation

To fix the asserted bug, we propose the following constraint: when pictureRateUpsamplingFlag is equal to 0 and nnpfa_num_output_entries is equal to NumInpPicsInOutputTensor, nnpfa_output_flag [ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfa_num_output_entries − 1, inclusive.

The implementation of the above proposed constraint is as follows:

…

nnpfa_num_output_entries specifies the number of nnpfa_output_flag[ i ] syntax elements present in the NNPFA SEI message. The value of nnpfa_num_output_entries shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive.

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive. When nnpfc_purpose & 0x08 is equal to 0 for the associated NNPFC and nnpfa_num_output_entries is equal to NumInpPicsInOutputTensor, nnpfa_output_flag [ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfa_num_output_entries − 1, inclusive.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).