| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0070-v1 |
| --- | --- |

| Title: | AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG electronics |  |  |

_____________________________

# Abstract

In the previous JVET meeting it was agreed to allow an NNPF to use input pictures that below to different CLVS. Furthermore, it is also allowed that one or more input picture to be not present in the bitstream (i.e., in the case when the NNPF is activated in the beginning of the bitstream). It is asserted that such design have two problems:

- The NNPF that use input pictures from previous CLVS may produce mismatched filtered picture at the event of random accessing / splicing and currently there is no way for decoder to know this and take it into account whether to activate the NNPF or not.

- It is asserted that unavailable input pictures should not have associated output picture. However, it is not clear how this can be prevented in the current specification.

This contribution proposes solutions the asserted problem above. The proposed solutions are as follows:

- For asserted problem 1, the following is proposed:

  - Signal a flag (i.e., nnpfa_broken_link_flag) in NNPFA SEI message that specifies whether the result of applying the target NNPF may result in broken-link / mismatch (i.e., filtered picture in decoder that is different from is corresponding filtered picture at encoder).

  - When the bitstream is spliced at CLVS that resulted in one or more input pictures of the target NNPF becomes unavailable, the value of nnpfa_broken_link_flag is set to be equal to 1.

  - The flag can be used by decoder to decide whether or not to activate / apply the NNPF.

- For asserted problem 2, we proposed to specify a constraint that ensure that input pictures that are not present in the bitstream do not have associated output picture.

# Introduction

At the 30th JVET meeting, the following update to the design of neural-network post-filter characteristic (NNPFC) and neural-network post-filter activation (NNPFA) SEI messages regarding to input pictures for an NNPF were agreed:

- An activated neural-network post-filter (NNPF) takes multiple input pictures and one or more of the input pictures belong to previous coded layered video sequence (CLVS).

- An activated NNPF that takes multiple input pictures have one or more of those input picture not available. This is the case when the NNPF is activated for pictures at the beginning of the bitstream. When one or more of input pictures is not available, the nnpfc_absent_input_pic_zero_flag syntax element that is present in NNPFC SEI message provides indication whether to replace the missing input picture(s) with a duplicate of the last available input picture or a picture with zero value pels.

It is asserted that such design related to the handling of input pictures for NNPF may have problems as further described below. Solutions for each asserted problem is given at last section of this contribution.

# Problem description

## Problem 1

Consider the scenario of an NNPF that is signalled in an NNPFC SEI message and it takes multiple input pictures. The NNPF is activated by an NNPFA SEI message associated with a picture that in the beginning of a CLVS that is not the first CLVS of the bitstream. When decoder does random access to the CLVS or the bitstream is spliced at that CLVS, the application of the NNPF may result in filtered picture(s) that is different from what is expected (i.e., mismatch issue may occur) since one or more input pictures for the filtering process may be different from what it is supposed to be.

## Problem 2

For each input picture of an NNPF, there is signalling (both in NNPFC and in NNPFA) that specifies whether filtering process using the NNPF will output an output picture associated with that input picture or not. The signalling of output pictures associated with input pictures is present in NNPFC SEI message and this can be later updated by signalling in NNPFA SEI message associated with the NNPFC SEI message such that the output of pictures may be cancelled.

It is asserted that a problem related to output pictures associated with input pictures that are not present (i.e., unavailable input pictures). It is expected that there should not be any output picture associated with such unavailable input pictures but yet in the current design of NNPFC and NNPFA SEI message, it is not clear how this can be prevented.

# Proposed solutions

## Solution for problem 1

We propose the following:

- Signal a flag (i.e., nnpfa_broken_link_flag) in NNPFA SEI message that specifies whether the result of applying the target NNPF may result in broken-link / mismatch (i.e., filtered picture in decoder that is different from is corresponding filtered picture at encoder).

- When the bitstream is spliced at CLVS that resulted in one or more input pictures of the target NNPF becomes unavailable, the value of nnpfa_broken_link_flag is set to be equal to 1.

- The flag can be used by decoder to decide whether or not to activate / apply the NNPF.

The implementation of the above proposed solution is as follows:

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_target_base_flag | u(1) |
| nnpfa_broken_link_flag | u(1) |
| nnpfa_persistence_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| } |  |
| } |  |

…

nnpfa_broken_link_flag equal 1 specifies that the post-processing filtering process using the target NNPF may produce different / mismatched picture. nnpfa_broken_link_flag equal 0 specifies that the above does not apply. When the number of input pictures for the target NNPF is equal to 1, the value of nnpfa_broken_link_flag shall not equal to 1.

NOTE – Mismatched filtered picture may be due to application of the target NNPF in the event of random access or after the bitstream was spliced and one or more input picture(s) for the target NNPF becomes unavailable.

NOTE – When nnpfa_broken_link_flag is equal to 1, due to possible mismatch between filtered picture(s) by encoder and decoder, decoder may choose not to apply / activate the filter.

…

## Solution for problem 2

We propose a constraint such that when an NNPFA SEI message activates an NNPFC SEI message for which the target NNPF takes multiple input pictures and one or more of the input pictures are unavailable (i.e., are not present in the bitstream) and the unavailable input pictures have associated output pictures (i.e., the value of nnpfc_input_pic_output_flag[ i ] of those input pictures is equal to 1), for each of those input pictures that are unavailable but has associated output picture, the nnpfa_output_flag[ i ] associated with the input picture shall be present and be equal to 0.

The implementation of the above proposed solution is as follows:

…

nnpfa_num_output_entries specifies the number of nnpfa_output_flag[ i ] syntax elements present in the NNPFA SEI message. The value of nnpfa_num_output_entries shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive.

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive.

When the target NNPF takes multiple input pictures and one or more of the input pictures are not present in the bitstream (i.e., unavailable input pictures) and the unavailable input pictures have associated output pictures (i.e., the value of their associated nnpfc_input_pic_output_flag[ i ] in NNPFC SEI message is equal to 1), for each of those unavailable input pictures that has associated output picture, the nnpfa_output_flag[ i ] associated with such input picture shall be present and be equal to 0.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).