| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0075-v1 |
| --- | --- |

| Title: | Non-EE2: On IntraTMP block vector |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Lai Zhang, Luhang Xu, Haoping Yu, Jonathan Gan, Fan Wang Zhihuang Xie, Dong Wang (OPPO) | Tel: Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

IntraTMP block vectors (BVs) are stored for coding future blocks. Currently, an intraTMP BV may have a quarter-pel resolution but it is always saved in a full-pel resolution after coding an intraTMP block. This contribution proposes to store IntraTMP BVs in their full resolution. In addition, it is also proposed that intraTMP BVs are stored for HMVP. On top of ECM-9.0, the simulation results are reported as follows:

AI: 		-0.07% (Y), -0.04% (U), -0.08% (V), 98.1% (EncT), 99.9% (DecT) for class F

-0.10% (Y), -0.11% (U), -0.14% (V), 99.0% (EncT), 99.1% (DecT) for class TGM

# Introduction

IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. In order to search for the best template, a cost function, e.g., SAD is defined and both the encoder and decoder use the same search strategy to obtain the best matched block. Once the best matched block is found, the corresponding reference block indicated by a block vector (BV) will be selected as the prediction block for coding the current block. In ECM-9.0, IntraTMP BVs are always stored for the purpose of coding future IBC blocks, after coding the block in the IntraTMP mode. However, IntraBVs are only saved in a full pel resolution, even though some of the IntraTMP BVs may have a quarter-pel resolution. In addition, intraTMP BV is not stored for HMVP.

# Proposed method

This contribution proposes to always save BVs in a quarter-pel resolution, instead of a full-pel resolution, after coding an intraTMP block. In addition, quarter-pel IntraTMP BVs are also stored for HMVP for coding future blocks.

# Simulation results

The proposed method is implemented on top of the ECM-9.0 software [1], and tested based on the common test condition [2]. The test results are summarized in Table 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.07% | -0.04% | -0.08% | 98.1% | 99.9% |
| Class TGM | -0.10% | -0.11% | -0.14% | 99.0% | 99.1% |

Table 1. The proposed method over ECM-9.0

# Conclusions

This contribution proposes to store quarter-pel BVs instead of full-pel BVs for  intraTMP blocks. In addition, quarter-pel IntraTMP BVs are also stored for HMVP. The proposed method brings 0.07% and 0.10% coding gain for class F and class TGM for AI without any complexity and memory increase over ECM-9.0. It is suggested to study the proposed method in EE.

# References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).